| 3GPP TR 28.832 V0.6.0 (2023-4) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on management aspects of URLLC (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions of terms, symbols and abbreviations	5

3.1	Terms	5

3.2	Symbols	5

3.3	Abbreviations	6

4	Concepts and overview	6

5	Key Issues Investigation and Potential Solutions	7

5.1	Key Issue #1: Classification of URLLC related RAN features from management perspective	7

5.1.1	Description	7

5.1.2	Potential solutions	7

5.1.2.1	Classification of URLLC related RAN features	7

5.1.2.2	Candidates of features to be studied in management	8

5.2	Issue #2: Support for performance management related on URLLC resource load	9

5.2.1	Description	9

5.2.1.1	It exists URLLC and eMBB coexistence scenarios	9

5.2.1.2	Support for performance management related on URLLC resource load	9

5.3	Issue #3: Support for URLLC Performance management on reliability in RAN	10

5.3.1	Description	10

5.3.1.1	Support for network performance on reliability in RAN	10

5.4	Issue #4: Configuration of reliability in slice profiles and service profile	10

5.4.1	Description	10

5.4.2	Potential solutions	10

5.5	Issue #5: Configuration of latency for URLLC in RAN over the air interface	11

5.5.1	Description	11

6	Conclusion and Recommendation	12

6.1	Issue #1: Classification of URLLC related RAN features from management perspective	12

6.Y	Issue #Y	12

Annex <X> (informative): Change history	13

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document investigates the potential requirements related to management of the URLLC features defined in RAN side. It documents key issues related to configuration management of URLLC features and to the performance measurements for URLLC service. It provides potential solutions and recommendations for the normative work.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.104: “Service requirements for cyber-physical control applications in vertical domains; Stage 1”

[3]	3GPP TS 22.261: “Service requirements for the 5G system; Stage 1”

[4]	3GPP TS 38.211: “NR; Physical channels and modulation”

[5]	3GPP TS 38.212: “NR; Multiplexing and channel coding”

[6]	3GPP TS 38.213: “NR; Physical layer procedures for control”

[7]	3GPP TS 38.214: “NR; Physical layer procedures for data”

[8]	3GPP TS 38.300: “NR; NR and NG-RAN Overall Description; Stage2”

[9]	3GPP TS 38.321: “NR; Medium Access Control (MAC) protocol specification”

[10]	3GPP TS 38.323: “NR; Packet Data Convergence Protocol (PDCP) specification”

[11]	3GPP TR 38.824: “Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)”

[12]	3GPP TS 28.541: “Management and orchestration; 5G Network Resource Model (NRM);”

[13]	3GPP TS 28.552: “Management and orchestration; 5G performance measurements”

[14]	3GPP TS 28.554: “Management and orchestration; 5G end to end Key Performance Indicators (KPI)”

[15]	3GPP TS 38.314: “NR; Layer 2 Measurements;”

[16]	ITU-R M.2410: “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<PI >	< Preemption Indication >

<CI >	< Cancellation Indication >

<PB >	< Power Boosting >

4	Concepts and overview

URLLC, Ultra Reliable Low Latency Communication, is a typical service of 5G network, which is mainly applicable to the scenarios having high requirements on latency and reliability. The deployment and commercialization of URLLC service are of great significance to network operators. Efforts to build URLLC service oriented to vertical industries may become a potential growth point to increase revenue.

In order to satisfy the requirements of URLLC service, 3GPP RAN groups have defined many features to decrease latency and increase reliability to guarantee the SLA of URLLC service. However, only principle and implementation mode of each feature are defined in RAN specifications, and it is not indicated that how to manage the features to enable URLLC service in actual network deployment. Consequently, the management of URLLC related features need to be studied to achieve the service requirements of URLLC defined in TS 22.261[3].

The characteristics of URLLC are mainly reflected in several aspects such as delay/latency, reliability and availability. The concepts and terms used in this document related to management of URLLC service are shown below:

| Term | Reference | Definition |
| --- | --- | --- |
| end-to-end latency | TS 22.261 [3] | The time that it takes to transfer a given piece of information from a source to a destination, measured at the communication interface, from the moment it is transmitted by the source to the moment it is successfully received at the destination. |
| reliability | TS 22.261 [3] | In the context of network layer packet transmissions, percentage value of the packets successfully delivered to a given system entity within the time constraint required by the targeted service out of all the packets transmitted. |
| survival time | TS 22.261 [3] | The time that an application consuming a communication service may continue without an anticipated message. |
| communication service availability | TS 22.261 [3] | Percentage value of the amount of time the end-to-end communication service is delivered according to a specified QoS, divided by the amount of time the system is expected to deliver the end-to-end service. |

The document studies on the management of URLLC related features defined in RAN groups, including sorting the features from latency, reliability and multiplexing aspects, investigating the requiremnets of configuration management and performance measurements in OAM.

5	Key Issues Investigation and Potential Solutions

5.1	Key Issue #1: Classification of URLLC related RAN features from management perspective

5.1.1	Description

URLLC is a set of service scenarios which require low-latency and high reliable communications. Specific SLAs are defined for each scenario correspondingly. Different scenarios have different requirements, some of which focus on latency (e.g. Motion control) and some focus on reliability (e.g. Discrete automation).

In order to satisfy the requirements of URLLC, many features have been defined by 3GPP RAN side to decrease latency and increase reliability to guarantee the SLAs of different URLLC service scenarios. These features with different functions and different effects are distributed in different specifications, which bring complexity to the invocation and management when deploying URLLC service. Features related to URLLC need to be classified from the perspective of management, so that different features can be invoked according to different SLAs, and the management of URLLC-related features can be achieved.

5.1.2	Potential solutions

5.1.2.1	Classification of URLLC related RAN features

This document sorts out the features related to URLLC defined in RAN and classifies them from a management perspective. According to the characteristics of URLLC service, features are classified into the following two categories based on their effects: low latency and ultra reliability. Features belonging to low-latency category are mainly used to reduce data transmission delay, and reliability features are mainly used to improve the reliability of transmission. Among the features belonging to low-latency category, some of them reduce the transmission latency from the effective mechanism and other features improve service priority to reduce URLLC latency in multi-service scenario.

| Feature | Category | Reference |
| --- | --- | --- |
| Mini-slot transmission | Low latency | TS 38.214[7] |
| Numerology/SCS | Low latency | TS 38.211[4] |
| UL configured grant | Low latency | TS 38.214[7] |
| DL SPS | Low latency | TS 38.213[6] |
| PDCCH monitoring | Low latency | TS 38.213[6] |
| Logical channel priority | Low latency | TS 38.321[9] |
| Short PUCCH | Low latency | TS 38.211[4] |
| UE processing capability#2 | Low latency | TS 38.214[7] |
| Span based PDCCH monitoring | Low latency | TS 38.212[5]/TS 38.213[6] |
| UL configured grant enhancements | Low latency | TS 38.214[7] |
| DL SPS enhancements | Low latency | TS 38.213[6] |
| Sub-slot level HARQ-ACK (UCI enhancements) | Low latency | TS 38.213[6] |
| Two HAQR-ACK codebooks (UCI enhancements) | Low latency | TS 38.213[6] |
| Low SE MCS/CQI table | Ultra reliability | TS 38.214[7] |
| PDSCH repetitions | Ultra reliability | TS 38.214[7] |
| PUSCH repetitions | Ultra reliability | TS 38.214[7] |
| PUCCH repetitions | Ultra reliability | TS 38.213[6] |
| PDCCH aggregation level 16 | Ultra reliability | TS 38.213[6] |
| PDCP duplication | Ultra reliability | TS 38.323[10] |
| PUSCH repetitions enhancements | Ultra reliability | TS 38.214[7] |
| DCI format 0_2 and DCI format 1_2 | Ultra reliability | TS 38.213[6] |
| DL Multi-TRP for URLLC data channel repetitions | Ultra reliability | TS 38.213[6] |
| PDCP duplication enhancements | Ultra reliability | TS 38.323[10] |
| DL Preemption Indication (PI) | Low latency | TS 38.213[6] |
| Code Block Group (CBG) | Low latency | TS 38.214[7] |
| Inter UE: UL Cancellation Indication(CI) | Low latency | TS 38.213[6] |
| Inter UE: Power Boosting | Low latency | TS 38.213[6] |
| Intra UE: UL Prioritization | Low latency | TS 38.213[6] |

5.1.2.2	Candidates of features to be studied in management

There are nearly 30 features classified in the clause 5.1.2.1. The mechanisms of the features above are various and the protocol layers where they have effects are different. Different features have different effects and they also show different performance in improving the characteristics of URLLC service. The amount of RAN features related to URLLC service is large. In order to ensure the accuracy and efficiency of the subsequent study, it is necessary to select some important features as candidates for the study of management. The potential results could be configuration parameters, feature switches, performance measurements, etc. Based on the conclusions made in TR 38.824[11] and TS 38.300 [8] together with the mechanisms of features, the following features are selected as candidates for management investigation.

PDCCH enhancements

- UE processing capability#2

- Span based PDCCH monitoring

- DCI format 0_2 and DCI format 1_2

UCI enhancements

- Sub-slot level HARQ-ACK

- Two HAQR-ACK codebooks

PUSCH enhancements

- PUSCH repetitions/PUSCH repetitions enhancement

- Mini-slot

PDSCH enhancements

- PDSCH repetitions

- Mini-slot

Inter-UE multiplexing

- UL Cancellation Indication

- UL Power Boosting

- DL Pre-emption Indication

Scheduling

- UL configured grant transmission

- DL SPS

- Low SE MCS/CQI table

Layer 2 enhancements

- Logical Channel Priority Restrictions

- PDCP Duplication

5.2	Issue #2: Support for performance management related on URLLC resource load

5.2.1	Description

5.2.1.1	It exists URLLC and eMBB coexistence scenarios

Under the new definition of 5G application scenarios, there are coexistence scenarios of URLLC and eMBB services, and 3GPP protocol also contains related contents of eMBB and URLLC multiplexing mechanisms.

Taking the uplink service scenario as an example, TR38.824[11] evaluates the performance of URLLC and eMBB services under enhanced UL inter UE Tx prioritization/multiplexing mechanisms, and proposes potential enhancements for UL inter UE Tx prioritization/multiplexing, which includes UE UL cancelation mechanisms and enhanced UL power control.

Corresponding to the UE UL cancelation mechanisms, there is a definition of Cancellation Indication (CI) in TS 38.213[6]. The Cancellation Indication instructs other UE services to cancel their transmissions, which can realize resource preemption for different services in the uplink transmissions. Corresponding to enhanced UL power control, there is a related definition of power boosting (PB). By increasing the uplink transmission power of the UE, it can resist the interference caused by the transmission of other UEs.

At the same time, preemption indication (PI) is also defined for resource preemption of different services in the downlink transmission, and PI can be used to indicate to other UEs that their resources are preempted.

The contents of the above protocols confirm the existence of URLLC and eMBB coexistence scenarios.

5.2.1.2	Support for performance management related on URLLC resource load

At present, the network resource load is mainly evaluated through resource usage-related measurements. Referring to TS 28.552[13], the evaluation measurements are mainly PRB usage rate-related measurements, which measures usage (in percentage) of physical resource blocks (PRBs). Although these measurements can evaluate the overall resource load of the cell, they cannot effectively evaluate the resource load of the URLLC service under the eMBB and URLLC multiplexing scenarios.

For example, in a statistical time period, the PRB usage rate of the network is low. Because the URLLC service has high requirements for delay sensitivity, it needs to be transmitted immediately. If the URLLC service has data transmission requirements on the resources scheduled by eMBB, the URLLC service will preempt eMBB service resources. In this case, since the PRB usage rate only reflects the overall resource load of the cell, it cannot reflect the situation that the resources of the URLLC service are insufficient at this time.

Therefore, the existing PRB related measurements cannot effectively evaluate the resource load of URLLC services under eMBB and URLLC multiplexing scenarios. And what this issue needs to solve is to propose measurement method for evaluating the resource load of URLLC services in eMBB and URLLC multiplexing scenarios.

5.3	Issue #3: Support for URLLC Performance management on reliability in RAN

5.3.1	Description

As a new service deployed in 5G, URLLC is significantly different from traditional eMBB service in terms of service requirement. In order to guarantee the performance of URLLC, the performance of 5G network which provides URLLC service needs to meet certain target for reliability accordingly.

Annex F in TS 22.104 [2] depicts relation of reliability and communication service availability. It has the following description, “Communication service availability addresses the availability of a communication service. This definition follows the vertical standard IEC 61907 [7]. On the other hand, reliability is a 3GPP term and addresses the availability of a communication network.”

The issue mainly focuses on the 5G RAN network that provides URLLC service. In particular, the measurements on communication network performance need to be investigated.

5.3.1.1	Support for network performance on reliability in RAN

Reliability is a typical network performance measurement used to evaluate whether the 5G network which provides URLLC services meets the corresponding performance requirements. As for 5G network, it is the radio network including air interface that mainly restricts the reliability and latency performance. Therefore, more attentions should be paid to the reliability and latency performance measurement for radio network from the perspective management. However, the reliability performance measurements for radio network are not totally enough.

The definition of reliability is specified in TS 22.261 [3]:

- reliability: in the context of network layer packet transmissions, percentage value of the packets successfully delivered to a given system entity within the time constraint required by the targeted service out of all the packets transmitted.

According to the above definition, when trying to calculate the reliability of a network, time constraint (a required maximum time) needs to be considered. Neither PER defined in TS 28.552[13] nor reliability KPIs defined in TS 28.554[14] seem to totally match the definition of URLLC reliability in RAN.

URLLC performance management on reliability and latency should be supported by 5G management system and the measurement method should be studied.

5.3.2	Potential Solution

Ultra-Reliable is a typical characteristic of URLLC service, thus reliability is an important metric of network which provides URLLC service. The measurement of reliability can be used to evaluate the performance of URLLC network. So the requirements for performance management of radio network providing URLLC services is:

-	The OAM should have the capability of providing the reliability of NG-RAN with specific delay threshold.

Since the existing measurements and KPIs can’t match the definition of reliability in RAN considering delay threshold well, a new approach needs to be studied to measure the reliability performance in RAN. According to the definition of reliability, this contribution proposes an algorithm in RAN, which is as follows: Calculate the ratio between the number of successfully transmitted packets with a delay threshold and the total number of packets between UE and gNB, and the result is reliability in RAN.

5.4	Issue #4: Configuration of reliability in slice profiles and service profile

5.4.1	Description

URLLC is a set of service scenarios which require low-latency and high reliable communications. TS 22.261 [3]specifies service requirements for the 5G system. There are use cases of different reliability requirement between UL and DL, like in 1st use case of table 7.6.1-1 in TS 22.261 [3]. However, the NRM for slice profiles in TS 28.541 [12] does not support reliability to distinguish between UL and DL configuration. Based on the above description, the configuration of reliability in slice profiles and service profile need to be studied.

5.4.2	Potential solutions

1. The ServiceProfile <<dataType>> defined in TS 28.541 [12] shall add dLReliability and uLReliability attributes. The original reliability attribute shall be replaced by these two new attributes.

The dLReliability and uLReliability attributes shall be added as follows:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dLReliability | O | T | T | F | T |
| uLReliability | O | T | T | F | T |

2. The CNSliceSubnetProfile<<dataType>> defined in TS 28.541 [12] shall add dLReliability and uLReliability attributes. The original reliability attribute shall be replaced by these two new attributes.

The dLReliability and uLReliability attributes shall be added as follows:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dLReliability | O | T | T | F | T |
| uLReliability | O | T | T | F | T |

3. The RANSliceSubnetProfile<<dataType>> defined in TS 28.541 [12] shall add dLReliability and uLReliability attributes. The original reliability attribute shall be replaced by these two new attributes.

The dLReliability and uLReliability attributes shall be added as follows:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dLReliability | O | T | T | F | T |
| uLReliability | O | T | T | F | T |

4. The TopSliceSubnetProfile<<dataType>> defined in TS 28.541 [12] shall add dLReliability and uLReliability attributes. The original reliability attribute shall be replaced by these two new attributes.

The dLReliability and uLReliability attributes shall be added as follows:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dLReliability | O | T | T | F | T |
| uLReliability | O | T | T | F | T |

5. The dLReliability and uLReliability attributes shall be defined in TS 28.541 [12] as follows.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| dLReliability | An attribute specifies in the context of network layer DL packet transmissions, percentage value of the amount of sent network layer packets successfully delivered to a given system entity within the time constraint required by the targeted service, divided by the total number of sent network layer packets, see TS 22.261 [3]. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| uLReliability | An attribute specifies in the context of network layer UL packet transmissions, percentage value of the amount of sent network layer packets successfully delivered to a given system entity within the time constraint required by the targeted service, divided by the total number of sent network layer packets, see TS 22.261 [3]. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |

5.5	Issue #5: Configuration of latency for URLLC in RAN including the air interface

5.5.1	Description

URLLC is a service with requirement of ultra-reliability and low latency. End to end latency is an important attribute used to describe the requirement for URLLC service. Latency in RAN is part of end to end latency and the corresponding attribute can be used to specify the maximum allowed delay in RAN including the air interface. The definition of latency in RAN is specified in ITU-R M.2410. The description is as follows:

4.7.1 User plane latency

User plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active state. This requirement is defined for the purpose of evaluation in the eMBB and URLLC usage scenarios.

According to the definition above, the latency in RAN should contain the delay over the air interface which is a crucial part of end to end latency. In TS 28.541[12], the attribute “dlLatency” and “ulLatency” in RANSliceSubnetProfile only represent the packet processing latency in gNB exluding that of air interface. The performance of delay over the air interface can’t be evaluated simply through the existing latency configuration. Concequently, the issue is that the existing attributes in RANSliceSubnetProfile cannot match the configuration requirement for latency when taking air interface into consideration.

5.5.2	Potential Solution

The requirement for configuration management of radio network providing URLLC services is:

-	The OAM should have the capability of configuring the maximum allowed downlink and uplink latency in RAN including the delay over air interface.

According to the requirement above, the attributes “dlLatency” and “ulLatency” of RANSliceSubnetProfile defined in TS 28.541should be modified to contain the latency over the air interface. The specific changes are proposed.

1. The attributes “dlLatency” and “ulLatency” of RANSliceSubnetProfile<<dataType>> defined in TS 28.541 shall be modified as follows:

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| RANSliceSubnetProfile.dlLatency | An attribute specifies the required DL packet transmission latency (millisecond) in RAN including the air interface of the network slice and is used to evaluate the delay between NG-RAN and UE, e.g. time between received DL packet from UPF the packet successfully received by UE. See clause 5.1.1.1.6 in TS 28.552[13]. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| RANSliceSubnetProfile.ulLatency | An attribute specifies the required UL packet transmission latency (millisecond) in RAN including the air interface of the network slice and is used to evaluate the delay between UE and NG-RAN, e.g. time between the UL packet scheduled in UE and the packet successfully sent to UPF. See clause 5.1.1.1.7 in TS 28.552[13]. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |

NOTE1: Even though this study is about URLLC, the two attributes are also applicable for latency requirement for non URLLC services.

NOTE2: The two attributes represent maximum latency allowed or target latency of RAN slice subnet.

6	Conclusion and Recommendation

6.1	Issue #1: Classification of URLLC related RAN features from management perspective

This issue and corresponding solution have investigated the features designed for URLLC and try to classify the RAN features related to URLLC into two categories based on the effects: low latency and ultra-reliability from management perspective. Some important features are identified as candidates for the investigation of URLLC management. Features to implement inter-UE multiplexing, PDSCH enhancement, PDCP duplication, etc. are selected as examples. The detailed classification method and candidate features are described in clause 5.1.2.

The above work is concluded as the summary of existing features related to URLLC and preparation the follow-up issues of this study and no further normative work needs to be recommended.

6.Y	Issue #Y

6.4	Issue #4: Configuration of reliability in slice profiles and service profile

This issue identifies a disalignment between the different requirements for UL and DL defined on reliability in TS 22.261[3] and the single reliability attribute used in slice and service profiles defined in TS 28.541[12].

TS 22.261[3] specifies service requirements for the 5G system. Clause 7.6.1 defines some use cases of URLLC. The first use case of table 7.6.1-1 has different requirements for uplink and downlink reliability. However, the existing service profile and slices profiles in TS 28.541[12] can’t support for configuring different reliabilities for UL and DL because there is only one reliability attribute in either service and slice profiles.

The corresponding solution tries to addresses the above issue by enhancing the reliability related attribute used in slice and service profiles from one single reliability attribute to two separate reliability attributes representing UL and DL respectively for URLLC. Detailed description about the solution is in clause 5.4.2. It is recommended to make some enhancement on NRM referred to this solution in the future normative

NOTE: Even though this study is about URLLC, the attributes are also applicable for reliability requirement for non URLLC services.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-04 | SA5#142e | S5-222292 S5-222307 S5-222344 S5-222660 |  |  |  | 1. TR 28.832-0.0.0 initial skeleton 2. pCR 28.832 Add scope 3. pCR 28.832 Add Concepts and overview 4. pCR TR28.832 Add key issue on classification of URLLC related RAN features from management perspective | 0.1.0 |
| 2022-05 | SA5#143e | S5-223367 |  |  |  | 1. pCR TR28.832 Add candidates for study of feature management | 0.2.0 |
| 2022-08 | SA5#145e | S5-225807 S5-225808 |  |  |  | 1. pCR TR28.832 Add Issue on support for performance management related on URLLC resource load 2. pCR TR28.832 Add Issue on URLLC Performance management on reliability | 0.3.0 |
| 2022-11 | SA5#146 | S5-226966 S5-226975 S5-226972 |  |  |  | 1. S5-226966 pCR TR 28.832 Add issue on configuration of reliability in slice profiles and service profile 2. S5-226971 pCR TR 28.832 Add potential solution for issue on configuration of reliability in slice profiles and service profile 3. S5-226972 pCR TR28.832 Add New Key issue on configuration of latency for URLLC in RAN | 0.4.0 |
| 2023-03 | SA5#147 | S5-232956 |  |  |  | 1. S5-232956 pCR TR28.832 Add Conclusion and recommendation for issue#1 | 0.5.0 |
| 2023-04 | SA5#148e | S5-233349 S5-233604 S5-233605 S5-233636 |  |  |  | 1. pCR 28.832 Correction of terminologies and missing references 2. pCR TR28.832 Add New Solution for configuration of latency for URLLC in RAN 3. pCR TR28.832 Add Conclusion and recommendation for issue#4 4. pCR TR28.832 Add New Solution for URLLC performance management related to reliability | 0.6.0 |