3GPP TSG-SA5 Meeting #149 	S5-233823

Berlin, Germany, 22nd May 2023 - 26th May 2023

# Source:	Ericsson Hungary

# Title:	pCR 28.831 Add updates and conclusion for createMOI

# Document for:	Approval

Agenda Item:	6.8.2.2

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below

# 2	References

[1]		3GPP TS 28.831: " Management and orchestration; Study on basic Service-Based Management Architecture (SBMA) enabler enhancements"

[2] 		S5-233284 Rel-18 CR 28.831 Add conclusion for issue 3 Definition of createMOI

# 3	Rationale

The conclusion part of this  pCR was already approved at SA5#147 (S5-232345, revised to S5‑233035), but erroneously not implemented.

Some corrections to Issue #3: Definition of createMOI are also needed because:

- Some stage 3 solutions indicate the default values in the stage 3 data models

- The comment for parameter attributeListOut contradicts paragraph 4 of the description.

# 4	Detailed proposal

The following changes are proposed for TR 28.831[1].

Begin of modifications

### 4.3.3	CR proposal

The existing clause 11.1.1.1 in TS 28.532 shall be replaced as follows:

****** BEGIN *********************************************************************************

#### 11.1.1.1	createMOI operation

##### 11.1.1.1.1	Description

This operation is invoked by MnS consumers to request a MnS producer to create a (single) managed object instance on the MnS producer.

The "managedObjectClass" parameter in the request specifies the class name and the "managedObjectInstance" parameter the instance name of the object to be created. Both parameters shall be included in the request.

The MnS consumer shall generate the instance name by first assigning a value to the naming attribute of the new instance, and then constructing a DN according to TS 32.300 [25].

The MnS consumer shall provide in "attributeListIn" none, some or all values for the attributes specified by the managed object class definition of the class to be created. The MnS producer shall not update attribute values or remove attribute values, that are provided in the request, before creating the object and returning the "createMOI" response.

The properties of an attribute determine if attribute values shall, shall not or may be provided in the "createMOI" request. If no value is provided and a default value is specified for the attribute, the MnS producer shall set the attribute value to the default value. For further information on attribute properties and their impact on the presence or absence of attribute values in object creation requests and MnS producer behaviour, see TS 32.156 [x], clause 5.2.1 and annex B.

When the MnS producer assigns a default value to one or more attributes, the MnS consumer shall be made aware of this by either

- by indicating the default value in the stage 3 data model or

- the MnS producer shall include "attributeListOut" in the "createMOI" response , otherwise "attributeListOut" may be omitted.

In case of a successful operation, the object shall be created immediately upon reception of the "createMOI" request, and the "createMOI" response shall be returned immediately after the creation of the object. The MnS producer shall not wait with the creation of the object or returning the response until some other potentially long-lasting process or activity, that might be triggered by the reception of the request or the creation of the object, has completed.

Only objects, whose parent exists, can be created (directly under that parent). The MnS producer shall consider an attempt to create an object whose parent object does not exist as an error.

Note, stage 3 protocols represent missing values for attributes, that are defined by the object class, in the "createMOI" request in different ways. For some protocols just the attribute name may be present, without an attribute value. For other protocols, the complete attribute name/value pair may be absent.

Some stage 3 protocols do not support returning "attributeListOut". In this case, the MnS producer shall not modify the attribute list provided in the request before creating the object. If required by TS 32.156 [x], clause 5.2.1 and annex B, the MnS producer shall assign default values to attributes only after returning the "createMOI" response. Attribute value change notifications may be used to notify MnS consumers about the changes. Only default values, that have a specific definitive value may be assigned upon object creation. This is because the MnS consumer knows the MnS producer will assign this value when required according to TS 32.156 [x], clause 5.2.1 and annex B. Default values that are determined by the MnS producer based on standardized or proprietary selection methods are typically not known to MnS consumers.

##### 11.1.1.1.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| managedObjectClass | M | string | Class name of the managed object to be created. |
| managedObjectInstance | M | DN | Instance name of the managed object to be created. |
| attributeListIn | M | LIST OF SEQUENCE< attribute name, attribute value> | List of attribute name/value pairs of the managed object to be created. |

##### 11.1.1.1.3	Output parameters

| Parameter name | S | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListOut | O | LIST OF SEQUENCE< attribute name, attribute value> | List of attribute name/value pairs of the created object. |
| Status | M | ENUM (OperationSucceeded, OperationSucceededAttributesModified, Failed) |  |

****** END ***********************************************************************************

### 4.3.4	Conclusion

It is recommended to update the definition of the operation "createMOI" in clause 11.1.1.1 of TS 28.532 as defined in clause 4.3.3 of the present document.

End of modifications