3GPP TSG-SA5 Meeting #149 	S5-233864

Berlin, Germany, 22 – 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | Draft CR | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Input to draft CR TS 28.105 solution for ML inference emulation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shangai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use case of ML inference emulation has been discussed and agreed (in TR 28.908). This CR is to normatively propose the solution for the mentioned use case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | sage 2 solution has been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for the use case “ML inference emulation” |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.V(new clauses added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

## 7.G	ML Inference emulation Information Model

## 7.G.1	Class diagram

##### 7.G.1.1	Relationship

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class ManagedEntity <<ProxyClass>>

class MLInferenceEmulator  <<InformationObjectClass>>

class MLInferenceEmulationJob << InformationObjectClass >>

class MLEntity << InformationObjectClass >>

class MLInferenceEmulationRequest << InformationObjectClass >>

class MLInferenceEmulationReport<<dataType>>

ManagedEntity "1" *-- "*" MLInferenceEmulator: <<names>>

MLInferenceEmulator "1" -l-> "*" MLEntity: <<names>>

MLInferenceEmulator "1" *-- "*" MLInferenceEmulationJob: <<names>>

MLEntity "1" -d-> "1" MLInferenceEmulationRequest

MLInferenceEmulator "1" *-- "*" MLInferenceEmulationRequest: <<names>>

MLInferenceEmulationRequest "1" -r-> "1" MLInferenceEmulationJob

MLEntity "1" --> "1" MLInferenceEmulationJob

MLInferenceEmulationJob "*" -r-> "*" MLInferenceEmulationReport

MLInferenceEmulator "*" *-- "*" MLInferenceEmulationReport: <<names>>

note left of ManagedEntity

Represents the following IOCs:

Subnetwork or

ManagedFunction or

ManagementFunction

end note

@enduml

Figure 7.G.1.1-1: NRM fragment for ML inference emulation Control

##### 7.G.1.2	Inheritance

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class Top <<InformationObjectClass>>

class ManagedFunction <<InformationObjectClass>>

class MLInferenceEmulator << InformationObjectClass >>

class MLInferenceEmulationJob << InformationObjectClass >>

class MLInferenceEmulationRequest << InformationObjectClass >>

ManagedFunction <|-- MLInferenceEmulator

Top <|-- MLInferenceEmulator

Top <|-- MLInferenceEmulationJob

Top <|-- MLInferenceEmulationRequest

@enduml

Figure 7.G.1.2-1: ML inference emulation Inheritance Relations

## 7.G.2	Class definition

##### 7.G.2.1 	MLInferenceEmulator <<IOC>>

###### 7.G.2.1.1	Definition

This IOC represents the properties of MLInferenceEmulator.

Each MLInferenceEmulator is a managed object instantiable from the MLInferenceEmulator information object class and name-contained in either a Subnetwork, a ManagedFunction or a  ManagementFunction. The MLInferenceEmulator is a type of managedFunction, i.e. the MLInferenceEmulator is a subclass of and inherits the capabilities of a managedFunction

- The MLInferenceEmulator shall be associated with one or more MLEntitys. E.g., the MLEntitys associated with MLInferenceEmulator may be associated via a list of MLEntityIdentifers. The MLEntityIdentifers should specify the MLEntity and any specific version thereof e.g. as defined for the MLModel in NC322249

- The MLInferenceEmulator contains one or more MLInferenceEmulationJobs, where each MLInferenceEmulationJob is instantiated following or in response to a received request to create the MLInferenceEmulationJob ManagedObjectinstance (MOI). Alternatively, the MLInferenceEmulator may allow to instantiate one or more MLInferenceEmulationRequests and as such may name-contain one or more MLInferenceEmulationRequests

- MLInferenceEmulator should be associated with one or more MLInferenceEmulationReports generated for any one or more MLInferenceEmulationJobs.

- Each MLInferenceEmulator states the amount of resources consumed by ongoing MLInferenceEmulationJobs. Alternatively, this can be indicated by a percentage / amount of available resources (e.g. CPU 30% / 3 GB available ).

7.G.2.1.2	Attributes

The MLInferenceEmulator IOC includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| MLEntitys | CM | T | F | F | F |
| MLInferenceEmulationJob | M | T | F | F | F |
| MLInferenceEmulationResourceState | M | T | F | F | T |
| MLInferenceEmulationRequest | O | T | T | F | T |
| MLInferenceEmulationReports | O | T | F | F | F |

##### 7.G.2.2 	MLInferenceEmulationRequest <<IOC>>

###### 7.G.2.2.1	Definition

This IOC represents the properties of MLInferenceEmulationRequest.

For each request to undertake Inference emulation, a consumer may create a new MLInferenceEmulationRequest on the MLInferenceEmulator, i.e.,  MLInferenceEmulationRequest shall be an information object class that is instantiated for each request for Inference emulation

- Each MLInferenceEmulationRequest is associated to exactly one MLEntity

- Each MLInferenceEmulationRequest may contain specific reporting requirements that define how the MLInferenceEmulator may report about the MLInferenceEmulationRequest or the associated MLInferenceEmulationJob. Such requirememts may e.g., include an reportingPeriod

- The MLInferenceEmulationRequest may have a source to identify where its coming from and which may be used to prioritize among different MLInferenceEmulationRequests from different sources. The sources may for example be an enumeration defined for network functions, operator roles, or other functional differentiations.

- Each MLInferenceEmulationRequest may have a RequestStatus field that is used to track the status of the specific MLInferenceEmulationRequest or the associated MLInferenceEmulationJob.

- The RequestStatus is written by the MLInferenceEmulator when there is a change in status of the Inference emulation progress. The RequestStatus is an enumeration with the following values:

  - "Pending": Indicates that the  MLInferenceEmulationRequest has been received but no action has been taken thereof. When other actions are taken regarding the MLInferenceEmulationRequest, the RequestStatus changes to one of the following values.

  - "Triggered": when a corresponding MLInferenceEmulationJob has been instantiated and is running

  - "Suspended": the status when for one reason or another the MLInferenceEmulationRequest has been suspended by the MLInferenceEmulator or another authorized consumer, e.g. the operator.

  - "Served": the status when an MLInferenceEmulationJob associated  to the specific MLInferenceEmulationRequest has run to completion.

- The reporting requirements contained in the MLInferenceEmulationRequest must be mapped to an existing MLInferenceEmulationJob instance or create a new instance as needed.

7.G.2.2.2	Attributes

The MLInferenceEmulationRequest IOC includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| MLInferenceEmulationRequestID | M | T | F | F | F |
| MLEntity | M | T | F | F | F |
| MLinference | M | T | F | F | F |
| Source | M | T | T | F | T |
| RequestStatus | M | T | T | F | T |
| reportingPeriod | O | T | T | F | T |

##### 7.G.2.3	MLInferenceEmulationJob <<IOC>>

###### 7.G.2.3.1	Definition

This IOC represents the properties of MLInferenceEmulationJob.

For each MLEntity to be executed under Inference emulation, a MLInferenceEmulationJob is required. The MLInferenceEmulationJob may be instantiated for each MLEntity or may apply to multiple MLEntitys, i.e. the MLInferenceEmulationJob is associated with one or more MLEntitys.

- MLInferenceEmulationJob can be instantiated directly by an authorized consumer. Alternatively,  MLInferenceEmulationJob can be instantiated by MLInferenceEmulator in response to a MLInferenceEmulationRequest instantiated directly by an authorized consumer. In this latter case, the MLInferenceEmulationJob shall be associated with one or more MLInferenceEmulationRequests. Correspondingly, the MLInferenceEmulationRequests and the MLInferenceEmulationJobs are conditionally mandatory in that at least one of them must be associated with an instance of MLInferenceEmulationJob.

- Each MLInferenceEmulationJob may have a status attribute used to indicate the level of success of the MLInferenceEmulationJob. For example, in the case where a MLInferenceEmulationJob is instantiated for exactly one MLEntity and one AI/MLinference, the status may reflect the status of the internal job (e.g. training) executed for that MLEntity.

- The MLInferenceEmulationJob may have a source to identify the entity which instantiated it and which may be used to prioritize among different MLInferenceEmulationJobs from different sources. The sources may for example be an enumeration defined for network functions, operator roles, or other functional differentiations.

- Each MLInferenceEmulationJob will have attributes specifying the ReportingPeriod   (e.g. periodically, after completion, etc.)

- The reporting requirements contained in the MLInferenceEmulationJob must be mapped to an existing MLInferenceEmulationJob instance or create a new instance as needed.

The MLInferenceEmulationJob also represents the capability of compiling and delivering reports and notifications about MLInferenceEmulator or  its associated MLInferenceEmulationRequests, and MLInferenceEmulationJobs. The MLInferenceEmulator shall be associated with one or more instances of MLInferenceEmulationJob,

- The MLInferenceEmulationJob may report on one or more MLInferenceEmulationRequests or one or more MLInferenceEmulationJobs

- The MLInferenceEmulationJob should include a MLInferenceEmulationReportingMatrix that defines the frequencies at which report should be sent, the specific entities to which each report shall be sent at each time instant and the objects which shall be included in each report.

The MLInferenceEmulationJob shall be associated with one or more MLInferenceEmulationReports,

7.G.2.3.2	Attributes

The MLInferenceEmulationJob IOC includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| MLInferenceEmulationJobID | M | T | F | F | F |
| MLEntityID | M | T | F | F | F |
| MLInferenceEmulationRequest | O | T | T | F | T |
| Source | M | T | T | F | T |
| ProgressStatus | M | T | T | F | T |
| reportingPeriod | O | T | T | F | T |
| MLInferenceEmulationReports | O | T | F | F | F |

## 7.G.3	DataType definition

##### 7.G.3.1 	MLInferenceEmulationReport << dataType >>

###### 7.G.3.1.1	Definition

This dataType represents the properties of MLInferenceEmulationReport.

- MLInferenceEmulator may generate one or more MLInferenceEmulationReports,

- Each MLInferenceEmulationReport is associated to one or more MLEntitys.

- All MLInferenceEmulationReports must be associated with the MLInferenceEmulationJob instance, i.e., the MLInferenceEmulationJob provides/instantiates reports about MLEntitys or about the MLInferenceEmulationJob that is associated with the MLEntitys for which inference emulation is requested and/or executed. It may also report on specific MLInferenceEmulationRequests.

7.G.3.1.2		Attributes

The MLInferenceEmulationReport includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| MLInferenceEmulationReportID | M | T | F | F | F |
| MLEntitys | M | T | F | F | F |
| Attributes related to role |  |  |  |  |  |
| MLInferenceEmulationRequests | O | T | F | F | F |
| MLInferenceEmulationJobRef | M | T | F | F | F |

## 7.G.4	Attribute definitions

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| MLEntitysList | It indicates the list of ML enabled functions and models available at the MLInferenceEmulator function | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| MLInferenceEmulationJobID | It indicates an identifier for a specific instantiated MLInferenceEmulationJob | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| MLEntityID | It indicates an identifier for a specific MLEntity or MLEntity. It may include the version identifiers of any such MLEntity or MLEntity | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| Source | It indicates a managed object that initiates the request for MLInferenceEmulator | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| MLinference | It indicates the AI/ML task needed to be executed in the ML inference emulator Allowed values: Training, Validation, Testing, Inference | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| ProgressStatus | It indicates the status of the MLInferenceEmulator process as evaluated by the MLInferenceEmulator function. | type: DN (see TS 32.156 [12]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| reportingPeriod | It defines how the MLInferenceEmulator may report about the MLInferenceEmulationRequest or the associated MLInferenceEmulationJob | type: DN (see TS 32.156 [12]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| MLInferenceEmulationReportingID | It indicates an identifier for a specific MLInferenceEmulationJob instance | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLInferenceEmulationReportID | It indicates an identifier for a specific MLInferenceEmulator report | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| emulatorResourceConsumption | It indicates the amount of resources used by ongoing MLInferenceEmulationJobs, or the percentage / amount of available resources e.g. CPU | type: string / integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |

End of Changes