3GPP TSG-SA5 Meeting #149 	S5-233874

Berlin, Germany, 22 – 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0030 | rev | - | Current version: | 17.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel18_CR_28312 Use case and Requirements on Intent-driven SON orchestration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shangai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_ph2 |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use cases of Intent-driven SON orchestration has been discussed and agreed (in TR 28.912). This CR is to normatively propose the use case and requirements for the mentioned use case |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Use case description and requirements have been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for the use cases “Intent-driven SON orchestration” |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1 (new clause added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge Link: |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

### 5.1.J		Intent-driven SON orchestration

#### 5.1.J.1	Introduction

Although SON focusses on the optimization of parameters based on operator stated targets, even when using SON to manage the radio parameters and KPIs, the complex interactions do remain critical. In TS 28.312 [2], an intent focuses more on describing the "What" needs to be achieved but less on "How" that outcome should be achieved. As such, intent-driven MnS can work as the interface between Operator and SON. Using that interface the Operator can specify the customization the Operator wants on certain radio parameters and/or KPIs, and the intent-driven MnS can generate appropriate commands to be executed by SON to achieve those targets. For example, the operator may state requirements for specific SON functions (SFs) that are responsible for optimizing the KPIs and/or control parameters specified within the intent. Such customizations may include specific values expected as outcomes on certain parameters or metrics.

For an intent to be fulfilled via SON orchestration, the intent needs to relate to network control parameter or to network KPIs. Relatedly, the intent driven SON orchestrator needs to characterize the received into one of three categories represented by table 5.1.J.1-1.

Table 5.1.J.1-1

| Type | Description | Example intents |
| --- | --- | --- |
| Type 1 | Fulfils one of: Intent parameters defined on network control parameters, and no context defined Intent context defined on network control parameters, and no parameters defined Both Intent context and intent parameters defined on network control parameters | Increase cell X TXP Make cell X TXP remain constant Make cell X TXP constant and change RET by 3 degrees |
| Type 2 | Fulfils one of: Intent parameters defined on KPI, and no context defined Intent context defined on KPI, and no parameter defined Both Intent context and intent parameters defined on KPI | Reduce cell X interference Avoid increase in cell X interference Increase cell X throughput without changing interference |
| Type 3 | Fulfils one of: Intent parameters defined on network control parameters while Intent context defined on KPI, Intent parameters defined on KPI while Intent context defined on network control parameters | Make cell X TXP constant and reduce interference Change cell X TXP to 45 dBm without changing interference |

Accordingly, given this evaluation within the MnS producer, if the intent does not relate to any of the three categories above, the Intent-driven SON orchestration should inform the intent consumer that the intent cannot be executed via SON orchestration.

Moreover, when executing via SON, the Intent-driven SON orchestration may observe contradictions in the intent - i.e., cases where targets on different expectation are in conflict. The Intent-driven SON orchestration should inform the intent MnS consumer of such contradictions which occur in the SON orchestration but not necessarily in the intent.

The related sequence of actions is highlighted by the figure 5.1.J..1-1

Figure 5.1.J.1-1: Intent-driven SON orchestration

#### 5.1.J.2	Requirements

REQ-Intent_SON_1: The intent driven MnS shall have the capability to enable a consumer to state an intent requesting for the SON Functions (SFs) responsible for optimizing the KPIs and/or control parameters specified in the intent.

REQ-Intent_SON_2: The intent driven MnS shall have the capability to enable a consumer to state an intent requesting to execute SON functions needed optimize the KPIs and/or control parameters specified in the intent.

End of Changes