3GPP TSG-SA5 Meeting #149 	S5-233913

# Berlin, Germany, 22nd May 2023 - 26th May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0037 | rev | - | Current version: | 17.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add use case and requirements for intent driven approach for radio network capacity optimization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei,Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_ph2 |  |  |  |  |  | Date: |  |  | 2023-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use case and requirements for intent driven approach for radio network capacity optimization scenario is discussed and described in clause 4.3 in TR 28.912 and recommended to be introduced in TS 28.312. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add use case and requirements for intent driven approach for radio network capacity optimization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use case and requirements for intent driven approach for radio network capacity optimization scenario is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.1.x	Intent containing an expectation on radio network capacity performance to be assured

5.1.x.1	Introduction

In this scenario, MnS consumer expresses its intent containing an intent expectation on radio network capacity performance to be assured to MnS producer, which may include area information (e.g. geographical area), RATs (e.g. NR only, EUTRAN only, or all RATs), NRfrequencyBands, radio network capacity targets (e.g. target percentage of high UL/DL PRB Load (e.g. < 70%)).

Based on the received intent containing expectation on radio network capacity performance to be assured, MnS producer collects and analyses corresponding radio network capacity related data (e.g. number of PRBs used for UL/DL traffic transmission, total number of PRBs available for UL/DL traffic transmission) of corresponding RAN NEs in the specified areas, identifies the potential radio network capacity issues (e.g. high UL/DL PRB Load issue (load > 90%) ), which will impact radio network capacity target satisfaction, analysis, evaluate, decides and adjusts the radio network capacity configuration parameters (e.g. MLB configuration parameters) for impacted RAN NEs in the specified areas to address the radio network capacity issues.

MnS producer continuously monitors the radio network capacity performance (e.g. percentage of high UL/DL PRB Load (e.g. < 70%)) for the specified area, and decides whether radio network capacity target is satisfied. If not satisfied, MnS producer iteratively executes above workflows (including collect, identification, analysis, evaluation, decision and adjustment) to fulfil the radio network capacity targets.

MnS producer may report MnS consumer about the intent fulfilment information and achieved value for radio network capacity targets (e.g. percentage of high UL/DL PRB Load) for the specified area which enables MnS consumer to monitor the intent containing an expectation on radio network capacity performance to be assured.

5.1.x.2	Requirements

REQ-Intent_Opt_Capacity-CON-1: The intent driven MnS for radio network shall have capabilities enabling the MnS consumer to express intent containing an expectation on radio network capacity performance to be assured for the specified area.

REQ-Intent_ Opt_Capacity-CON-2: The intent driven MnS for radio network shall have capabilities enabling the MnS consumer to obtain intent report information (including fulfilment information and achieved value) for intent containing an expectation on radio network capacity performance to be assured.

Note: the example of radio network capacity performance is target percentage of high UL/DL PRB Load for a specified Geographical area.

End of Changes