3GPP TSG-SA5 Meeting #149 	S5-234024

Berlin, Germany, 22 – 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0048 | rev | - | Current version: | 17.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel18 CR 28312 Use case and Requirements exposing Intent-Logic |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shangai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_ph2 |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Fullfilment of intents may be complex with different impacts on the network. Intents MnS consumers may as such need to see or know the applicable flow of logic expected to support their intents. This CR is to normatively propose the use case and requirements for exposing Intent-Logic flow |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use case description and requirements have been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for the use cases “exposing Intent-Logic flow” |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1 (new clause added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge Link: |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

## 5.1.M	Exposing Intent-Logic

#### 5.1.M.1	Introduction

Fulfilment of intents is implementation specific and may be complex with different impacts on the network. However, there is no guarantee that intent MnS consumers will have default trust that the actions or logic flows proposed by intent MnS producers are accepted. The Intent MnS consumers may as such need to see or know the applicable flow of logic expected to support their intents even if they cannot get to see the executable actions. The intent MnS producer needs to support means to inform the Intent MnS consumers of the applicable Intent Logic when needed.

It can be expected that an intent fulfilment function has library with of intent logic units form which specific which intent logic units are executed in some sequence to fulfil a given intent expectation or to achieve a given expectation target. Each intent logic unit is a wrapper around the logic or command sequences that need to be executed to achieve a specific intent expectation or expectation target. In other words, the intent logic unit is a description of the actions that need to be taken to achieve a certain outcome. Accordingly, each intent logic unit may be characterized by an identifier; a name and the logic or sequence of steps to be executed. It may, however, also have a description intended for human users who may either want to revise, reuse, enable or disable the intent logic unit

The intent MnS producer may support multiple intent logic units which can be combined in different flows to fulfil a given intent expectation. The Intent MnS consumer may wish to know the available candidate intent logic units and their flows to be sure that they are appropriate to fulfil their desired intent.

#### 5.1.M.2	Requirements

REQ-Intent_CL_01: The intent MnS producer shall have a capability enabling an authorized  MnS consumer (e.g., an operator) to be informed about the set of intent logic units available at the intent MnS producer.

REQ-Intent_CL_02: The intent MnS producer shall have a capability enabling an authorized  MnS consumer (e.g., an operator) to be informed about the flow of a specific intent logic unit available at the intent MnS producer.

REQ-Intent_CL_03: The intent MnS producer shall have a capability enabling an authorized  MnS consumer (e.g., an operator) to enable or disable one or more intent logic units available at the intent MnS producer.

End of Changes