3GPP TSG-SA5 Meeting #149 	S5-234085

Berlin, Germany, 22 - 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0050 | rev | - | Current version: | 17.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.312 add procedure of intent conflict resolution based on intent preemption |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_ph2 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The potential solution of intent conflicts based on preemption has been discussed and agreed in TR 28.912. This CR is to normatively propose the proceduer of intent conflict resolution based on intent pre-emption. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The proceduer of intent conflict resolution based on intent preemption have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The intent conflict resolution based on intent preemption shall not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3/ Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

6.3	Procedures for intent management

6.3.X	Resolution of an intent conflict based on  preemption

Figure 6.3.X-1 illustrates the procedure for intent conflict resolution based on  preemption.

Figure 6.3.2-1: Procedure for resolution of an intent conflict based on preemption

Step1 check intent priority level of intents which are in conflict.

Step2

- In case intent priority level are different, MnS producer prioritize higher priority level intent, if the new intent is prioritized, go to step 4

- In case intent priority level are same, MnS producer check the intent preemption capability of new intent and intent preemption protection of existing one, and handle as Step3.

Step3

- If intent preemption protection of existing intent is not preemptable, MnS producer prioritize existing intent (can’t proceed new intent)

- If intent preemption protection of existing intent is preemptable and intent preemption capability of new intent shall trigger, MnS producer prioritizes the new intent over the existing intent. (go to step 4)

- If intent preemption protect of existing intent is preemptable and intent preemption capability of new intent shall not trigger, MnS producer prioritizes the existing intent (can’t proceed new intent)

Step 4

- When the new intent is prioritized over the existing intent which has conflict with the new intent, the MnS producer can take the following action,

    - Notify the conflict and ask for approval of termination or suspension to the MnS consumer.

End of changes

Second change

Annex A (informative):
PlantUML source code

A.1	Procedures for intent management

A.1.1	Create an intent

@startuml

title "[Create an intent]"

actor "MnS Consumer" as MnS_Consumer

participant "MnS Producer" as MnS_Producer

Collections "ManagedEntity" as ManagedEntity

MnS_Consumer -> MnS_Producer: 1. Request to create an intent instance (list of attributes of intent IOC)

MnS_Producer -> MnS_Producer: 2. Create and configure intent MOI

MnS_Producer -> MnS_Consumer: 3. Response for create an intent instance

MnS_Producer -> MnS_Producer: 4. Perform the feasibility check of the intent instance

alt feasibility check result is "Feasible"

Ref over MnS_Producer, ManagedEntity: 5a. Perform service or network management tasks

loop

Ref over MnS_Producer, ManagedEntity: 6. Evaluate intent fulfilment

opt

Ref over MnS_Producer, ManagedEntity: 7. Adjust to fulfil the intent requirement

end

end

MnS_Producer -> MnS_Consumer:8. Notify of intent fulfillment Information\n (DN of intent MOI, FulfilmentInfo)

else feasibility check result is "inFeasible"

MnS_Producer -> MnS_Consumer: 5b. Notify of intent infeasibile information

end

hide footbox

@enduml

A.1.2	Modify an intent

@startuml

title "[Create an intent]"

actor "MnS Consumer" as MnS_Consumer

participant "MnS Producer" as MnS_Producer

Collections "ManagedEntity" as ManagedEntity

MnS_Consumer -> MnS_Producer: 1. Request to create an intent instance (list of attributes of intent IOC)

MnS_Producer -> MnS_Producer: 2. Create and configure intent MOI

MnS_Producer -> MnS_Consumer: 3. Response for create an intent instance

MnS_Producer -> MnS_Producer: 4. Perform the feasibility check of the intent instance

alt feasibility check result is "Feasible"

Ref over MnS_Producer, ManagedEntity: 5a. Perform service or network management tasks

loop

Ref over MnS_Producer, ManagedEntity: 6. Evaluate intent fulfilment

opt

Ref over MnS_Producer, ManagedEntity: 7. Adjust to fulfil the intent requirement

end

end

MnS_Producer -> MnS_Consumer:8. Notify of intent fulfillment Information\n (DN of intent MOI, FulfilmentInfo)

else feasibility check result is "inFeasible"

MnS_Producer -> MnS_Consumer: 5b. Notify of intent infeasibile information for \n the created intent instance

end

A.1.3	Delete an intent

@startuml

title "[Delete an intent]"

actor "MnS Consumer" as MnS_Consumer

participant "MnS Producer" as MnS_Producer

MnS_Consumer -> MnS_Producer: 1.Requst to delete an intent instance\n('objectInstance' of intent MOI)

MnS_Producer -> MnS_Producer: 2.Delete the intent MOI

MnS_Producer -> MnS_Consumer: 3.Response for delete an intent instance\n ('objectInstance' of intent MOI)

hide footbox

@enduml

A.1.4	Query an intent

@startuml

title "[Query an intent]"

actor "MnS Consumer" as MnS_Consumer

participant "MnS Producer" as MnS_Producer

MnS_Consumer -> MnS_Producer: 1.Request to query an intent instance(objectInstance,AttributeList of Intent IOC )

MnS_Producer -> MnS_Producer: 2.Query the intent MOI

MnS_Producer -> MnS_Consumer: 3.Response for query an intent instance \n (objectClass,objectInstance,status,list of [Attribute,value] of Intent IOC )

hide footbox

@enduml

A.1.X	Resolution of an intent conflict based on  preemption

@startuml

title "[pre-emption handling]"

actor "MnS Consumer1" as MnS_Consumer1

actor "MnS Consumer2" as MnS_Consumer2

participant "MnS Producer" as MnS_Producer

Note over MnS_Consumer2,MnS_Producer

created  intent

end Note

Note over MnS_Consumer1

decide to create new intent

end Note

MnS_Consumer1 -> MnS_Producer: Request to create an intent instance

Note over MnS_Producer

Detect new intent and existing intent are in conflict

end Note

Note over MnS_Producer

Step1 check intent prioriy level of intents which are in conflict

Compare the new intent(MnS_Consumer1) prioritylevel and e existing(MnS_Consumer2) intent prioritylevel

end Note

group  Step2

alt intent priority levels are different  (the new intent prioritylevel != the existing intent prioritylevel)

alt the new intent prioritylevel > the existing intent prioritylevel

Ref over  MnS_Producer , MnS_Consumer1

Step4

end

else the new intent prioritylevel  < the existing intent prioritylevel

MnS_Producer -> MnS_Consumer1: Response (Failed of new intent)

end

else intent priority levels are same

group Step3

Note over MnS_Producer

check the intent preemption capability of new intent,

intent preemtption protection of existing one

end Note

alt intent preemtption protection of existing intent = not preemtable

Note over MnS_Producer

MnS producer prioritizes the existing intent (can't proceed new intent)

end Note

MnS_Producer -> MnS_Consumer1: Response (Failed of new intent)

else intent preemtption protection of existing intent = preemtable, and intent preemtpion capability of new intent = shall trigger

Note over MnS_Producer

MnS producer prioritizes the new intent over the exising

end Note

Ref over  MnS_Producer , MnS_Consumer1

Step4

end

else intent preemtption protection of existing intent = preemtable ,and intent preemtpion capability of new intent = shall not trigger

Note over MnS_Producer

MnS producer priotize existing intent (can't proceed new intent)

end Note

MnS_Producer -> MnS_Consumer1: Response (Failed of new intent)

end

end group

end

end group

== Step4 ==

MnS_Producer -> MnS_Consumer2: Notification(Ask for approval of termination/suspention.)

hide footbox

@enduml

End of changes