3GPP TSG-SA5 Meeting #149	S5-234086

# Berlin, Germany, 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0413 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Add MA PDU session creation measurements for ATSSS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecomunication Corp.,Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | ATSSS can reduce mobile network congestion and deliver better customer experience. The key enabler of ATSSS is the Multi Access-PDU (MA PDU) Session. Therefore, it is necessary to monitor the performance of MDA PDU session establishment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add ATSSS related performance measurement including: - Number of MA PDU session creation requests - Number of successful MA PDU session creations - Number of failed MA PDU session creations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot monitor the performance of ATSSS and effectively solve the corresponding network problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2 , 5.3.1.X(new), A.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 23.501 [4].

CHO	Conditional Handover

DAPS	Dual Active Protocol Stack

HO	Handover

kbit	kilobit (1000 bits)

LHO	Legacy Handover

MN	Master Node.

NG-RAN	Next Generation Radio Access Network

PI	Performance Indicator

SN	Secondary Node.

TEID	Tunnel Endpoint Identifier

MA PDU	Multi-Access PDU

2nd change

#### 5.3.1.X	MA PDU session management

5.3.1.x.1	Number of MA PDU session creation requests

a)	This measurement provides the number of MA PDU sessions requested to be created by the SMF.

b)	CC

c)	On receipt by the SMF from AMF of Nsmf_PDUSession_CreateSMContext Request including "MA PDU Request" indication (see clause 4.22.2 of TS 23.502 [7]). Each MA PDU session requested to be created is added to the relevant subcounter per S-NSSAI.

d)	Each subcounter is an integer value

e)	SM.MAPDUSessionCreationReq.SNSSAI,
Where SNSSAI identifies the S-NSSAI.

f)	SMFFunction

g) 	Valid for packet switched traffic

h) 	5GS

5.3.1.x.2	Number of successful MA PDU session creations

a)	This measurement provides the number of MA PDU sessions successfully created by the SMF.

b)	CC

c)	On transmission by the SMF to AMF of Nsmf_PDUSession_CreateSMContext Response indicating the requested MA PDU session creation is accepted (see TS 29.502 [14]). Each MA PDU session successfully created is added to the relevant subcounter per S-NSSAI.

d)	Each subcounter is an integer value

e)	SM.MAPDUSessionCreationSucc.SNSSAI,
Where SNSSAI identifies the S-NSSAI.

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

5.3.1.x.3	Number of failed MA PDU session creations

a)	This measurement provides the number of MA PDU sessions failed to be created by the SMF.

b)	CC

c)	On transmission by the SMF to AMF of Nsmf_PDUSession_CreateSMContext Response indicating a failure or rejection of the corresponding MA PDU session creation request (see TS 29.502 [14]). Each MA PDU session failed or rejected to be created is added to the relevant subcounter per failure cause.

d)	Each subcounter is an integer value

e)	SM.MAPDUSessionCreationFail.cause,
Where cause indicates the failure cause for the MA PDU session creation.

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

3rd change

# A.X	Use case of Monitoring of MA PDU session management for ATSSS

The ATSSS feature enables a multi-access PDU Connectivity Service, which can exchange PDUs between the UE and a data network by simultaneously using one 3GPP access network and one non-3GPP access network. The multi-access PDU Connectivity Service is realized by a Multi-Access PDU (MA PDU) Session.

When establishing an MA PDU session, the AMF informs the SMF that the request is for a MA PDU Session. The number and success rate of MA PDU session creations, and the number of PDU sessions running on the SMF are some of the basic performance measurements for monitoring the performance of MA PDU session establishment. And the failed MA PDU session creations measured per cause are helpful to pinpoint and solve the network issues in case the performance is below the expectation.

End of changes