3GPP TSG-SA5 Meeting #149 	S5-234111

Berlin, Germany, 22 - 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0049 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add information elements related to fault management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-5-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In eMDAS_Ph2, the new requirements might lead to add some IE for analytics report for fault management analytics capability. Furthermore, the analytics reporting may have to include the reponse of the result for this analytics, for example, the availability of this analytics output. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the information elemements related to analytics outputs for fault management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Analytics output for fault prediction analysis can not cover some information of analytics reporting when requiring. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.3.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

### 8.3.1	Common information element definitions

The common information elements of the analytics outputs are defined in Table 8.3.1-1.

Table 8.3.1-1: Common information elements of analytics outputs

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| mDAType | It indicates the MDA type. The allowed values are the MDA type names defined for each MDA capability respectively in clause 8.4. | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| analyticsId | The identifier of the analytics output. | M | type: string multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| analyticsOutputGenerationTime | It indicates the time when the analytics output is generated. Examples: "20:15:00", "20:15:00-08:00" (for 8 hours behind UTC). | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

2nd change

##### 8.4.3.1.3	Analytics output

The specific information elements of the analytics output for failure prediction analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.3.1.3-1.

Table 8.4.3.1.3-1: Analytics output for fault prediction analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| failurePredictionObject | Indication of NR cells or NFs where the failure related issues occurred or potentially occur. | M | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| - objType | This field holds the NFtype for analtycs output for management domain. The valid value of this field will be network functions like SMF, AMF, gNB etc which are specified in TS 23.501. | O | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| potentialFailureType | Indication of type of issues that can cause the failures. The valid value(s) of this field may be derived from the management daomain and the managed functions. NOTE 1: The values can be defined as a list of example values: "Operational Violation", "Physical Violation" and "Time Domain Violation",see alarmType described in TS 28.532 [11], and other specific type of issues , e.g., "virtual resource capacity ". | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| eventTime | This field holds the time of potential failure predicted. Examples: "20:15:00", "20:15:00-08:00" (for 8 hours behind UTC). | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| issueID | This filed holds the ID of this failure prediction which is reported. When reports, this identifier can be used to provide the information to management system to maintain. | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| perceivedSeverity | This field holds the value to indicate relative level of urgency for operator attention. NOTE 2: The value can be Critical, Major, Minor, Warning, Indeterminate, Cleared, see Recommendation ITU-T X.733 [27]. | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of changes