3GPP TSG-SA5 Meeting #149 	S5-234163

# Berlin, Germany, 22nd May 2023 - 26th May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0060 | rev | - | Current version: | 17.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.312 Add general concept and requirements for intent conflict |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_ph2 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | New generic capability and corresponding requirements for intent confliction has been studied in TR 28.912 in clause 4.2 and recommended to be introduced in TS 28.312. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add introduction and requirements for intent conflict management based on clause 4.2 in TR 28.912. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The management capability for intent conflict management is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.X	Generic capabilities for intent driven MnS

5.X.1	Intent conflict management

5.X.1.1	Introduction

The MnS consumer may create an intent containing two or more intent expectations, and each intent expectation may contain multiple expectation targets. For example, a Radio Network related intent may express an intent with targets on radio network parameters (like, downlink transmit power, remote electrical tilt) or on KPIs (like, DL UE throughput target, average RSRP target, coverage area). On receiving and after analysing the intent, the MnS producer may realize that the intent expectations or expectation targets in one intent are contradicted, i.e. the MnS producer may detect conflicts in the intent. Also, an intent is considered to have conflict with other intents if the requirements (includes intent expectation and corresponding expectation targets) stated in one intent is conflicted with the requirements (includes intent expectation and corresponding expectation targets) stated in another intent. Following are the intent related conflict scenarios:

- Target conflict, which represents the conflict between two or more expectation targets within the same intent expectation.

- Expectation conflict, which represent the conflict between two or more intent expectations with the same intent

- Intent conflict, which represent the conflict between two or more different intents.

For the intent conflict, the two or more intents may be proposed by the same MnS consumer, or may be proposed by different MnS consumers. An example of the latter is that the MnS producer cannot satisfy the intents of two MnS consumers simultaneously.

When such conflicts are detected, the MnS producer needs to notify the MnS consumer about the conflict, indicating the intent, intent expectations or expectation targets which give rise to the conflict. Additionally, the MnS producer may also notify the MnS consumer about the additional information (e.g. the impact for other expectation targets when fulfil the specified expectation targets in the same or different intent) for the conflict.

Thereby, MnS consumer may give some intent conflict handling guidelines (e.g. assign priority for such intent or intent expectation or expectation targets) to MnS producer to solve such intent conflict or eliminate the terminated state which is caused by the reason of conflict detected.

#### 5.X.1.2	Requirements

REQ-Intent_conflict-CON-1: The intent driven MnS should have the capability to inform the authorized MnS consumer about intent related conflicts if they are identified.

REQ-Intent_conflict-CON-2: The intent driven MnS should have capability enabling MnS consumer to provide guidance to assist the MnS producer to resolve conflicts between intents.

End of Change