3GPP TSG-SA5 Meeting #149 	S5-234177

Berlin, Germany, 22 - 26 May 2023

# Source:	China Mobile,Huawei

# Title:	pCR TR 28.909 Add potential KEI for fault management

# Document for:	approval

Agenda Item:	6.7.2.2

1	Decision/action requested

The group is asked to discuss and approval.

2	References

[1]	3GPP draft TR 28.909: “Management and orchestration; Study on evaluation of autonomous network levels v0.2.1”.

3	Rationale

This contribution proposes to add key issues for KEI of autonomous network levels evaluation for fault management based on concept of KEI in clause 4.1.3.

4	Detailed proposal

It proposes to make the following changes to TR 28.909[1].

1st Change

5.X	Key Issue# 5.X: KEI for evaluating autonomy capability for fault management

5.X.1	Description

Key effectiveness indicator (KEI) describes the effective of introducing autonomy capability into telecom system. Regarding the fault management related scenarios, following aspects can be considered as evaluation effect for autonomy capability for radio fault management.

-	Automation effect for corresponding radio fault management tasks by introducing autonomy capability. For example, telecom system A without autonomy capability can recognize 30% of total network and service failures and analyse the root cause for 20% network and service failures, while after introducing certain autonomy capability, telecom system A can analyse recognize 80% of total network and service failures and analyse the root cause for 70% network and service failures.

- Recovery time reduction for radio fault management. For example, telecom system A without autonomy capability needs one week to analyse the detailed root cause of the failure and recover the network, while after introducing certain autonomy capability, telecom system A only needs one day to analyse the detailed root cause of the failure and recover the network.

5.X.2	Potential Solution

Based on the description in clause 5.X.1, following are two dimensions used to evaluate the autonomy capability for radio fault management:

-	Automation ratio of fault management, including the automation ratio for corresponding fault management tasks (including task of fault recognition and task of fault root cause analysis). For example, fault root cause analysis automation ratio represents the proportion of the number of network and service failures whose root cause analysed by the telecom system automatically to the total number of network and service failures.

-	Reduction ratio of failure recovery time, which means the reduced ratio of the time for telecom system taken for failure recovery. For example, the reduction ratio of failure recovery time can be proportion of the reduced time for recover to the original time for recover before introducing the autonomy capability for fault management.

End of Changes