3GPP TSG-SA5 Meeting #149 	S5-234183

Berlin, Germany, 22 - 26 May 2023

# Source:	Huawei

# Title:	Discussion Paper for roaming charging profiles update with profile change time

# Document for:	Endorsement

Agenda Item:	7.5.2

1	Decision/action requested

The group is asked to discuss and endorse the proposal.

2	References

[1]	3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

3	Rationale

The solution #2.17 in TR 28.827 allows the H-CHF provide multiple Roaming Charging Profiles in the charging data response at the PDU session establishment, some Roaming Charging Profiles are with Profile change time, some Roaming Charging Profiles are without Profile change time. The effective time is different for the two types of roaming charging profiles, i.e.

- the Roaming Charging Profiles with Profile change time are applied at the Profile change time.

- the Roaming Charging Profiles without Profile change time are applied immediately.

This solution could provide support to special use cases (e.g. holiday or event) that can determine the roaming charging profile update in advance and take effect at future time. It could avoid network congestion due to a large number of requests simultaneously sent from one place.

For example, during world cup, operator might want to increase the upper limit for data volume, e.g. the trigger “Expiry of data volume limit per QoS Flow”, and reset this value after the world cup. Requesting for the RCP update during the event would result in network congestion since a large number of roaming subscribers are gathering at one place. It is thus necessary to pre-define the changes and enable it at a given time. There are many cases like this, including but not limited to: new year eve, sports event, sales season, etc.

In addition, the solution could also support pending policy counter for spending limit in TS 29.594.

4	Proposal for endorsement

It is proposed that solution #2.17 be taken into conclusion of clause 7.2 (pCR S5-234189).