3GPP TSG-SA5 Meeting #149 	S5-234262

Berlin, Germany, 22 - 26 May 2023

# Source:	Huawei

# Title:	pCR TR 28.925 Update solution and add conclusion and recommendation for issue#13 Analysis on IRP specification needs to support SBMA

# Document for:	Approval

Agenda Item:	6.8.1.2

1	Decision/action requested

Discuss and approve on the proposal.

2	References

[1]	3GPP TR 28.925 enhancement of service based management architecture v0.a.0

[2]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

3	Rationale

The potential solution of Issue#13 in TR 28.925 [1] identifies the three categories of 3GPP IRPs:

- Category 1: correponding SBMA specification is needed (for those items whose value in the thrid column of Table 4.13.2-1 and 4.13.2-2 is “yes”.)

- Category 2: correponding SBMA specification is not needed (for those items whose value in the thrid column of Table 4.13.2-1 and 4.13.2-2 is “Not needed” or “Not applicable”.)

- Category 3: whether or not the correponding SBMA specification is needed is not concluded in this study (for those items whose value in the thrid column of Table 4.13.2-1 and 4.13.2-2 is “discontinue MnS as there is lack of operators requirements”.)

This contribution is to complete the analysis and provide recommendations.

4	Detailed proposal

This document proposes the following changes in TR 28.925 [1].

1st Change

4.13.2	Potential solutions

Following table 4.13.2-1 provides the overview analysis on 3GPP Interface IRPs and the related SBMA specifications.

Table 4.13.2-1: overview analysis on 3GPP Interface IRPs and the related SBMA specifications

| 3GPP Interface IRP specifications |  | Whether correponding SBMA specification is needed | Status |
| --- | --- | --- | --- |
| Generic/Common Interface IRPs | Name convention for Managed Objects (32.300) | Yes | The name convention is used in SBMA. |
|  | Notification IRP (32.301/2/6) | Yes | SBMA specification is already defined (TS 28.532) |
|  | Generic IRP (32.31x) | Not needed as it’s management of interface IRPs |  |
| FM related interface IRPs | Alarm IRP (32.111-x) | Yes | SBMA specification is already defined (TS 28.532) |
|  | Advanced Alarm Management IRP (32.12x) | Yes | Aready covered by FS_FSEV SID |
|  | Test management IRP (32.32x) | discontinue MnS as there is lack of operators requirements. |  |
| CM related interface IRPs | Basic CM IRP (32.60x) | Yes | SBMA specification is already defined (TS 28.532) |
|  | Bulk CM IRP (32.61x) | Yes | YANG (RFC 7950) and Netconf (RFC6241) allows updating a single boolean attribute/leaf or the whole configuration. Individual and bulk are not handled separately. Restful |
|  | Kernel CM IRP (32.66x) | Yes | YANG (RFC 7950) and Netconf (RFC6241) allows updating a single boolean attribute/leaf or the whole configuration. Individual and bulk are not handled separately. Part of operations defined in Kernel CM IRP have already a SBMA specification, e.g. notifyMOICreation, notifyMOIDeletion, etc. (cf. TS 28.532). |
| PM file format/collection & Trace related Interface IRPs | Performance Management IRP (32.41x) | Yes | SBMA specification is already defined (TS 28.532 and TS 28.550) |
|  | Performance Measurement collection (32.43x) | Yes | SBMA specification is already defined (TS 28.532 and TS 28.550) |
|  | Trace Management IRP (32.42x, 32.44x) | Yes | Already reused for SBMA |
|  | Notification Log IRP (32.33x) | Yes | Aready covered by FS_eSBMAe SID (see FS_eSBMAe_WoP#7) |
| Supporting interface IRPs | File Transfer IRP (32.34x) | Yes | SBMA specification is already defined (TS 28.537 and TS 28.622) |
|  | Communication Surveillance IRP (32.35x) | Yes | SBMA specification is already defined (TS 28.537,TS 28.622, TS 28.532) |
|  | Entry Point IRP (32.36x) | Yes | SBMA specification is already defined (5GDMS, TS 28.537, TS 28.622) |
| Other interface IRPs | Partial Suspension of Itf-N IRP (32.38x) | discontinue MnS as there is lack of operators requirements. |  |
|  | Delta synchronization IRP (32.39x) | discontinue MnS as there is lack of operators requirements. |  |
| SON related Interface IRPs | Self-Configuration IRP (32.50x) | Yes | SBMA specification for PnC is already defined (TS 28.314/5/6) SBMA specification for SBMA is covered by RANSC WI |
|  | Software Management IRP (32.53x) | discontinue MnS as there is lack of operators requirements. | Analysis is covered by Issue #6 in clause 4.6 |

Following table 4.13.2-2 give the overview analysis on  3GPP NRM IRPs to derive which IRP spepcifications needs to be convert to SBMA specifications in the future.

Table 4.13.2-2: overview analysis on 3GPP NRM IRPs and the related SBMA specifications

| 3GPP NRM IRP specifications |  | Whether correponding SBMA specification is needed | Status |
| --- | --- | --- | --- |
| FMC model | Umbrella Information Model (28.620) | Yes | Reused for SBMA |
| Common NRM IRP | Generic NRM IRP (28.621/2/3) | Yes | Covered by Issue #4 in clasue 4.1. |
| 3GPP Access Network (AN) related NRM IRPs | Generic RAN NRM IRP (28.661/2/3) | Yes | Covered by Issue #4 in clasue 4.3 |
|  | GERAN NRM IRP (28.654/5/6) | Not applicable |  |
|  | UTRAN NRM IRP (28.651/2/3) | Not applicable |  |
|  | E-UTRAN NRM IRP (28.657/8/9) | Yes | Covered by Issue #4 in clasue 4.3 |
|  | Generic RAN NRM IRP 28.661/2/3) | Yes | There is a dependency from 28.541 to 28.662 |
| 3GPP Core Network (CN)/IMS related NRM IRPs | CN NRM IRP (28.701/2/3) | Not applicable |  |
|  | CN NRM IRP (28.704/5/6) | Not applicable |  |
|  | EPC NRM IRP (28.707/8/9) | Not applicable |  |
| 3GPP H(e)NB related NRM IRPs | HNS NRM IRP (28.671/2/3) | Not applicable |  |
|  | HeNS NRM IRP (28.674/5/6) | Not applicable |  |
| Service management related NRM IRPs | SuM NRM IRP (28.751/2/3) | discontinue MnS as there is lack of operators requirements. |  |
| Supporting NRM IRPs | Inventory Management NRM IRP (28.631/2/3) | Yes | Covered by Issue #7 in clasue 4.7 and5.4. They are already included in (informative) annex E in TS 28.533 about 5G specifications though still lacking SBMA-compatible solution set |
|  | Transport Network NRM IRP (28.731/2/3) | Not applicable |  |
|  | Signalling Transport NW IF NRM IRP (28.734/5/6) | Not applicable |  |
|  | State management data definition IRP (28.624/5/6) | Yes | Most of the relevant contents from 28.625 have been copied into 28.622/28.541. |
| Self-Organizing Networks NRM IRPs | SON Policy NRM IRP (28.627/8/9) | Not applicable |  |
| Non-3GPP access interworking NRM IRPs | EPC and non-3GPP access interworking system Network Resource Model (NRM) IRP (28.611/2/6) | Not applicable |  |
| Wireless Local Area Network NRM IRP | WLAN NRM IRP (28.681/2/3) | Not applicable |  |

Next Change

5	Conclusion and Recommendation

5.13	Issue #13: Analysis on IRP specification needs to support SBMA

The potential solution identifies the following IRP features are necessary to support SBMA:

For Interface IRP:

- Name convention for Managed Objects (32.300)

- Notification IRP (32.301/2/6).

- Alarm IRP (32.111-x)

- Advanced Alarm Management IRP (32.12x)

- Basic CM IRP (32.60x)

- Bulk CM IRP (32.61x)

- Kernel CM IRP (32.66x)

- Performance Management IRP (32.41x)

- Performance Measurement collection (32.43x)

- Trace Management IRP (32.42x, 32.44x)

- Notification Log IRP (32.33x)

- File Transfer IRP (32.34x)

- Communication Surveillance IRP (32.35x)

- Entry Point IRP (32.36x)

- Self-Configuration IRP (32.50x)

For NRM IRP:

- Umbrella Information Model (28.620)

- Generic NRM IRP (28.621/2/3)

- Generic RAN NRM IRP (28.661/2/3)

- E-UTRAN NRM IRP (28.657/8/9)

- Generic RAN NRM IRP 28.661/2/3)

- Inventory Management NRM IRP (28.631/2/3)

- State management data definition IRP (28.624/5/6)

End of change