3GPP TSG-SA5 Meeting #149 	S5-234321

Berlin, Germany, 22 - 26 May 2023

# Source:	Ericsson

# Title:	Rel-18 pCR 28.203 Adding SCUR for number of UEs per NSSAI

# Document for:	Approval

Agenda Item:	7.4.1

# 1	Decision/action requested

Include the proposed changes in TS 28.203.

# 2	References

[1]		3GPP TS 28.203: "Charging management; Network slice admission control charging in the 5G System (5GS)"

# 3	Rationale

Adding flow supporting SCUR for number of UEs per NSSAI.

# 4	Detailed proposal

First change

##### 5.2.2.2.x	Number of UEs per network slice – SCUR charging

The following figure 5.2.2.2.x-1 describes a Registration charging message flow in SCUR charging, based on figure  4.2.11.2-1 of  3GPP TS 23.502 [4] description:

Figure 5.2.2.2.x-1: Number of UEs per S-NSSAI – SCUR charging

1.	Steps 1 to 3: per 3GPP TS 23.502 [4] Figure 4.2.11.2-1 Number of UEs per network slice availability check and update procedure.

2.	Trigger met: "Nb of UEs threshold crossed for initial" trigger is met for one specific S-NSSAI.

3.	Charging Data Request [Initial, Quota request, Number]: The NSACF sends the request to the CHF with request for quota and "Number of UEs".

4.	Account balance, Rating: the CHF may rate the requests, and check account balance.

5.	Open CDR: CHF may open a CDR with the “Number of UEs”.

6.	Charging Data Response [Initial, Quota grant, Triggers]: The CHF grants authorization to NSACF, and may include granted quota and new triggers setting.

7.	Step 4: per 3GPP TS 23.502 [4] Figure 4.2.11.2-1 Number of UEs per network slice availability check and update procedure.

8.	Steps 1 to 3: per 3GPP TS 23.502 [4] Figure 4.2.11.2-1 Number of UEs per network slice availability check and update procedure.

9.	Trigger met: granted quota threshold or "Nb of UEs threshold crossed upwards/downwards" trigger is met for one specific S-NSSAI.

10.	Charging Data Request [Update, Quota request, Number]: The NSACF sends the request to the CHF with request for quota and "Number of UEs"

11.	Account balance, Rating: the CHF may rate the requests, and check account balance.

12.	Update CDR: CHF may update the CDR with the “Number of UEs”.

13.	Charging Data Response [Update, Triggers]: The CHF grants authorization to NSACF, and may include granted quota and new triggers setting.

14.	Step 4: per 3GPP TS 23.502 [4] Figure 4.2.11.2-1 Number of UEs per network slice availability check and update procedure.

15.	Steps 1 to 3: per 3GPP TS 23.502 [4] Figure 4.2.11.2-1 Number of UEs per network slice availability check and update procedure.

16.	Trigger met: " Nb of UEs threshold crossed for termination" trigger is met for one specific S-NSSAI.

17.	Charging Data Request [Termination, Number]: The NSACF sends the request for termination to the CHF with "Number of UEs".

18.	Account balance, Rating: the CHF may rate the requests, and check account balance.

19.	Close CDR: CHF may close the CDR with the “Number of UEs”.

20.	Charging Data Response [Termination]: The CHF informs the NSACF of the result of the request.

End of changes