3GPP TSG-SA5 Meeting #149 	S5-234322

Berlin, Germany, 22 - 26 May 2023

# Source:	Ericsson

# Title:	Rel-18 pCR 28.203 Adding PEC for number of UEs per NSSAI

# Document for:	Approval

Agenda Item:	7.4.1

# 1	Decision/action requested

Include the proposed changes in TS 28.203.

# 2	References

[1]		3GPP TS 28.203: "Charging management; Network slice admission control charging in the 5G System (5GS)"

# 3	Rationale

Adding flow supporting PEC for number of UEs per NSSAI.

# 4	Detailed proposal

First change

##### 5.2.2.2.y	Number of UEs per network slice – PEC charging

The following figure 5.2.2.2.y-1 describes a Registration charging message flow in SCUR charging, based on figure  4.2.11.2-1 of  3GPP TS 23.502 [4] description:

Figure 5.2.2.2.y-1: Number of UEs per S-NSSAI – PEC charging

1.	Steps 1 to 3: per 3GPP TS 23.502 [4] Figure 4.2.11.2-1 Number of UEs per network slice availability check and update procedure.

2.	Step 4: per 3GPP TS 23.502 [4] Figure 4.2.11.2-1 Number of UEs per network slice availability check and update procedure.

3.	Trigger met: "Nb of UEs threshold crossed upwards/downwards" trigger is met for one specific S-NSSAI.

4.	Charging Data Request [Event, Number]: The NSACF sends the request to the CHF with "Number of UEs".

5.	Create CDR: CHF creates a CDR with the “Number of UEs”.

6.	Charging Data Response [Event]: The CHF informs the NSACF of the result of the request.

End of changes