3GPP TSG-SA5 Meeting #149 	S5-234327

Berlin, Germany, 22 - 26 May 2023

# Source:	Ericsson

# Title:	Rel-18 pCR 28.826 Adding solution using container reference for message size

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

Include the proposed changes in TR 28.826.

# 2	References

[1]		3GPP TR 28.826: " Study on Nchf charging services phase 2 improvements and optimizations"

# 3	Rationale

Adding new solution using container reference for message size.

# 4	Detailed proposal

First change

#### 5.2.5.x	Solution #2.x: Introduce a container reference

A possible solution for key issues 2x and 2b covering requirement REQ-CH_REQ-x, trigger setting for event based charging.

Introduction of the possibility to refer to another container (PDU or QBC) if they contain the same information.

Table 5.2.5.x: Additions to definition of type PDUContainerInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pDUContainerInfoId | string | OC | 0..1 | Identifies the PDU container information within a charging data request. |  |
| pDUContainerInfoRef | string | OC | 0..1 | Reference to another PDU container information within a charging data request that contains the charging data i.e., this means that no charging data is required to be included in this container |  |
| NOTE 1: Either the pDUContainerInfoId or pDUContainerInfoRef may be included. |  |  |  |  |  |

Table 5.2.5.x: Additions to definition of type QFIContainerInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| qFIContainerInfoId | string | OC | 0..1 | Identifies the QFI container information within a charging data request |  |
| qFIContainerInfoRef | string | OC | 0..1 | Reference to another QFI container information within a charging data request that contains the charging data i.e., this means that no charging data is required to be included in this container |  |
| NOTE 1: Either the qFIContainerInfoId or qFIContainerInfoRef may be included. |  |  |  |  |  |

End of changes