3GPP TSG-SA5 Meeting #149	S5-234369

# Berlin, Germany, 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0435 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR for TS28.552 Add packet measurements valid for split gNB deployment scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ePM_KPI_5G |  |  |  |  |  | Date: |  |  | 2023-05-1 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Existing 5.1.3.1 Packet Loss Rate and 5.1.3.2 Packet Drop Rate cannot provide the numerator and denominator for accurately obtaining these indicators. The packet loss rate varies with the QoS and traffic. Therefore, add accurate number of packets to facilitate O&M monitoring and analysis. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add packet measurements valid for split gNB deployment scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The accurate number of packets cannot be obtained. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3.x, 5.1.3.x.1, 5.1.3.x.2, 5.1.3.x.3, A.2, A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### Start of changes

5.1.3.x	Packet  measurements

5.1.3.x.1 Total number of UL PDCP SDU Packets

a) This measurement provides the total number of PDCP SDU packets which are expected received at gNB-CU-UP.  Only user-plane traffic (DTCH) and only PDCP SDUs that have entered PDCP (and given a PDCP sequence number) are considered.  The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3), and subcounters per supported S-NSSAI.

b)	CC.

c)	This measurement is obtained as:  Total number of UL PDCP sequence numbers (also including missing sequence numbers) of a bearer, starting from the sequence number of the first packet delivered by UE PDCP to gNB-CU-UP until the sequence number of the last packet. If transmission of a packet might continue in another cell, it shall not be included in this count. Separate counters are optionally maintained for mapped 5QI (or QCI for NR option 3) and per supported S-NSSAI.

d)	Each measurement is an integer value. If the optional QoS and S-NSSAI level measurements are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

e)	The measurement name has the form DRB.TotalPdcpPacketUl and optionally DRB.TotalPdcpPacketUl.QOS where QOS identifies the target quality of service class, and DRB. TotalPdcpPacketUl.SNSSAI where SNSSAI identifies the S-NSSAI.

f)	GNBCUUPFunction.

NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality). NRCellCU measurement applies only for 2-split deployment.

5.1.3.x.2 Total number of DL RLC SDU Packets in gNB-DU

a)	This measurement provides the total number of RLC SDU packets which are received on the downlink in the gNB-DU from gNB-CU-UP. Only user-plane traffic (DTCH) is considered. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3), and subcounters per supported S-NSSAI.

b)	CC.

c)	This measurement is obtained as: the total Number of DL packets for data radio bearers that were received from gNB-CU-UP. Separate counters are optionally maintained for mapped 5QI (or QCI for NR option 3) and per supported S-NSSAI.

d)	Each measurement is an integer value. If the optional QoS and S-NSSAI level measurement are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

e)	The measurement name has the form DRB. TotalRlcPacketDl and optionally DRB.TotalRlcPacketDl.QOS 
where QOS identifies the target quality of service class, and DRB.TotalRlcPacketDl.SNSSAI where SNSSAI identifies the S-NSSAI.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

5.1.3.x.3 Total number of DL PDCP SDU Packets in gNB-CU-UP

a)	This measurement provides the total number of PDCP SDU packets which are received on the downlink in the gNB-CU-UP. Only user-plane traffic (DTCH) is considered. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3), and subcounters per supported S-NSSAI.

b)	CC.

c)	This measurement is obtained as: Number of DL packets for data radio bearers that has entered PDCP upper SAP. Separate counters are optionally maintained for mapped 5QI (or QCI for NR option 3) and per supported S-NSSAI.

d)	Each measurement is an integer value. If the optional QoS and S-NSSAI level measurement are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

e)	The measurement name has the form DRB.TotalPdcpPacketDl and optionally DRB.TotalPdcpPacketDl.QOS 
where QOS identifies the target quality of service class, and DRB.TotalPdcpPacketDl.SNSSAI where SNSSAI identifies the S-NSSAI.

f)	GNBCUUPFunction.

NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality). NRCellCU measurement applies only for 2-split deployment.

#### The second change

# A.2	Monitoring of UL and DL packet loss in NG-RAN

Keeping track of UL and DL packet loss in the NG-RAN is essential, since for certain services packets that are lost along the way through the system may have a noticeable impact on the end user. UL and DL packet loss measurements (associated with packets measurements at the same time) can be useful for evaluation, optimization and for performance assurance within the integrity area (user plane connection quality). Subcounters per QoS Level as well as per supported S-NSSAI is helpful for operator to pinpoint the reason for high packet loss rate.

UL packet loss is a measure of packets dropped in the UE and the packets lost on the interfaces (air interface and F1-U interface). DL packet loss is a measure of packets lost on the interfaces (air interface and F1-U interface). If parts of the gNB are deployed in a virtualized environment, it is important to measure the packet loss on air interface and F1-U interface in a separate measurements, to be able to pinpoint the reason for high packet loss.

#### The third change

# A.3	Monitoring of DL packet drop in NG-RAN

Keeping track of DL packet drops in the NG-RAN is essential, since for certain services packets that are dropped along the way through the system may have a noticeable impact on the end user. DL packet drop measurements (associated with packets measurements at the same time) can be useful for evaluation, optimization and for performance assurance of the network. Subcounters per QoS Level as well as per supported S-NSSAI is helpful for operator to pinpoint the reason for high packet drop rate.

For gNBs that are deployed in a split architecture, e.g. when parts of a gNB are deployed in a virtualized environment, the DL packet drops may occur in two parts; the gNB CU-UP and the gNB DU. Therefore, it is important to measure this separately.

#### End of changes