3GPP TSG-SA5 Meeting #149 	S5-234407

Berlin, Germany, 22 - 26 May 2023

# Source:	MATRIXX Software

# Title:	pCR 28.203 Introduce Nb of PDUs IEC message flow

# Document for:	Approval

Agenda Item:	7.4.1

# 1	Decision/action requested

This pCR is to Introduce Nb of PDUs IEC message flow

# 2	References

[1]		3GPP TR 32.847 "Study on Charging Aspects for Network Slicing Phase 2"

[2]	3GPP TS 28.203 "Charging management; Network Slice Admission Control charging in the 5G System (5GS); Stage 2".

# 3	Rationale

This pCR is to Introduce Nb of PDUs IEC message flow.

# 4	Detailed proposal

The following changes are proposed to be incorporated into TS 28.203 [2]:

First change

##### 5.2.2.x.3	Number of PDU sessions per network slice – IEC

The following figure 5.2.2.x.3-1 describes a Number of PDU sessions per network slice charging message flow in IEC, based on figure  4.2.11.4-1 of  3GPP TS 23.502 [4] description:

Figure 5.2.2.x.3-1: Number of PDU sessions per S-NSSAI – IEC

Steps 1 to 3: per 3GPP TS 23.502 [4] Figure 4.2.11.4-1 Number of PDU sessions per network slice availability check and update procedure.

3ch-a: For a particular S-NSSAI, e.g. the "Nb of PDU sessions threshold crossed upwards" trigger is met.

3ch-b: The NSACF sends Charging Data Request [Event] to CHF with "Number of PDU sessions ".

3ch-c: The CHF creates a CDR for this " Number of PDU sessions" for the S-NSSAI.

3ch-d: CHF provides response to NSACF.

Step 4: per 3GPP TS 23.502 [4] Figure 4.2.11.4-1 Number of PDU sessions per network slice availability check and update procedure.

End of change