3GPP TSG-SA5 Meeting #149 	S5-234408

Berlin, Germany, 22 - 26 May 2023

# Source:	MATRIXX Software

# Title:	pCR 28.203 Introduce Nb of PDU sessions ECUR message flow

# Document for:	Approval

Agenda Item:	7.4.1

# 1	Decision/action requested

This pCR is to Introduce Nb of PDU sessions ECUR message flow

# 2	References

[1]		3GPP TR 32.847 "Study on Charging Aspects for Network Slicing Phase 2"

[2]	3GPP TS 28.203 "Charging management; Network Slice Admission Control charging in the 5G System (5GS); Stage 2".

# 3	Rationale

This pCR is to Introduce Nb of PDU sessions ECUR message flow.

# 4	Detailed proposal

The following changes are proposed to be incorporated into TS 28.203 [2]:

First change

##### 5.2.2.x.4	Number of PDU sessions per network slice – ECUR

The following figure 5.2.2.x.4-1 describes a Number of PDU sessions per network slice charging message flow in ECUR, based on figure  4.2.11.4-1 of  3GPP TS 23.502 [4] description:

Figure 5.2.2.x.4-1: Number of PDU sessions per S-NSSAI – ECUR

A.Steps 1 to 3: per 3GPP TS 23.502 [4] Figure 4.2.11.4-1 Number of PDU sessions per network slice availability check and update procedure.

3ch-a: For a particular S-NSSAI, e.g. the "Nb of PDUs threshold crossed upwards" trigger is met.

3ch-b: The NSACF sends Charging Data Request [Initial] to CHF with "Number of PDU sessions" and "Quota Requested",  to be granted authorization, and to reserve a "Number of PDU sessions Quota limit".

3ch-c: Account, Rating, reservation control  by the CHF. The CHF opens a CDR

3ch-d: CHF provides response to NSACF.

A.Steps 4: per 3GPP TS 23.502 [4] Figure 4.2.11.4-1

B.Steps 1 to 4: per 3GPP TS 23.502 [4] Figure 4.2.11.4-1: occuring multiple times without any interaction with CHF.

C.Steps 1 to 3: per 3GPP TS 23.502 [4] Figure 4.2.11.4-1.

C.3ch-a: the "Number of PDU sessions Quota limit" is reached.

C.Step 4: per 3GPP TS 23.502 [4] Figure 4.2.11.4-1: NSACF indicates to SMF the "Maximum Nb of PDU sessions" is reached for SMF to reject the PDU session.

4ch-a: The NSACF sends Charging Data Request [Termination] to CHF with "Number of PDU sessions".

4ch-b: Account, Rating Control by the CHF. The CHF closes the CDR

4ch-c: CHF provides response to NSACF.

Next change

Next change

End of change