3GPP TSG-SA5 Meeting #149	S5-234419

# Berlin, Germany, 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.405 | CR | 0023 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR for TS28.405 several editorial Corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-05-1 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | several editorial Corrections |  |  |  |  |  |  |  |  |
| Summary of change: |  | several editorial Corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | several editorial Corrections |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2.2，4.2.2.2，Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### Start of changes

#### 4.1.2.2	Handover between RNCs

The figure 4.1.2.2-1 and the text below describes the handling at handover between RNCs.

Figure 4.1.2.2-1: Handling of QMC activation in case of handover in UTRAN

1.	Source RNC sends the message HandoverRequest to Target RNC and includes areaScope, qoECollectionEntityAddress and a recording session indication indicating whether a recording session has started.

2.	If a recording session has not started, the target RNC checks if the target cell is inside the areaScope or not.

3.	The target RNC sends the message Handover Request Ack to the source RNC.

4.	The source RNC sends the message RRCReconfiguration to the UE.

5.	The access stratum in the UE sends the AT command +CAPPLEVMC to application level.

6.	The UE sends the message RRCReconfigurationComplete to the RNC.

7.	When the application becomes active, QMC is initiated if the UE is inside the areaScope.

8.	The application layer sends the AT command +CAPPLEVMR including a recording session indication to the access stratum.

9.	The UE sends the message MeasurementReport including the recording session indication to the RNC.

10.	The RNC sends a notification including the recording session indication to the NM.

11.	When the QMC is completed, the recorded information is collected in a QMC report [6] and [7] including qoEReference and recordingSessionId. The qoEReference, Client Id [6] and [7] in the reporting container (that represent the UE request session) and recordingSessionId are needed in the QoE collection entity for post processing purposes.

12.	The application layer sends the AT command +CAPPLEVMR including qoEReference and the QMC report to the access stratum.

13.	The UE sends the message MeasurementReport including qoEReference and the QMC report to the RNC.

14.	The RNC sends the QMC report to the MCE associated to the qoEReference.

#### The Second Change

#### 4.2.2.2	Handover between eNBs

The figure 4.2.2.2-1 and the text below describes the handling at handover between eNBs.

Figure 4.2.2.2-1: Handling of QMC activation in case of handover in LTE

1.	Source eNB sends the message HandoverRequest to Target eNB and includes areaScope and qoECollectionEntityAddress.

2.	The target eNB checks if the target cell is inside the areaScope or not.

3.	The target eNB sends the message HandoverRequestAcknowledge to the source eNB.

4.	The source eNB sends the message RRCConnectionReconfiguration to the UE.

5.	The access stratum in the UE sends the AT command +CAPPLEVMC to application level.

6.	The UE sends the message RRCConnectionReconfigurationComplete to the eNB.

7.	When the application becomes active, QMC is initiated if the UE is inside the areaScope.

8.	The application layer sends the AT command +CAPPLEVMR including a recording session indication indicating that a session has started to the access stratum.

9.	The UE sends the message MeasReportAppLayer including the recording session indication to the eNB.

10.	The eNB sends a notification including the recording session indication to the NM.

11.	When the QMC is completed, the recorded information is collected in a QMC report [6] and [7] including qoEReference and recordingSessionId. The qoEReference, Client Id [6] and [7] in the reporting container (that represent the UE request session) and recordingSessionId are needed in the QoE collection entity for post processing purposes.

12.	The application layer sends the AT command +CAPPLEVMR including qoEReference and the QMC report to the access stratum.

13.	The UE sends the message MeasReportAppLayer including qoEReference and the QMC report to the eNB.

14.	The eNB sends the QMC report to the MCE associated to the qoEReference.

#### The Third Change

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-07 | SA#88e |  |  |  |  | Upgrade to version under change control | 16.0.0 |
| 2021-09 | SA#93e | SP-210883 | 0001 | - | F | Aligning with RAN specifications 36.331 and 25.331 | 16.1.0 |
| 2021-12 | SA#94e | SP-211454 | 0004 | 1 | F | Correct the description of QoE reference and PLMN target | 16.2.0 |
| 2021-12 | SA#94e | SP-211453 | 0003 | 1 | B | Adding Signalling Based Activation for UTRAN and LTE | 17.0.0 |
| 2022-03 | SA#95e | SP-220181 | 0005 | - | B | Adding Management Based Activation and Temporary stop and restart during RAN overload in NR | 17.1.0 |
| 2022-03 | SA#95e | SP-220181 | 0008 |  | B | Update to include NR and adding VR | 17.1.0 |
| 2022-06 | SA#96 | SP-220500 | 0011 | 1 | F | Adding missing parameters and updating definition | 17.2.0 |
| 2022-09 | SA#97e | SP-220850 | 0012 | - | F | Align Requirements of handling QMC during RAN overload to RAN specifications | 17.3.0 |
| 2022-09 | SA#97e | SP-220855 | 0014 | 1 | B | Adding Signalling Based Activation for NR | 18.0.0 |
| 2022-09 | SA#97e | SP-220855 | 0015 | 1 | B | Handling of QoE measurement collection at handover in NR | 18.0.0 |
| 2022-12 | SA#98e | SP-221167 | 0017 | 2 | F | Clean up | 18.1.0 |
| 2022-12 | SA#98e | SP-221176 | 0018 | 1 | F | Management based activation in NR correction for SBMA | 18.1.0 |
| 2022-12 | SA#98e | SP-221176 | 0019 | - | B | Add MDT alignment Information and RAN visible QoE Metrics to Handling QMC at Handover | 18.1.0 |
| 2022-12 | SA#98e | SP-221176 | 0021 | - | B | Definition of parameters MDT Alignment Information and Available RAN Visible QoE Metrics | 18.1.0 |
| 2023-03 | SA#99 | SP-230204 | 0022 | - | B | Add MDT Alignment Information and RAN visible QoE Metrics to Signalling Based Activation | 18.2.0 |

#### End of changes