3GPP TSG-SA5 Meeting #149 S5-234425 # Berlin, Germany, 22 – 26 May, 2023 CR-Form-v12.2. CHANGE REQUEST. 28.552; CR; 0438; rev; -; Current version:; 18.2.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests..

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New measurement “In-session activity time per RAT” |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In-session activity time for DRB for 5GS has been already defined in TS 28.552 clause 5.1.1.10.4. Similarly, In-session activity time for UE for EPS has been already defined in TS 32.425 clause 4.2.4.1. In this CR, it is proposed to define an aggregated active session time for UEs per RAT at the combined (EPS+5GS) NE, as this performance measure is used to define 5G and EUTRAN in-session retention rate KPI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new performance measure capturing number of “in session” seconds aggregated on all E-RABs as well as number of “in session” seconds aggregated on all QoS flows. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without this performance measure, NR and EUTRAN in-session retention rate KPI (TS 28.554 CR 0123) cannot be derived. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.13.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 28.554 CR 0123 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

##### 5.1.1.13.2.X	In-session activity time per RAT

a)	This measurement provides the aggregated active session time for UEs per RAT.

b)	CC.

c)	Number of "in session" seconds aggregated on all E-RABs as well as number of "in session" seconds aggregated on all QoS flows. 
For E-RABs/QoS flow with bursty flow, a UE is said to be "in session" if any data on a Data Radio Bearer (UL or DL) has been transferred during the last 100 ms. 
For E-RABs/QoS flow with continuous flow, the E-RABs/QoS flow (and the UE) is always seen as being "in session" in the context of this measurement as long as the UE is in RRC connected state, and the session time is increased from the first data transmission on the E-RABs/QoS flow until 100ms after the last data transmission on the ERABs/QoS flow (see clause 5.1.1.10.4 and clause 4.2.4.1 in TS 32.425 [9])

d)	Each measurement is an integer value.

e)	Combi.RB.SessionTimeUE			Combined

Combi.RB.SessionTimeUE.E		EPS

Combi.RB.SessionTimeUE.5G		5GS

f)	Concatenation of EUtranCell (EUtranCellFDD and EUtranCellTDD) and NRCellCU.

g)	Valid for packet switched traffic.

h)	Combined NE of EPS/5GS.

i)	This measurement is to support the in-session retention KPI.

# >>>>END OF CHANGES<<<<