| 3GPP TS 28.204 V0.2.0 (2023-05) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Charging management; Network slice-specific authentication and authorization charging in the 5G System (5GS); Stage 2 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

Annex <A> (informative): Change history	11

# Foreword

This Technical Specification| has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the Converged Charging description for network slice-specific authentication and authorization charging in the 5G System (5GS) based on Network Slice-Specific Authentication and Authorization Function (NSSAAF) of 5GS architecture and procedures specified in 3GPP TS 23.501 [3] and 3GPP TS 23.502 [4].

The scope is the Network Slice-Specific Authentication and Authorization as specified in TS 23.502 [4] with a AAA Server (AAA-S).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[3]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 32.256: "Telecommunication management; Charging management; 5G connection and mobility domain charging; stage 2".

[6]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5G System: 3GPP system consisting of 5G Access Network (AN), 5G Core Network and UE.

Network Slice: A logical network that provides specific network capabilities and network characteristics.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Bnssaa	Reference point for the CDR file transfer from the NSSAAF CGF to the BD.

Ga	Reference point for CDR transfer between a CDF and the CGF.

Nchf	Service based interface exhibited by CHF.

N10y	Reference point between NSSAAF and the CHF.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

3GPP	3rd Generation Partnership Project

5GS	5G System

AAA 	Authentication Authorization Accounting

AAA-S	AAA Server

NSSAA	Network Slice-Specific Authentication and Authorization

PDU	Protocol Data Unit

S-NSSAI	Single Network Slice Selection Assistance Information

UE	User Equipment

# 4	Architecture considerations

## 4.1	High-level 5G System architecture

### 4.1.1	Non-roaming reference architecture

Figure 4.1.1.1 shows the Non-roaming 5G System high level architecture in the service-based representation, as defined in 3GPP TS 23.501 [3], with Network Slice-Specific Authentication and Authorization (NSSAAF):

Figure 4.1.1.1: Non-Roaming 5G System architecture

## 4.2	Network Slice-Specific Authentication and Authorization converged charging architecture

### 4.2.1	Non-roaming

Architectural options for Network Slice-Specific Authentication and Authorization converged charging in service-based representation are depicted in figure 4.2.1.1.

Figure 4.2.1.1: Non-Roaming Network Slice-Specific Authentication and Authorization converged charging architecture

Architectural options of figure 4.2.1.1 apply to any Network Slice-Specific Authentication and Authorization converged charging architecture of this clause.

AMF is part of this architecture for the purpose of Network Slice-Specific Authentication and Authorization converged charging. For AMF 5G connection and mobility converged charging see TS 32.256 [5].

Details on the interfaces and functions can be found in 3GPP TS 32.240 [2] for the general architecture components, Ga is described in clause W and Bnssaa in clause Z of this document, and Nchf is described in 3GPP TS 32.290 [6].

Figure 4.2.1.2 shows the Network Slice-Specific Authentication and Authorization converged charging architecture in reference point representation for non-roaming:

Figure 4.2.1.2: Non-Roaming Network Slice-Specific Authentication and Authorization converged charging architecture - reference point representation

# 5	Network Slice-Specific Authentication and Authorization charging principles and scenarios

## 5.1	Network Slice-Specific Authentication and Authorization charging principles

### 5.1.1	General

The charging functions specified for Network Slice-Specific Authentication and Authorization charging, are based on Network Slice-Specific Authentication and Authorization with a AAA Server (AAA-S) functionality as specified in TS 23.501 [3], supported by:

-	NSSAAF,	and

-	optionaly AMF.

The network slice is identified by a S-NSSAI.

For AMF the present specification only covers Network Slice-Specific Authentication and Authorization charging. For AMF 5G connection and mobility charging see TS 32.256 [5].

### 5.1.2	Requirements

The following are high-level charging requirements specific to Network Slice-Specific Authentication and Authorization charging:

-	The NSSAAF shall support converged charging using service based interface.

-	The NSSAAF shall support converged charging for Network Slice-Specific Authentication and Authorization procedure per S-NSSAI per UE.

-	The AMF shall support converged charging for Network Slice-Specific Authentication and Authorization procedure per S-NSSAI per UE.

### 5.1.3	Charging information

The charging information for Network Slice-Specific Authentication and Authorization charging are:

-	GPSI;

-	SUPI;

-	S-NSSAI;

- 	AAA-S address.

Annex <A> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-04 | SA5#148e |  |  |  |  | Initial skeleton | 0.0.0 |
| 2023-04 | SA5#148e | S5-233220 S5-233222 S5-233223 S5-233656 |  |  |  | Introduction of the Reference Introduction of the Terms Introduction of the Abbreviations Introduction of the Scope | 0.1.0 |
| 2023-05 | SA5#149 | S5-234473 S5-234474 |  |  |  | Introduce architecture Introduce charging principles | 0.2.0 |