3GPP TSG-SA5 Meeting #149 	S5-234472

Berlin, Germany, 22 - 26 May 2023

# Source:	MATRIXX Software, Ericsson

# Title:	pCR 28.203 Introduce message flows SCUR

# Document for:	Approval

Agenda Item:	7.4.1

# 1	Decision/action requested

This pCR is to introduce message flows SCUR

# 2	References

[1]		3GPP TR 32.847 "Study on Charging Aspects for Network Slicing Phase 2"

[2]	3GPP TS 28.203 "Charging management; Network Slice Admission Control charging in the 5G System (5GS); Stage 2".

# 3	Rationale

This pCR is to introduce message flows SCUR.

# 4	Detailed proposal

The following changes are proposed to be incorporated into TS 28.203 [2]:

First change

#### 5.2.2.y	Number of UEs and Number of PDU sessions per network slice charging from NSACF – single charging session

##### 5.2.2.y.1	General

The subclause below describes the Network Slice Admission Control charging for Number of UEs and Number of PDU sessions per network slice in SCUR with a single charging session for the S-NSSAI, based on figure 4.2.11.2-1 and figure 4.2.11.4-1 of 3GPP TS 23.502 [4] description..

##### 5.2.2.y.2	Number of UEs and Number of PDU sessions per network slice charging – initial

The following figure 5.2.2.y.2-1 describes Network Slice Admission Control charging for Number of UEs and Number of PDU sessions per network slice charging session initiation message flows in SCUR:

Figure 5.2.2.x.2-1: Number of UEs and Number of PDU sessions per S-NSSAI – SCUR - initial

Och. No charging session exists for the S-NSSAI.

A.Steps 1 to 3: per 3GPP TS 23.502 [4] Figure 4.2.11.2-1 Number of UEs per network slice availability check and update procedure.

B.Steps 1 to 3: per 3GPP TS 23.502 [4] Figure 4.2.11.4-1 Number of PDU sessions per network slice availability check and update procedure.

A and B occurs in parallel and asynchroneously from each other.

3ch-a: For a particular S-NSSAI, one of the Nb of UEs threshold or Nb of PDU sessions trigger reached for initial.

3ch-b: The NSACF sends Charging Data Request [Initial] to CHF and, depending on the trigger:

if "Nb of UEs threshold reached for initial", sends the "Number of UEs" and may request for a "Number of UEs Quota limit",

if "Nb of PDU sessions threshold reached for initial", sends the "Number of of PDU sessions" and may request for a "Number of of PDU sessions Quota limit".

3ch-c: Account, Rating Control by the CHF. The CHF opens a CDR.

3ch-d: CHF provides response to NSACF and may include quota granted and new triggers setting.

A.Step 4: per 3GPP TS 23.502 [4] Figure 4.2.11.2-1

B.Step 4: per 3GPP TS 23.502 [4] Figure 4.2.11.4-1

##### 5.2.2.y.3	Number of UEs and Number of PDU sessions per network slice charging – update

The following figure 5.2.2.y.3-1 describes Network Slice Admission Control charging for Number of UEs and Number of PDU sessions per network slice charging session update message flows in SCUR:

Figure 5.2.2.y.3-1: Number of UEs and Number of PDU sessions per S-NSSAI – SCUR - update

Och. A charging session is established for the S-NSSAI.

A.Steps 1 to 3: per 3GPP TS 23.502 [4] Figure 4.2.11.2-1 Number of UEs per network slice availability check and update procedure.

B.Steps 1 to 3: per 3GPP TS 23.502 [4] Figure 4.2.11.4-1 Number of PDU sessions per network slice availability check and update procedure.

A and B occurs in parallel and asynchroneously from each other.

3ch-a: For a particular S-NSSAI, one of the trigger related to "Nb of UEs or Nb of PDU sessions threshold crossed upwards/downwards" or granted quota is met.

3ch-b: The NSACF sends Charging Data Request [update] to CHF and, depending on the trigger:

if  "Nb of UEs" trigger, sends the "Number of UEs" and may request for a "Number of UEs Quota limit",

if "Nb of PDU sessions" trigger, sends the "Number of of PDU sessions" and may request for a "Number of of PDU sessions Quota limit".

3ch-c: Account, Rating Control by the CHF. The CHF updates the CDR.

3ch-d: CHF provides response to NSACF and may include quota granted and new triggers setting.

A.Step 4: per 3GPP TS 23.502 [4] Figure 4.2.11.2-1

B.Step 4: per 3GPP TS 23.502 [4] Figure 4.2.11.4-1

##### 5.2.2.y.4	Number of UEs and Number of PDU sessions per network slice charging – termination

The following figure 5.2.2.y.4-1 describes Network Slice Admission Control charging for Number of UEs and Number of PDU sessions per network slice charging session terminatio message flows in SCUR:

Figure 5.2.2.y.4-1: Number of UEs and Number of PDU sessions per S-NSSAI – SCUR - termination

Och. A charging session is established for the S-NSSAI.

A.Steps 1 to 3: per 3GPP TS 23.502 [4] Figure 4.2.11.2-1 Number of UEs per network slice availability check and update procedure.

B.Steps 1 to 3: per 3GPP TS 23.502 [4] Figure 4.2.11.4-1 Number of PDU sessions per network slice availability check and update procedure.

A and B occurs in parallel and asynchroneously from each other.

3ch-a: For a particular S-NSSAI, the "Nb of UEs and Nb of PDU sessions threshold reached for termination.

3ch-b: The NSACF sends Charging Data Request [Termination] to CHF and, depending on the trigger:

if  "Nb of UEs" trigger, sends the "Number of UEs".

if "Nb of PDU sessions" trigger, sends the "Number of of PDU sessions".

3ch-c: Account, Rating Control by the CHF. The CHF closes the CDR.

3ch-d: CHF provides response to NSACF

A.Step 4: per 3GPP TS 23.502 [4] Figure 4.2.11.2-1

B.Step 4: per 3GPP TS 23.502 [4] Figure 4.2.11.4-1

End of change