3GPP TSG-SA5 Meeting #149 	S5-234473

Berlin, Germany, 22 - 26 May 2023

# Source:	MATRIXX Software

# Title:	pCR 28.204 Introduce architecture

# Document for:	Approval

Agenda Item:	7.4.2

# 1	Decision/action requested

This pCR is to introduce architecture.

# 2	References

[1]		3GPP TR 32.847 "Study on Charging Aspects for Network Slicing Phase 2"

[2]	3GPP TS 28.204 "Network slice-specific authentication and authorization charging in the 5G System (5GS)".

# 3	Rationale

This pCR is to introduce architecture.

# 4	Detailed proposal

The following changes are proposed to be incorporated into TS 28.204 [2]:

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[3]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[x]	3GPP TS 32.256: "Telecommunication management; Charging management; 5G connection and mobility domain charging; stage 2".

[y]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

Next change

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Bnssaa	Reference point for the CDR file transfer from the NSSAAF CGF to the BD.

Ga	Reference point for CDR transfer between a CDF and the CGF.

Nchf	Service based interface exhibited by CHF.

N10y	Reference point between NSSAAF and the CHF.

Next change

# 4	Architecture considerations

## 4.1	High-level 5G System architecture

### 4.1.1	Non-roaming reference architecture

Figure 4.1.1.1 shows the Non-roaming 5G System high level architecture in the service-based representation, as defined in 3GPP TS 23.501 [3], with Network Slice-Specific Authentication and Authorization (NSSAAF):

Figure 4.1.1.1: Non-Roaming 5G System architecture

Next change

## 4.2	Network Slice-Specific Authentication and Authorization converged charging architecture

### 4.2.1	Non-roaming

Architectural options for Network Slice-Specific Authentication and Authorization converged charging in service-based representation are depicted in figure 4.2.1.1.

Figure 4.2.1.1: Non-Roaming Network Slice-Specific Authentication and Authorization converged charging architecture

Architectural options of figure 4.2.1.1 apply to any Network Slice-Specific Authentication and Authorization converged charging architecture of this clause.

AMF is part of this architecture for the purpose of Network Slice-Specific Authentication and Authorization converged charging. For AMF 5G connection and mobility converged charging see TS 32.256 [x].

Details on the interfaces and functions can be found in 3GPP TS 32.240 [2] for the general architecture components, Ga is described in clause W and Bnssaa in clause Z of this document, and Nchf is described in 3GPP TS 32.290 [y].

Figure 4.2.1.2 shows the Network Slice-Specific Authentication and Authorization converged charging architecture in reference point representation for non-roaming:

Figure 4.2.1.2: Non-Roaming Network Slice-Specific Authentication and Authorization converged charging architecture - reference point representation

End of change