3GPP TSG-SA5 Meeting #149 	S5-234480

Berlin, Germany, 22 - 26 May 2023

# Source:	Ericsson LM

# Title:	Rel-18 pCR 28.826 Used unit containers for one rating group split over messages

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

Include the proposed changes in TR 28.826.

# 2	References

[1]		3GPP TR 28.826: " Study on Nchf charging services phase 2 improvements and optimizations"

# 3	Rationale

New use case for used unit containers for one rating group split over messages.

# 4	Detailed proposal

First change

#### 5.1.2.2	Use Case #1b: Size of charging information

An operator has a rating group containing only services charged online and only set immediate for triggers relating to quota management for volume, all other triggers for the rating group are set to deferred. This may in some cases result in a case where a request will be required to be split over several charging data requests e.g., when the payload size of HTTP requests exceeds the recommended values.

The operator may alternatively in this case want to optimize their storage and would therefore like to control the size of the converged charging requests based on the available space in each CHF. It should be noted that there is a trade of between size of the requests and the number of requests.

There are two parts of to the storage in the CHF one is the CDR storage before sending it to the CGF and the allocation of the resource for the SBI. The resource may be stored uncompressed for fast access, whereas the charging information in the converged charging request may be compressed to limit network utilization. This means that the operator would like to control the size of the message both before and after the compression.

When a converged charging request is sent the amount of information included in the intimal is often limited since not much usage can have been done, so it is more important to be able to control the sizes of the update and termination than the initial.

The converged charging request can contain request for units, offline unit reports and online units report for the same rating group.

End of changes