3GPP TSG-SA5 Meeting #149 	S5-234509

Berlin, Germany, 22 - 26 May 2023

# Source:	Huawei

# Title:	pCR TR 28.925 Update solution and add conclusion and recommendation for issue#12 illustration of using MnS in management reference model in TS 32.101

# Document for:	Approval

Agenda Item:	6.8.1.1

# 1	Decision/action requested

Discuss and approve on the proposal.

# 2	References

[1]	3GPP TR 28.925 enhancement of service based management architecture v0.a.0

[2]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

# 3	Rationale

SA5 adopted SBMA from Rel-15, in SBMA concept, the standardized interface is represented as communication between MnS consumer and MnS producer. In reality, the standardized interfaces related to management features may be used between multiple players.

In TS 32.101, the standardized interfaces are described between the following entities:

A number of management interfaces in a PLMN are identified in figure 1, namely:

1)	between the Network Elements (NEs) and the Element Manager (EM) of a single PLMN Organisation;

2)	between the Element Manager (EM) and the Network Manager (NM) of a single PLMN Organisation;

NOTE:	In certain case the Element Manager functionality may reside in the NE in which case this interface is directly from NE to Network Manager). These management interfaces are given the reference name Itf-N and are the primary target for standardization.

3)	between the Network Managers and the Enterprise Systems of a single PLMN Organisation;

4)	between the Network Managers (NMs) of a single PLMN Organisation;

4a) between the Domain Managers (DMs) of a single PLMN Organisation.

5)	between Enterprise Systems & Network Managers of different PLMN Organisations;

5a) between the Domain Managers (DMs) of different PLMN Organisations.

6)	between Network Elements (NEs).

7)	between the Network Management Layer Service (NMLS) and the Network Manager (NM).

So far SA5 provided a list of MnS, it would be beneficial to provide the profiling information for better describe the usage of different set of MnS in the real deployment network.

The current solution for Issue#12 in TR 28.925 [1] is plain worded and may bring difficulties to readers to fully understand how the MnS is used to improve the interoperability between different entities it is proposed to add corresponding clarifications.

# 4	Detailed proposal

This document proposes the following changes in TR 28.925 [1].

1st Change

## 4.12 	Issue#12: illustration of using MnS in management reference model in TS 32.101

### 4.12.1	Description

In TS 32.101[3], Clause 5.1 illustrates the management reference model which shows the Operations Systems interfacing with other systems. An Operations System supports management interfaces to other systems.

Figure 1: Management reference model

A number of management interfaces in a PLMN are identified in figure 1, namely:

1)	between the Network Elements (NEs) and the Element Manager (EM) of a single PLMN Organisation;

2)	between the Element Manager (EM) and the Network Manager (NM) of a single PLMN Organisation;

NOTE:	In certain cases, the Element Manager functionality may reside in the NE in which case this interface is directly from NE to Network Manager). These management interfaces are given the reference name Itf-N and are the primary target for standardization.

3)	between the Network Managers and the Enterprise Systems of a single PLMN Organisation;

4)	between the Network Managers (NMs) of a single PLMN Organisation;

4a) between the Domain Managers (DMs) of a single PLMN Organisation.

5)	between Enterprise Systems & Network Managers of different PLMN Organisations;

5a) between the Domain Managers (DMs) of different PLMN Organisations.

6)	between Network Elements (NEs).

7)	between the Network Management Layer Service (NMLS) and the Network Manager (NM).

IRPs may be implemented at interfaces 2, 3, 4, 5 and 7.

TS 28.533[2] introduces the Service Based Management Architecture (SBMA). The fundamental building block of the Service Based Management Architecture (SBMA) is the Management Service (MnS). A MnS is a set of offered capabilities for management and orchestration of network and services. An MnS producer offers its services via a standardized service interface composed of individually specified MnS components (MnS component type A, B, C).

Analysis:

- In TS 32.101, there is clearly showing the entities and the corresponding interfaces in the management reference model.

- In TS 28.533, the interaction of paradigm of MnS producer and MnS Consumer is defined without indicating the entities.

- The entities in TS 32.101 can be illustrated using the MnS consumer and MnS producer according to the way of using interface IRPs.

### 4.12.2	Potential solutions

There are the following aspects to be considered in the solution of using MnS.

- The management services may be used to support the interoperability in different management scenarios

  - Interoperability within Operation system of a single PLMN Organisation

  - Interoperability between BSS and Operation systems within a single PLMN Organization

  - Interoperability between BSS of different PLMN Organisations.

  - Interoperability between Operation Systems of different PLMN Organisations.

- Different set of management services may be used for the interoperability between different entities.

Figure 4.12.2-1 illustrates an example of using management services for supporting interoperability in a multiple players scenario:

Figure 4.12.2-1: Example of possbile usage of MnS

Next Change

# 5	Conclusion and Recommendation

## 5.12	Issue #12: illustration of using MnS in management reference model in TS 32.101

A MnS is a set of offered capabilities for management and orchestration of network and services, SBMA provides the following flexibility:

- SBMA does not limit who can play the role of an MnS consumer. Any entities consume SA5 defined MnS are called MnF.

- Depending on different scenarios, an entity either plays the role as MnS producers or as MnS consumers .

It is recommended to add examples of using MnS in different scenarios to illustrate the flexibility of Service Based Management Architecture (SBMA).

End of change