3GPP TSG-SA5 Meeting #149 	S5-234587

Berlin, Germany, 22 - 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0173 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Procedure of Network Slice Instance Allocation to support asynchronous operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei, Telefonica, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNETSLICE_PRO |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | S5-233155 was endorsed with the following: - Introduce a solution using NetworkSliceController IOC to monitor progress and result for network slice (subnet) allocation and deallocation as an enhancement to support async mode in 3GPP Rel-18. - The solution introduced above will support both network slice lifecycle management using allocate/deallocate operation and CRUD based operation. Network Slice and Network Slice Subnet LCM operations are long running and hence need to be asynchronous. The changes in this CR introduces support for asynchronous mode of operations for Procedure of Network Slice Instance Allocation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Procedure of Network Slice Instance Allocation to support asynchronous or long running operations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support of shared objects as well as asynchronous or long running operations for Network Slice Instance Allocation is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS28.541 CR 0800 TS28.623 CR 0214 |  |  |
| Other comments: |  | Proposal from S5-234078 has been merged into this tDoc. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - S5-233898 is revised to S5-234587 |  |  |  |  |  |  |  |  |

1st Change

## 7.2	Procedure of Network Slice Instance Allocation

The Figure 7.2-1 illustrates the procedure of creating a new NSI or using an existing NSI to satisfy the required network slice related requirements.

Figure 7.2-1: Network Slice Instance Allocation Request procedure

1a-1)	Network Slice Management Service Provider (NSMS_P) receives an AllocateNsi request (see AllocateNsi operation defined in clause 6.5.1) from Network Slice Management Service Consumer (NSMS_C) with network slice related requirements (the network slice related requirements are defined as the attributes in the ServiceProfile see clause 6.3.3 in TS 28.541 [6]).

1a-2)	The NSMS_P creates an NetworkSliceController MOI and configures attribute serviceProfileId to control and monitor network slice allocation process. NSMS_P starts to perform the network slice allocation process.

1a-3)	The NSMS_P sends AllocateNsi response to the NSMS_C. The serviceProfileId is included in the AllocateNsi response.

1a-4) NSMS_P sends notification (see notfyMOICreation defined in TS 28.532 [8]) to NSMS_C to notify the DN of the created NetworkSliceController instance.

1b-1)	NSMS_P receives a createMOI request (see createMOI operation defined in TS 28.532 [8]) from NSMS_C with network slice controller (see NetworkSliceController IOC and its attribute defined in TS 28.541[6]) including the network slice related requirements in attribute inputServiceProfile.

1b-2)	The NSMS_P creates an MOI of NetworkSliceController.

1b-3)	The NSMS_P sends the response to NSMS_C for the createMOI request.

NOTE: Network slice allocation may be initiated using AllocateNsi operation as described in steps 1a or createMOI operation as described in 1b.

2)	Based on the network slice related requirements and the knowledge of the capabilities of existing deployed network slices, the NSMS_P may compare/match the provided requirements against all the candidate NetworkSlice MOIs, and then decides whether to use an existing NetworkSlice MOI or create a new NetworkSlice MOI. If the network slice related requirements allow the requested NetworkSlice MOI to be shared and if an existing suitable NetworkSlice MOI can be reused, the NSMS_P may decide to use the existing NetworkSlice MOI.

3a-1)	If using an existing NetworkSlice MOI, the NSMS_ P identifies the NetworkSlice MOI to be re-used.

3a-2)	The NSMS_P configures NetworkSlice MOI with the DN of NetworkSliceController MOI.

3a-3)	NSMS_P configures the NetworkSliceController MOI attribute networkSliceRef with DN of NetworkSlice MOI (identified in step 3a-1).

3a-4) If the existing NetworkSlice MOI needs to be modified to satisfy the network slice related requirements, the NSMS_P invokes the procedure of network slice instance modification as described in clause 7.6.

3b-1) If creating a new NetworkSlice MOI, the NSMS_P derives the network slice subnet related requirements from the received network slice related requirements. Before NSMS_P derives the network slice subnet related requirements, NSMS_P may invoke corresponding network slice subnet capability information querying procedure as described in clause 7.8.

3b-2)	The NSMS_P invokes the procedure of network slice subnet instance allocation as described in clause 7.3. Before NSMS_P invokes the procedure, NSMS_P may invoke corresponding procedure of feasibility check and reservation of network slice subnet as described in clause 7.14.

3b-3)	The NSMS_P creates the NetworkSlice MOI and configures the DN of NetworkSliceSubnet MOI and DN of NetworkSliceController MOI, other configuration information may be configured for the created NetworkSlice MOI.

Note:	The detailed configuration information is described in network slice NRM (see NetworkSlice IOC defined in clause 6.3.1 in TS 28.541 [6]).

3a-5)	NSMS_P configures the NetworkSliceController MOI attribute networkSliceRef with DN of NetworkSlice MOI (created in step 5b-1).

4)	NSMS_P configures NetworkSliceController MOI attributes operationalState, administrativeState, availabilityStatus and processMonitor to indicate completion of the procedure.

The NSMS_C may check the status and completion of the network slice allocation procedure any time by monitoring the values of NetworkSliceController MOI attributes operationalState, administrativeState, availabilityStatus and processMonitor by querying the values or by subscribing to notifications.

Following step 5) describes the procedures for NSMS_C to monitor network slice allocation progress and result. These steps can happen anytime after the NetworkSliceController MOI is created and its DN is notified to the NSMS_C (step 1a-1 or step 1b-3).

5a) The NSMS_P sends notification (see notifyMOIAttributeValueChanges defined in TS 28.532 [8]) to NSMS_C to notify the progress and result for network slice allocation process (see attributes in NetworkSliceController IOC).

5b) The NSMS_P sends query request to NSMS_P to query the attribute values of NetworkSliceController MOI (see getMOIAttributes operation defined in TS 28.532 [8]) to obtain the progress and result (including DN of the NetworkSlice MOI) for network slice allocation process.

Next change

Annex X (informative):
Plant UML source code

# X.1	General

This annex contains the PlantUML source code for procedures defined in clause 7 of the present document.

# X.a	Procedure of Network Slice Instance Allocation

@startuml

skinparam shadowing false

skinparam monochrome true

hide footbox

actor "NSMS_C" as NSMS_Consumer

participant "\tNSMS_P\t\t" as NSMS_Provider

alt NSMS_C sends AllocateNsi request

NSMS_Consumer -> NSMS_Provider: 1a-1. AllocateNsi(ServiceProfile) Request

NSMS_Provider -> NSMS_Provider: 1a-2. Creates NetworkSliceController MOI

NSMS_Provider --> NSMS_Consumer: 1a-3. AllocateNsi(ServiceProfileId) Response

NSMS_Provider --> NSMS_Consumer: 1a-4. notifyMOICreation(\n\tobjectClass=NetworkSliceController, \n\tobjectInstance=DN of NetworkSliceController MOI,...)

else NSMS_C sends createMOI request

NSMS_Consumer -> NSMS_Provider: 1b-1. createMOI(\n\t managedObjectClass=NetworkSliceController, \n\t attributeListIn=inputServiceProfile,...) Request

NSMS_Provider -> NSMS_Provider: 1b-2. Creates NetworkSliceController MOI

NSMS_Provider --> NSMS_Consumer: 1b-3. createMOI() Response

end alt

NSMS_Provider -> NSMS_Provider: 2. Decides to create a new NetworkSlice MOI\

\n\t or use an existing NetworkSlice MOI.

alt Using an existing NetworkSlice MOI

NSMS_Provider -> NSMS_Provider: 3a-1. Identifies the NetworkSlice MOI to re-use

NSMS_Provider -> NSMS_Provider: 3a-2. Configures NetworkSlice MOI with \n DN of NetworkSliceController MOI

NSMS_Provider -> NSMS_Provider: 3a-3. Configures NetworkSliceController MOI with \n DN of NetworkSlice MOI

opt If the existing NetworkSlice MOI needs to be modified

ref over NSMS_Provider

3a-4. Procedure of Network Slice Instance Modification

end ref

end opt

else Creating a new NetworkSlice MOI

NSMS_Provider -> NSMS_Provider: 3b-1. Derives the network slice subnet related requirements

ref over NSMS_Provider

4b-2. Procedure of Network Slice Subnet Instance Allocation

end ref

|||

NSMS_Provider -> NSMS_Provider: 3b-3. Creates NetworkSlice MOI and \n configures NetworkSlice MOI with \nDN of NetworkSliceController MOI and \nDN of NetworkSliceSubnet MOI

NSMS_Provider -> NSMS_Provider: 3b-4. Configures NetworkSliceController MOI with \n DN of NetworkSlice MOI

|||

end alt

NSMS_Provider -> NSMS_Provider: 4. Updates attributes of NetworkSliceController MOI\nindicating completion of the procedure

group Monitoring network slice allocation progress and result. \nThis can be happened anytime after step NetworkSliceController is created (i.e., step 1a-2 or step 1b-2)

alt obtain the progress and result by subscribe-notification method

NSMS_Provider -> NSMS_Consumer: 5a. Notify network slice allocation progress and result \n (DN of NetworkSliceController MOI and its attributes)

else obtain the progress and result by query method

NSMS_Consumer -> NSMS_Provider: 5b-1. Request to query the \n NetworkSliceController information (DN of NetworkSliceController)

NSMS_Provider -> NSMS_Consumer: 5b-2. Response with the NetworkSliceController \n information (DN of NetworkSliceController and values of its attribute)

end alt

@enduml

End of change