3GPP TSG-SA5 Meeting #149 	S5-234591

Berlin, Germany, 22 - 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0177 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Procedure of network slice subnet instance deallocation to support asynchronous operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Telefonica, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNETSLICE_PRO |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | S5-233155 was endorsed with the following: - Introduce a solution using NetworkSliceController IOC to monitor progress and result for network slice (subnet) allocation and deallocation as an enhancement to support async mode in 3GPP Rel-18. - The solution introduced above will support both network slice lifecycle management using allocate/deallocate operation and CRUD based operation. Network Slice and Network Slice Subnet LCM operations are long running and hence need to be asynchronous. The changes in this CR introduces support for asynchronous mode of operations for Procedure of network slice subnet instance deallocation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Procedure of network slice subnet instance deallocation to support asynchronous or long running operations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support of shared objects as well as asynchronous or long running operations for network slice subnet instance deallocation is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS28.541 CR 0800 TS28.623 CR 0214 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - S5-233902 is revised to S5-234591 |  |  |  |  |  |  |  |  |

1st Change

## 7.5	Procedure of network slice subnet instance deallocation

Figure 7.5-1 depicts the procedure of deallocating a network slice subnet instance by the network slice subnet management service provider to satisfy the NSSI deallocation request received from an authorized consumer.

Figure 7.5-1: Network slice subnet instance deallocation procedure

1a-1) The network slice subnet management service provider (NSSMS_P) receives DeallocateNssi request (see DeallocateNssi operation defined in clause 6.5.4) from network slice subnet management service consumer (NSSMS_C) indicating that the NetworkSliceSubnet MOI is no longer needed for the given requirements i.e SliceProfile.

1a-2)	NSSMS_P identifies the DN of NetworkSliceSubnet MOI from the attribute networkSliceSubnetRef and value of attribute sliceProfileId from the NetworkSliceSubnetController MOI to be deleted.

1a-3)	NSSMS_P updates NetworkSliceSubnet MOI (identified in step 1b-2) attribute networkSliceSubnetControllerRef to remove the DN of NetworkSliceSubnetController MOI to be deleted.

1a-4)	NSSMS_P deletes NetworkSliceSubnetController MOI.

1a-5) NSSMS_P sends response (see DeallocateNssi operation defined in clause 6.5.4) of NSSI deallocation service to NSSMS_C.

1b-1)	If CRUD operations are visible to the NSSMS_C, then NSSMS_P receives a deleteMOI request (see deleteMOI operation defined in TS 28.532 [8]) from NSSMS_C with network slice controller (see NetworkSliceSubnetController IOC and its attribute defined in TS 28.541[6])).

1b-2)	NSSMS_P identifies the DN of NetworkSliceSubnet MOI from the attribute networkSliceSubnetRef and value of attribute sliceProfileId from the NetworkSliceSubnetController MOI to be deleted.

1b-3)	NSSMS_P updates NetworkSliceSubnet MOI (identified in step 1b-2) attribute networkSliceSubnetControllerRef to remove the DN of NetworkSliceSubnetController MOI to be deleted.

1b-4)	NSSMS_P deletes NetworkSliceSubnetController MOI.

1b-5)	NSSMS_P sends the response to NSSMS_C for the deleteMOI request.

2) The NSSMS_P may decide to delete or modify the NetworkSliceSubnet MOI (identified in step 1b-2).

3a)	If NSSMS_P decides to delete the NetworkSliceSubnet MOI, then it performs the following steps:

3a-1)	NSSMS_P invokes procedure of network slice subnet instance deallocation as described in clause 7.5 for each constituent NetworkSliceSubnet MOI(s) that is configured in attribute networkSliceSubnetRef of NetworkSliceSubnet MOI (identified in step 1b-2).

3a-2) For each constituent NF instance (ManagedFunction MOI DN that is configured in attribute of managedFunctionRef of NetworkSliceSubnet MOI identified in step 1b-2), NSSMS_P invokes procedure of NF instance deletion as described in clause 7.12 only if the NF is dedicated for the NetworkSliceSubnet MOI (identified in step 1b-2)  and not being used by any other NetworkSliceSubnet MOIs in the network, otherwise, NSSMS_P procedure of NF instance modification as described in clause 7.11.

3a-3) NSSMS_P invokes TN related coordination procedure with responsible manager as described in clause 7.9 if the NetworkSliceSubnet MOI consists of TN part.

3a-4) NSSMS_P invokes NS termination procedure if the NetworkSliceSubnet MOI contains virtualized part.

Note:	NS termination procedure is described in TS 28.526 [7].

3a-5)	NSSMS_P deletes the NetworkSliceSubnet MOI.

3b)	If NSSMS_P decides to modify the NetworkSliceSubnet MOI, then it performs the following sub steps:

3b-1)	For each constituent NetworkSliceSubnet MOI, NSSMS_P invokes the procedure of network slice subnet instance modificationas described in clause 7.6.

3b-2)	NSSMS_P modifies the NetworkSliceSubnet MOI to update the sliceProfileList (using the sliceProfileId identified in step 1b-2) and may update other attributes.

If the NSSMS_C has subscribed for the notification, then NSSMS_P sends notification for deletion (see notifyMOIDeletion operation defined in TS 28.532 [8]) and notification for the attribute value changes (see notifyMOIAttributeValueChanges operation defined in TS 28.532 [8]) of NetworkSliceSubnetController MOI and NetworkSliceSubnet MOI.

End of change