3GPP TSG-SA5 Meeting #149 	S5-234624

Berlin, Germany, 22 - 26 May 2023

# Source:	Ericsson LM

# Title:	Rel-18 pCR 28.827 Adding additional actor with CHF solution in clause 7.3

# Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

Adding solution in clause 7.3 for additional actor with CHF.

# 4	Detailed proposal

First change

#### 7.3.4.1	Solution #3.1: CDR in home MNO for wholesale of non-roaming 5G of additional actor without CHF

Second change

##### 7.3.4.1.2	Reference architecture

Figure 7.3.4.2-1: Additional actor 5G scenario in service-based interface representation

Figure 7.3.4.1.2-2: Additional actor 5G scenario in reference point representation

The AMF, SMF, and SMSF with CTF interact with CHF using Nchf interface, without any requirement of quota management. The CCS produces CDRs which may be send for wholesale charging which then can be used to aggregate and calculate the amount of data per additional actor, it could be done based on SUPI, IMSI series, SNSSAI(s), DNN(s) etc., the wholesale part is only included for completeness and is currently outside the scope of 3GPP SA5.

Third change

#### 7.3.4.1	Solution #3.x: CDR in home MNO for wholesale of non-roaming 5G of additional actor with CHF

##### 7.3.4.1.1	General

A possible solution for key issue #3a, #3b, and #3c covering requirements REQ-CH_MNOMVNO-01, REQ-CH_MNOMVNO-02, and REQ-CH_MNOMVNO-03, charging for 5G data connectivity, connection and mobility, SMS, provided to the additional actor (owning a CHF) by the home MNO, covering non-roaming case.

##### 7.3.4.1.2	Reference architecture

The architecture would be the same as in solution #1.3, clause 7.1.4.3.2, and solution #1.6, clause 7.1.4.6.2, where the Visited MNO is replaced by the Home MNO and the Home MNO is replaced by the Additional Actor.

##### 7.3.4.1.3	Message flows

The flow would be the same as clauses 7.1.4.3.3 and 7.1.4.6.3 where the V-CHF corresponds to the home MNO’s CHF and the H-CHF corresponds to the additional actor’s CHF.

End of changes