3GPP TSG-SA5 Meeting #149 	S5-233862

Berlin, Germany, 22 – 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | Draft CR | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Input to draft CR TS 28.105 Use case and requirements for ML Testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shangai Bell, Intel, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TBD |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use case of Testing ML entities has been discussed and agreed (in TR 28.908). This CR is to normatively propose the use case and requirements for the mentioned use case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use case description and requirements have been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for the use case “ML Testing” |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.V(new clauses added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

## 6.V	ML testing

### 6.V.1	Description

During ML entity training phase, after the training and validation, the ML entity needs to be tested to evaluate the performance of the ML entity when it conducts inference using the testing data. Testing may involve interaction with third parties (besides the developer of the ML training function), e.g., the operators may use the ML training function or third-party systems/functions that may rely on the inference results computed by the ML entity for testing.

If the testing performance is not acceptable or does not meet the pre-defined requirements, the consumer may request the ML training producer to re-train the ML entity with specific training data and/or performance requirements.

### 6.V.2	Use cases

#### 6.V.2.1	Consumer-requested ML entity testing

After receiving an ML training report about a trained ML entity from the ML training MnS producer, the consumer may request the ML testing MnS producer to test the ML entity before applying it to the target inference function.

The ML testing is to conduct inference on the tested ML entity using the testing data as the inference inputs and produce the inference output for each testing dataset example.

The ML testing MnS producer may be the same as or different from the ML training MnS producer.

After completing the ML testing, the ML testing MnS producer provides the testing report indicating the success or failure of the ML testing to the consumer. For a successful ML testing, the testing report contains the testing results, i.e., the inference output for each testing dataset example.

The ML testing MnS producer needs to have the capabilities to provide the services needed to enable the consumer to request testing and receive results on the testing of an ML entity.

##### 6.V.2.2	Producer-initiated ML entity testing

The ML entity testing may also be initiated by the MnS producer, after the ML entity is trained and validated. A consumer (e.g., an operator) may still need to define the policies (e.g., allowed time window, maximum number of testing iterations, etc.) for the testing of a given ML entity. The consumer may pre-define performance requirements for the ML entity testing and allow the MnS producer to decide on whether re-training/validation need to be triggered. Re-training may be triggered by the testing MnS producer itself based on the performance requirements supplied by the MnS consumer.

### 6.L.3	Requirements for ML testing

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_TEST-1 | The ML testing MnS producer should have a capability to allow an authorized consumer to request the testing of a specific ML entity. | Consumer-requested ML entity testing (clause 6.V.2.1) |
| REQ-ML_TEST-2 | The ML testing MnS producer shall have a capability to trigger the testing of an ML entity and allow the MnS consumer to set the policy for the testing. | Producer-initiated ML entity testing (6.V.2.2) |
| REQ-ML_TEST-3 | The ML testing MnS producer shall have a capability to report the performance of the ML entity when it performs inference on the testing data. | Consumer-requested ML entity testing (clause 6.V.2.1), and producer-triggered ML entity testing (clause 6.V.2.2) |

End of Changes