3GPP TSG-SA5 Meeting #149 	S5-234666

Berlin, Germany, 22 - 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0042 | rev | - | Current version: | 17.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the solution for 5GC Network Expectation of delivering 5GC network and performance assurance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AsiaInfo, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_ph2 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The objective of IDMS_MN_ph2 is that specifies requirements and solutions for new scenarios for Intent driven approach for 5GC management |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the solution for 5GC Network Expectation of 5GC network delivering and performance assurance |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[3]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[4]	3GPP TS 28.530: "Management and orchestration; Concept, use cases and requirements".

[5]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[6]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM); Integration Reference Point (IRP); Information Service (IS)".

[7]	TM Forum IG1253A: "Intent Common Model v1.1.0".

[8]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[9]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

[10]	3GPP TS 28.658: "Telecommunications management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[X]	3GPP TS 28.552: " Management and orchestration; 5G performance measurements".

[Y]	3GPP TS 29.510: " 5G System; Network Function Repository Services".

2st Change

6.2.2	Scenario specific IntentExpectation definition

6.2.2.1	Scenario specific IntentExpectation definition

6.2.2.1.1	Radio Network Expectation

6.2.2.1.1.1	Definition

Radio Network Expectation is an IntentExpectation which can be used to represent MnS consumer's expectations for radio network (RAN SubNetwork) delivering and performance assurance.

The Radio Network Expectation is defined by utilizing the construct of the generic IntentExpectation <<dataType>> with set of allowed values and concrete dataTypes specified.

Following are the specific allowed values when implemented the IntentExpectation for Radio Network Expectation.

Table 6.2.2.1.1.1-1

| Attribute Name | Allowed Values |
| --- | --- |
| objectType (CM) | RAN SubNetwork |
| objectInstance (CM) | DN of the RAN SubNetwork |

NOTE:	Following are the qualifier description for attribute "objectType" and "objectInstance":

-	In case of the intent expectation is not for a specific RAN SubNetwork instance or/and MnS consumer have no knowledge of the DN of this RAN SubNetwork instance, the attribute "objectType" needs to be specified.

-	In case of the intent expectation is for a specific RAN SubNetwork instance and MnS consumer have the knowledge of the DN of this RAN SubNetwork instance, the attribute "objectInstance" needs to specified.

6.2.2.1.1.2	ObjectContexts

Following provides the concrete ObjectContexts for Radio Network Expectation based on the common structure of ObjectContext. The properties of the attributes in the following table should be same with properties of ObjectContexts defined in clause 6.2.1.3.

Table 6.2.2.1.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| coverageAreaPolygonContext | O | T | T | F | F |
| coverageTACContext | O | T | T | F | F |
| pLMNContext | O | T | T | F | F |
| nRFqBandContext | O | T | T | F | F |
| rATContext | O | T | T | F | F |

6.2.2.1.1.3	ExpectationTargets

Following provides the concrete ExpectationTargets for Radio Network Expectation based on the common structure of ExpectationTarget. The properties of the attributes in the following table should be same with properties of ExpectationTargets defined in clause 6.2.1.3.

Table 6.2.2.1.1.3-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| weakRSRPRatioTarget | O | T | T | F | F |
| lowSINRRatioTarget | O | T | T | F | F |
| aveULRANUEThptTarget | O | T | T | F | F |
| aveDLRANUEthptTarget | O | T | T | F | F |
| lowULRANUEThptRatioTarget | O | T | T | F | F |
| lowDLRANUEThptRatioTarget | O | T | T | F | F |

6.2.2.1.2	Service Support Expectation

6.2.2.1.2.1	Definition

Service Support Expectation is an IntentExpectation which can be used to represent MnS consumer's expectations for service deployment.

The Service Support Expectation is defined utilizing the constructs of the generic IntentExpectation <<dataType>> with set of allowed values and concrete dataTypes specified.

Following are the specific allowed values when implemented the IntentExpectation for Service Support Expectation.

Table 6.2.2.1.2.1-1

| Attribute | Allowed Values |
| --- | --- |
| ObjectType (CM) | ServiceSupport |
| objectInstance (CM) | DN of the ServiceSupport |

NOTE:	Following are the qualifier description for attribute "objectType" and "objectInstance":

-	In case of the intent expectation is not for a specific service instance or/and MnS consumer have no knowledge of the DN of this service instance, the attribute "objectType" needs to be specified.

-	In case of the intent expectation is for a specific service instance and MnS consumer have the knowledge of the DN of this service instance, the attribute "objectInstance" needs to be specified.

6.2.2.1.2.2	ObjectContexts

Following provides the concrete ObjectContexts for Service Support Expectation based on the common structure of ObjectContext. The properties of the attributes in the following table should be same with properties of ObjectContexts defined in clause 6.2.1.3.

Table 6.2.2.1.2.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| edgeIdenfiticationIdContext | CM | T | T | F | F |
| edgeIdenfiticationLocContext | CM | T | T | F | F |
| coverageAreaTAContext | CM | T | T | F | F |

NOTE:	Following are the qualifier description for attribute "edgeIdentificationId" and " edgeIdentificationLoc":

-	In case of the Service deployment is needed at a particular edge data network, the attribute " edgeIdentificationId " needs to be specified.

-	In case of the Service deployment is needed at a particular location, the attribute "edgeIdentificationLoc" needs to be specified.

6.2.2.1.2.3	ExpectationTargets

Following provides the concrete ExpectationTargets for Service Support Expectation based on the common structure of ExpectationTarget. The attribute properties defined in the table below should be same with the properties defined for ExpectationTargets in clause 6.2.1.3.

Table 6.2.2.1.2.3-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dlThptPerUETarget | O | T | T | F | F |
| UlThptPerUETarget | O | T | T | F | F |
| dLLatencyTarget | O | T | T | F | F |
| uLLatencyTarget | O | T | T | F | F |
| maxNumberofUEsTarget | O | T | T | F | F |
| activityFactorTarget | O | T | T | F | F |
| uESpeedTarget | O | T | T | F | F |

6.2.2.1.2.4	ExpectationContexts

Following provides the concrete ExpectationContexts for Service Deployment Expectation based on the common structure of ExpectationContext. The attribute properties defined in the table below should be same with the properties defined for ExpectationContexts in clause 6.2.1.3.

Table 6.2.2.1.2.4-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serviceStartTimeContext | O | T | T | F | F |
| serviceEndTimeContext | O | T | T | F | F |
| uEMobilityLevelContext | O | T | T | F | F |
| resourceSharingLevelContext | O | T | T | F | F |

##### 6.2.2.1.X	5GC Network Expectation

6.2.2.1.X.1	Definition

5GC Network Expectation is an IntentExpectation which can be used to represent MnS consumer's expectations for 5GC network (5GC SubNetwork) delivering

The 5GC Network Expectation is defined by utilizing the construct of the generic IntentExpectation <<dataType>> with set of allowed values and concrete dataTypes specified.

Following are the specific allowed values when implemented the IntentExpectation for 5GC Network Expectation.

Table 6.2.2.1.1.1-1

| Attribute Name | Allowed Values |
| --- | --- |
| objectType (CM) | 5GC SubNetwork |
| objectInstance (CM) | DN of the 5GC SubNetwork |

NOTE:	Following are the qualifier description for attribute "objectType" and "objectInstance":

-	In case of the intent expectation is not for a specific 5GC SubNetwork instance or/and MnS consumer have no knowledge of the DN of this 5GC SubNetwork instance, the attribute "objectType" needs to be specified.

-	In case of the intent expectation is for a specific 5GC SubNetwork instance and MnS consumer have the knowledge of the DN of this 5GC SubNetwork instance, the attribute "objectInstance" needs to specified.

6.2.2.1.X.2	ObjectContexts

Following provides the concrete ObjectContexts for 5GC Network Expectation based on the common structure of ObjectContext. The properties of the attributes in the following table should be same with properties of ObjectContexts defined in clause 6.2.1.3.

Table 6.2.2.1.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nfTypeContext | O | T | T | F | F |
| nfInstancelocationContext | O | T | T | F | F |
| pLMNInfoContext | O | T | T | F | F |
| taicontext | O | T | T | F | F |

6.2.2.1.X.3	ExpectationTargets

Following provides the concrete ExpectationTargets for 5GC Network Expectation based on the common structure of ExpectationTarget. The properties of the attributes in the following table should be same with properties of ExpectationTargets defined in clause 6.2.1.3.

Table 6.2.2.1.1.3-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| maxNumberofPDUsessionsTarget | O | T | T | F | F |
| maxNumberofRegisteredsubscribersTarget | O | T | T | F | F |

Editor's note: other ExpectationTargets of 5GC network is FFS.

3st Change

6.2.2.2	Attribute definition

Table 6.2.2.2-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| coverageAreaPolygonContext | It describes the coverage areas for the RAN SubNetwork that the intent expectation is applied in the form of polygon. CoverageAreaPolygonContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "CoverageAreaPolygon" - contextCondition: " IS_WITHIN_RANGE " - contextValueRange: a list of CoverageArea defined in 3GPP TS 28.541 [5] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| coverageTACContext | It describes the coverage areas for the RAN SubNetwork that the intent expectation is applied in the form of TAC. CoverageTACContext is a Context including attributes: contextAttribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "CoverageAreaTAC" - contextCondition: " IS_WITHIN_RANGE " - contextValueRange: a list of nRTAC defined in 3GPP TS 28.541 [5] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| plMNContext | It describes the PLMN(s) supported by the RAN SubNetwork that the intent expectation is applied. PLMNContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "PLMN" - contextCondition:" IS_WITHIN_RANGE " - contextValueRange: a list of PLMNId defined in TS 28. 658 [10] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| nRFqBandContext | It describes the nRFqBands supported by the RAN SubNetwork that the intent expectation is applied. nRFqBandContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "NRFqBand" - contextCondition: " IS_WITHIN_RANGE " - contextValueRange: a list of NRFqBand expressed as string. Valid frequency band values are specified in clause 5.4.2 in 3GPP TS 38.104 [8] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| rATContext | It describes the RAT supported by the RAN SubNetwork that the intent expectation is applied. RATContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "RAT" - contextCondition: " IS_WITHIN_RANGE " - contextValueRange: a list of ENUM with allowed value: UTRAN, EUTRAN and NR | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| weakRSRPRatioTarget | It describes the downlink weak coverage ratio target for the RAN SubNetwork that the intent expectation is applied. WeakRSRPRatioTarget is an ExpectationTarget including attributes: targetName, targetCondition,targetValueRange and targetContext. Following are the allowed values: - targetName: "WeakRSRPRatio" - targetCondition: "IS_LESS_THAN" - targetValueRange: integer with allowed value [0,100] - targetContext: WeakRSRPContext | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| WeakRSRPRatioTarget.weakRSRPContext | It describes the threshold for downlink weak RSRP of the RAN SubNetwork that the intent expectation is applied. WeakRSRPContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "WeakRSRPThreshold" - contextCondition: "IS_LESS_THAN" - contextValueRange: Float | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| LowSINRRatioTarget | It describes the low SINR ratio target for the RAN SubNetwork that the intent expectation is applied. LowSINRRatioTarget is an ExpectationTarget including attributes: targetName, targetCondition,targetValueRange and targetContxt. Following are the allowed values: - targetName: "LowSINRRatio" - targetCondition: "IS_LESS_THAN" - targetValueRange: integer with allowed value [0,100] - targetContext: LowSINRContext | type:ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| LowSINRRatioTarget.lowSINRContext | It describes the threshold for low SINR for RAN SubNetwork that the intent expectation is applied. LowSINRContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "LowSINRThreshold" - contextCondition: "IS_LESS_THAN" - contextValueRange: integer | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| aveULRANUEThptTarget | It describes the average UL RAN UE throughput target for RAN SubNetwork that the intent expectation is applied. AveULRANUEThptTarget is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. Following are the allowed values: - targetName: "AveULRANUEThpt" - targetCondition: "IS_GREATER_THAN" - targetValueRange: integer | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| aveDLRANUEThptTarget | It describes the average DL RAN UE throughput target for RAN SubNetwork that the intent expectation is applied. AveDLRANUEThptTarget is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. Following are the allowed values: - targetName: "AveDLRANUEThpt" - targetCondition: "IS_GREATER_THAN" - targetValueRange: integer | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| lowULRANUEThptRatioTarget | It describes the low UL RAN UE throughput ratio target for the RAN SubNetwork that the intent expectation is applied. LowULRANUEThptRatioTarget is an ExpectationTarget including attributes: targetName, targetCondition,targetValueRange and targetContext. Following are the allowed values: - targetName: "LowULRANUEThptRatio" - targetCondition: "IS_LESS_THAN" - targetValueRange: integer with allowed value [0,100] - targetContext: LowULRANUEThptContext | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| LowULRANUEThptRatioTarget.lowULRANUEThptContext | It describes the threshold for the low UL RAN UE throughput of the RAN SubNetwork that the intent expectation is applied LowULRANUEThptContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "LowULRANUEThptThreshold" - contextCondition: "IS_LESS_THAN" - contextValueRange: Float | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| lowDLRANUEThptRatioTarget | It describes the low DL RAN UE throughput ratio target for the RAN SubNetwork that the intent expectation is applied. LowDLRANUEThptRatioTarget is an ExpectationTarget including attributes: targetName, targetCondition,targetValueRange and targetContext. Following are the allowed values: - targetName: "LowDLRANUEThptRatio" - targetCondition: "IS_LESS_THAN " - targetValueRange: integer with allowed value [0,100] - targetContext: LowDLRANUEThptContext | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| owDLRANUEThptRatioTarget.lowDLRANUEThptContext | It describes the threshold for the low DL RAN UE throughput of the RAN SubNetwork that the intent expectation is applied. LowDLRANUEThptContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "LowDLRANUEThptThreshold" - contextCondition: "IS_LESS_THAN" - contextValueRange: Float | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| serviceStartTimeContext | This describes the start time at which the service shall be available. This contributes to the selection of the appropriate edge data network to be used for service deployment. Following are the allowed values: - contextAttribute: "serviceStartTime" - contextCondition: "IS_EQUAL_TO" - contextValueRange: start time stamp | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| serviceEndTimeContext | This describes the end time after which the service shall not be available. This contributes to the selection of the appropriate edge data network to be used for service deployment. Following are the allowed values: - contextAttribute: "serviceEndTime" - contextCondition: "IS_EQUAL_TO" - contextValueRange: end time stamp | type:Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| edgeIdenfiticationIdContextt | This identifies the edge network where the service need to be deployed. This should be used when the edge identification is known to the consumer Following are the allowed values: - contextAttribute: "edgeIdentificationId" - contextCondition: "IS_EQUAL_TO" - contextValueRange: EDNidentifier as defined in 3GPP TS 28.538 [9] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| edgeIdenfiticationLocContext | This identifies the location where the service need to be deployed. This should be used when the edge identification is not known to the consumer Following are the allowed values: - contextAttribute: "edgeIdentificationTarget" - contextCondition: "IS_EQUAL_TO" - contextValueRange: geographical target location. This will take a form of either single latitude & longitude or a TAI | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| coverageAreaTAContext | It describes Tracking Coverage Areas for service supporting that the intent expectation is applied. coverageAreaTAContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "coverageAreaTA" - contextCondition: "IS_WITHIN_RANGE" - contextValueRange: a list of Tracking Coverage Areas, coverageAreaTAList in clause 6.3.1 in 3GPP TS 28.541[5] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| dlThptPerUETarget | It describes the DL throughput target by the per UE for the service Supporting that the intent expectation is applied. DLThptperUETarget is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange: - targetName: "DLThptperUE" - targetCondition: "IS_GREATER_THAN" - targetValueRange: dlThptPerUE defined in clause 6.3.1 of 3GPP TS 28.541 [5] | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| UlThptPerUETarget | It describes the UL throughput target by the per UE for the service Supporting that the intent expectation is applied. ULThptperUETarget is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. - targetName: "UlThptperUE" - targetCondition: "IS_GREATER_THAN" - targetValueRange: uLThptPerUE defined in clause 6.3.1 of 3GPP TS 28.541 [5] | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| dLLatencyTarget | It describes the DL latency target for the service Supporting that the intent expectation is applied. DLLatencyTarget is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. - targetName: "UlThptperUE" - targetCondition: "IS_LESS_THAN" - targetValueRange: dLLatency defined in clause 6.3.1 of 3GPP TS 28.541 [5] | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| uLLatencyTarget | It describes the UL latency target for the service Supporting that the intent expectation is applied. UlThptperUETarget is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. - targetName: "UlThptperUE" - targetCondition: "IS_LESS_THAN" - targetValueRange: ULLatency defined in clause 6.3.1 of 3GPP TS 28.541 [5] | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| maxNumberofUEsTarget | It describes the maximum number of UEs for service supporting that the intent expectation is applied. maxNumberofUEsContext is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. Following are the allowed values: - targetName: "maxNumberofUEs" - targetCondition: " IS_LESS_THAN" - targetValueRange: maxNumberofUEs in clause 6.3.1 in 3GPP TS 28.541 [5] | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| activityFactorTarget | It describes the percentage value of the amount of simultaneous active UEs to the total number of UEs where active means the UEs are exchanging data with the network for service supporting that the intent expectation is applied. activityFactorContext is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. Following are the allowed values: - targetName: " activityFactorContext " - targetCondition: " IS_EQUAL_TO" - targetValueRange: activityFactor in clause 6.3.1 in 3GPP TS 28.541 [5] | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| uESpeedTarget | It describes the maximum speed (in km/hour) supported for service supporting that the intent expectation is applied. uESpeedContext is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. Following are the allowed values: - targetName: "uESpeedContext" - targetCondition: " IS_LESS_THAN" - targetValueRange: uESpeedContext in clause 6.3.1 in 3GPP TS 28.541 [5] | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| uEMobilityLevelContext | It describes the mobility level of UE for service supporting that the intent expectation is applied. uEMobilityLevelContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: " uEMobilityLevel " - contextCondition: "IS_WITHIN_RANGE" - contextValueRange: uEMobilityLevel in clause 6.3.1 in 3GPP TS 28.541 [5] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| resourceSharingLevelContext | It describes the resource sharing level for service supporting that the intent expectation is applied. resourceSharingLevelContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "resourceSharingLevel" - contextCondition: "IS_WITHIN_RANGE" - contextValueRange: resourceSharingLevel in clause 6.3.1 in 3GPP TS 28.541 [5] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| nfTypeContext | It identifies the types of NF supported by the 5GC SubNetwork that the intent expectation is applied. nfTypeContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: " nfType " - contextCondition:" IS_WITHIN_RANGE " - contextValueRange: a list of ENUM with allowed value: Enumeration NFType in 3GPP TS 29.510[Y] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| nfInstancelocationContext | It describes the location of NF instance supported by the 5GC SubNetwork that the intent expectation is applied. nfInstancelocationContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: " nfInstancelocation " - contextCondition:” IS_WITHIN_RANGE " - contextValueRange: a list of ENUM with allowed value: Enumeration LocalityType in 3GPP TS 29.510[Y] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| plMNInfoContext | It describes the PLMN(s) supported by the 5GC SubNetwork that the intent expectation is applied. pLMNInfoContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "PLMNInfo" - contextCondition:" IS_WITHIN_RANGE " - contextValueRange: a list of PLMNInfo defined in TS 28. 541 [5] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| taicontext | It describes the tracking area Identifiers supported by the 5GC SubNetwork that the intent expectation is applied. taiContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "tai" - contextCondition:" IS_WITHIN_RANGE " - contextValueRange: a list of tai defined in TS 28. 541 [5] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| maxNumberofPDUsessionsTarget | It describes the maximum number of PDU seesions for 5GC SubNetwork that the intent expectation is applied. maxNumberofPDUsessions Context is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. Following are the allowed values: - targetName: "maxNumberofPDUsessions" - targetCondition: " IS_LESS_THAN" - targetValueRange: maxNumberofPDUsessions in clause 5.3.1.2 in 3GPP TS 28.552 [X] | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| maxNumberofRegisteredsubscribersTarget | It describes the maximum number of Registered subscribers for 5GC SubNetwork that the intent expectation is applied. maxNumberofRegisteredsubscribers Context is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. Following are the allowed values: - targetName: "maxNumberofRegisteredsubscribers" - targetCondition: " IS_LESS_THAN" - targetValueRange: maxNumberofRegisteredsubscribers in clause 5.6.2 in 3GPP TS 28.552 [X] | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |