3GPP TSG-SA5 Meeting #149 	S5-234670

Berlin, Germany, 22 – 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0032 | rev | 1 | Current version: | 17.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel18 CR 28312 Use case and Requirements on Intent conflicts |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shangai Bell,Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_ph2 |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use case of Intent conflicts has been discussed and agreed (in TR 28.912). This CR is to normatively propose the use case and requirements for the mentioned use case |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Use case description and requirements have been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for the use cases “Intent conflicts” |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1 (new clause added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge Link: |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-233876 |  |  |  |  |  |  |  |  |

Start of First change

### 5.X.	Generic use cases for Intent Driven Management

### 5.X.A	Intent conflicts

#### 5.X.A.1	Introduction

The MnS consumer may create an intent containing two or more intent expectations, and each intent expectation may contain multiple expectation targets. For example, a Radio Network related intent may express an intent with targets on radio network parameters (e.g., downlink transmit power, remote electrical tilt) or on KPIs (e.g., DL UE throughput target, average RSRP target, coverage area). On receiving and after analysing the intent, the MnS producer may realize that the intent expectations or expectation targets in one intent are contradicted, i.e. the MnS producer may detect conflicts in the intent. Also, an intent is considered to have conflict with other intents if the requirements (includes intent expectation and corresponding expectation targets) stated in one intent is conflicted with the requirements (includes intent expectation and corresponding expectation targets) stated in another intent. Following are the intent related conflict scenarios:

- 	Target conflict, which represents the conflict between two or more expectation targets within the same intent expectation.

-	Expectation conflict, which represents the conflict between two or more intent expectations with the same intent

- 	Intent conflict, which represents the conflict between two or more different intents.

For example, consider two targets target_1=: throughput > threshold_1 and target_2=: interference < threshold_2, and while trying to achieve target_1, target_2 gets degraded, which means actions taken by these two targets have mutually exclusive effects, so the producer will see that the targets are conflicting. There is intent conflict between them if they are in different intents, but there is expectation conflict or target conflict between them if they are in the same intent.

For the intent conflict, the two or more intents may be proposed by the same MnS consumer, or may be proposed by different MnS consumers. An example of the latter is that the MnS producer cannot satisfy the intents of two MnS consumers simultaneously. From the perspective of intent creation time, conflicting intents may be proposed at different time, so a newer intent may be in conflict with an intent that is being executed but has not yet been fulfilled. The MnS producer may terminate the intent execution task(s) to avoid the conflicted intents being executed simultaneously until the intent conflict is solved.

When such conflicts are detected, the MnS producer needs to notify the MnS consumer about the conflict, indicating the intent, intent expectations or expectation targets which give rise to the conflict. Additionally, the MnS producer may also notify the MnS consumer about the additional information (e.g., the impact for other expectation targets when fulfil the specified expectation targets in the same or different intent) for the conflict. The MnS producer can also provide solutions regarding intent conflicts, such as termination of the whole intent, recommended intent (e.g. recommended expectations or targets, or termination of part of intent), or updating the execution time of the intent.

Thereby, the MnS consumer may take actions (e.g., modify and delete the intent or intent expectation or expectation targets,) to address such intent conflict or MnS consumer may give some intent conflict handling guidelines (e.g. assign priority for such intent or intent expectation or expectation targets) to MnS producer to solve such intent conflict or eliminate the terminated state which is caused by the reason of conflict detected.

#### 5.X.A.2	Requirements

REQ-Intent_conflict-CON-1: The intent driven MnS producer should have the capability to inform the authorized MnS consumer about intent related conflicts, including intent conflict, expectation conflict and target conflict.

REQ-Intent_conflict-CON-2: The intent driven MnS producer should have the capability enabling the authorized MnS consumer to give to the MnS producer an intent indicating the prioritization to be applied when handling conflict among intents, expectations or targets.

End of Changes