3GPP TSG-SA5 Meeting #149	S5-234688

# Berlin, Germany,  22 - 26 May 2023 									    			 revision of S5-234149

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0426 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add measurements related to the NWDAF service failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MANWDAF |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently PM cannot support the measurements related to NWDAF service failure. The TR 28.864 proposed performance measurements of the NWDAF on service error aspect. It is necessary to add the related measurements in TS 28.552 based on the recommendations of TR 28.864. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added measurements related to NWDAF service failure based on the recommendations in TR 28.864. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance of NWDAF on service failure cannot be evaluated efficiently. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

5.X	Performance measurements for NWDAF

5.X.D	Measurements related to the NWDAF service failure

5.X.D.1	Number of events for time misconfiguration

a)	This measurement provides the number of events in which the "Time when Analytics information is needed" is lower than the "Supported Analytics Delay". The measurement is calculated per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [X]).

b)	CC

c)	When the "Time when Analytics is needed" is lower than the "Supported Analytics Delay", the related event will be added to the corresponding counter.

d)	An integer value

e)	The measurement name has the form DANS.TimeMisconfigEve, and

DANS.TimeMisconfigEve.AnalyticsID, where AnalyticsID identifies different analytics.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

5.X.D.2	Number of events for late delivery

a)	This measurement provides the number of events in which the "Time when Analytics information is needed" is lower than the "Time when Analytics is delivered to the consumer". The measurement is calculated per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [X]).

b)	CC

c)	When the "Time when Analytics information is needed" is lower than the "Time when Analytics is delivered to the consumer", the related event will be added to the corresponding counter.

d)	An integer value

e)	The measurement name has the form DANS.LateDeliveryEve, and

DANS.LateDeliveryEve.AnalyticsID, where AnalyticsID identifies different analytics.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

5.X.D.3	Number of events for late analytics generating

a)	This measurement provides the number of events in which the "Supported Analytics Delay" is lower than the "Time when Analytics is delivered to the consumer". The measurement is calculated per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [X]).

b)	CC

c)	When the "Supported Analytics Delay" is lower than the "Time when Analytics is delivered to the consumer", the related event will be added to the corresponding counter.

d)	An integer value

e)	The measurement name has the form DANS.LateAnaGenerating, and

DANS.LateAnaGenerating.AnalyticsID, where AnalyticsID identifies different analytics.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

End of modification