3GPP TSG-SA5 Meeting #149 	S5-234690

Berlin, Germany, 22 - 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0416 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add internal group ID to 5G LAN related performance measurements for SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GLAN_Mgt |  |  |  |  |  | Date: |  |  | 2023-5-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The requirements for 5G VN group communication may need to add new granularity of performance measurement for SMF, since network operator may require the performance meassurements related to 5G VN group communication, such the number of successful PDU session establishment of a 5G VN group communication, the mean number of PDU sessions of a 5G VN group communication, etc. This tdoc is to initiate a new granularity of internal VN group for some related performance measurements for SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add internal group ID as a new granularity of subcounter for some performance measurement for SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5G VN group is not applicable as the granularity of subconters. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1.1, 5.3.1.2, 5.3.1.3, 5.3.1.4, 5.3.1.5, 5.3.1.6.1, 5.3.1.6.2, 5.3.1.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

5.3.1	Session Management

5.3.1.1	Number of PDU sessions (Mean)

- This measurement provides the mean number of PDU sessions.

- SI

- The measurement is obtained by sampling at a pre-defined interval, the number of PDU sessions established by SMF, and then taking the arithmetic mean. The measurement is optionally split into subcounters per S-NSSAI.

This measurement is optionally split into subcounters per 5G VN internal Group ID.

- A single integer value

- SM.SessionNbrMean.SNSSAI, 
Where SNSSAI identifies the S-NSSAI.

SM.SessionNbrMean. InternalGroupID

Where internal Group ID identifies a 5G VN group communication, as specified in TS 23.501[4].

- SMFFunction

- Valid for packet switched traffic

- 5GS

5.3.1.2	Number of PDU sessions (Maximum)

a)	This measurement provides the max number of PDU sessions.

b)	SI

c)	The measurement is obtained by sampling at a pre-defined interval, the number of PDU sessions established by SMF, and then selecting the maximum value. The measurement is optionally split into subcounters per S-NSSAI.

This measurement is optionally split into subcounters per 5G VN internal Group ID.

d)	A single integer value

e)	SM.SessionNbrMax.SNSSAI
Where SNSSAI identifies the S-NSSAI

SM.SessionNbrMean.InternalGroupID

Where internal Group ID identifies a 5G VN group communication, as specified in TS 23.501[4].

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

2nd change

5.3.1.6.1	Number of requested PDU session modifications (UE initiated)

a)	This measurement provides the number of PDU session modification requests (initiated by UE) received by the SMF.

b)	CC.

c)	On receipt of Nsmf_PDUSession_UpdateSMContext Request which includes the N1 SM container IE indicating the "PDU Session Modification Request" (see TS 23.502 [7]) by the SMF from AMF.

On receipt of Nsmf_PDUSession_UpdateSMContext Request, the SMF may generate subcounters sorts by internal Group ID per 5G VN group communication.

d)	A single integer value.

e)	SM.PduSessionModUeInitReq.

SM.PduSessionModUeInitReq. InternalGroupID

Where internal Group ID identifies a 5G VN group communication, as specified in TS 23.501[4].

f)	SMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

5.3.1.6.2	Number of successful PDU session modifications (UE initiated)

a)	This measurement provides the number of successful PDU session modifications (initiated by UE) acknowledged by the SMF.

b)	CC.

c)	On transmission of Nsmf_PDUSession_UpdateSMContext Response indicating a successful PDU session modification by the SMF to AMF as reply to a smf_PDUSession_UpdateSMContext Request that includes the N1 SM container IE indicating the "PDU Session Modification Complete" (see TS 23.502 [7]) for a PDU session modification request (initiated by the UE).

This measurement is optionally split into subcounters per 5G VN internal Group ID.

d)	A single integer value.

e)	SM.PduSessionModUeInitSucc.

SM.PduSessionModUeInitReq. InternalGroupID

Where internal Group ID identifies a 5G VN group communication, as specified in TS 23.501[4].

f)	SMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

End of changes