3GPP TSG-SA5 Meeting #149 	S5-234737

Berlin, Germany, 22 - 26 May 2023

# Source:	Ericsson, Deutsche Telekom

# Title:	pCR 28.836 Add alternatives and evaluation to 6.2.2

# Document for:	Approval

Agenda Item:	6.7.3

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	3GPP TS 28.312 Management and orchestration; Intent driven management services for mobile networks

[2]	3GPP TR 28.912 Study on enhanced intent driven management services for mobile networks v1.2.0

# 3	Rationale

The Study on intent-driven management for network slicing has documented use cases, requirements, solutions, and procedures. Use cases in the areas of both network slice life cycle management and network slice service assurance have been documented. While the network slice lifecycle management is responsibility of the producer the requirements are controlled by the consumer, the format of the requirements follow that of the intent expectation data types.

In clause 6.2.2 a single alternative is described for the solution for expressing service requirements as intent expectations, there are however other alternatives that need to be included in the report to allow an evaluation to take place.

The Editor’s Note “Whether the exact mapping between the ServiceProfile attributes and IntentExpectation components is to be standardization in FFS”. Has been addressed as the following sentence is added with each alternative “The attributes that are not mapped in any of the intent datatypes can be treated as extension where its mapping is implementation dependent.”

The alternatives and evaluation are proposed to be added to the report as per detailed proposal.

# 4	Detailed proposal

Start of changes.

## 6.2		Solution for expressing service requirements as intent expectations.

### 6.2.1	Description

In TS 28.541 [4], clause 6.3.3.1 the description says that the service profile represents the properties of the network slice related requirements and that these should be supported by a NetworkSlice instance in a 5G network. Based on the definition for service profile, it is obvious that service profile represent MnS consumer’s expectations for network slice, which is aligned with intent definition (expectations including requirements, goals and constraints given to a 3GPP system, without specifying how to achieve them).

Following are some benefits to introduce the intent driven approach for network slice compared to existing slice solution:

- 	Simplified approach, allows using one single unified solution (CRUD operation with intent NRM fragment) to support the wide variety of slice management functionalities which fulfilled by different solutions in the existing slicing management approach, including

-	Network Slice provisioning allocation/deallocation functionality. In existing slicing management approach, this is implemented by (de-)allocateNSI operation defined in TS 28.531.

-	Network Slice modification, query functionality. In existing slicing management approach, this is implemented by CRUD operation with Network slicing NRM fragment defined in TS 28.532 and TS 28.541.

-	Network Slice SLS assurance functionality. In existing slicing management approach, this is implemented by CRUD operation with Assurance closed control loop NRM fragment defined in TS 28.532 and TS 28.536.

-	Network Slice Feasibility check. In existing slicing management approach, this is implemented by CRUD operation with feasibility check NRM fragment defined in TS 28.532 and TS 28.541.

-	Allows decoupling the lifecycle management of network slice requirements from the lifecycle management of the actual network slice. This enables MnS consumer to focus on expressing network slicing requirements (and obtain fulfilment information for those requirements), without knowing how to deal with the detailed management of a network slice. The latter is on MnS producer side.

Currently the ServiceProfile is modelled as a set of properties, while IntentExpectation is composed of expectationObject, expectationTargets and expectationContexts, so it is necessary to investigate each attribute in ServiceProfile to identify which attribute can be represented by which intent expectation components (including expectationObject, expectationTargets and expectationContexts).

The following figure gives one example to illustrate how ServiceProfile can be represented by intent expectation components.

Figure 6.2.1.1 Example of how ServiceProfile will be represented by which intent expectation components

Following subclauses will give the detailed solution for how to model service profile as intent expectation and how to model slice subnet profiles for RAN and CN as intent expectations.

### 6.2.2	Alternative 1: Network slice and network slice subnet expectations

#### 6.2.2.1	Modelling service profile attributes as intent expectation for network slice

##### 6.2.2.1.1	ExpectationObject

Following are the specific allowed values when implemented the IntentExpectation for Network Slice Expectation.

Table 6.2.2.1.2.1-1

| Attribute | Allowed Values |
| --- | --- |
| ObjectType (CM) | NetworkSlice |
| objectInstance (CM) | DN of the NetworkSlice |

Following attributes in ServiceProfile can be modelled as ObjectContexts

TBD

##### 6.2.2.1.2	ExpectationTargets

Following attributes in ServiceProfile can be modelled as ExpectationTargets

TBD

##### 6.2.2.1.3	ExpectationContext

Following attributes in ServiceProfile can be modelled as ExpectationContext:

TBD

#### 6.2.2.2	Modelling RANSliceProfile attributes as intent expectation for RAN network slice subnet service delivering and assurance

The RadioServiceExpectation described in TR 28.912 [7] can be reused for expectations for RAN network slice subnet service delivering and assurance with some potential extensions. The detailed definition for RadioServiceExpectation see clause 5.1.2 in TS 28.912 [7]. The RAN network slice subnet service can be represented by the RadioService object in clause 5.1.2 in TS 28.912 [7].

#### 6.2.2.3	Modelling CNSliceProfile attributes as intent expectation for CN network slice subnet

TBD

### 6.2.3	Alternative 2: Communication service and RAN/CN service expectations

##### 6.2.3.1	Description

In this alternative an MnS producer can handle the following:

- provision a communication service and assure its performance when consumer conveys a CommunicationService expectation;

- provision a RadioService and assure its performance when consumer conveys a RadioService expectation;

- provision a CNService and assure its performance when consumer conveys a CNService expectation.

The RadioServiceExpectation described in TR 28.912 [7] can be reused for expectations for RAN network slice subnet service delivering and assurance with some potential extensions. The detailed definition for RadioServiceExpectation see clause 5.1.2 in TS 28.912 [7]. The RAN network slice subnet service can be represented by the RadioService object in clause 5.1.2 in TS 28.912 [7].

##### 6.2.3.2	ExpectationObjects

Following are the specific allowed values when implementing the IntentExpectation for CommunicationService, RadioService or CNService Expectation.

Table 6.2.2.1.2.1-1 Expectation object allowed values

| Attribute | Allowed Values |
| --- | --- |
| ObjectType (CM) | CommunicationService, RadioService, CNService |
| objectInstance (CM) | DN of the CommunicationService, RadioService, CNService |

Following attributes in ServiceProfile can be used as input for modelling ObjectContexts, these attributes represent the functional requirements for the communication service, radio service or core network service.

| Attributes in ServiceProfile | Object Context |
| --- | --- |
| coverageArea | contextAttribute: "coverageArea" contextCondition: "IS_WITHIN_RANGE" contextValueRange: coverageArea defined in clause 6.4.1 of 3GPP TS 28.541 [2]. |
| radioSpectrum | contextAttribute: "radioSpectrum" contextCondition: "IS_WITHIN_RANGE" contextValueRange: radioSpectrum defined in clause 6.3.31 of 3GPP TS 28.541 [2]. |

##### 6.2.3.3	ExpectationTargets

Following attributes in ServiceProfile can be modelled as ExpectationTargets, these attributes represent the capacity and performance requirements for the communication service, radio service or core network service.

| Attributes in ServiceProfile | Expectation Target |
| --- | --- |
| dLLatency | targetName: "dLLatency" targetCondition: "IS_LESS_THAN " targetValueRange: integer |
| uLLatency | targetName: "uLLatency" targetCondition: "IS_LESS_THAN" targetValueRange: integer |
| dLThptPerUE | targetName: "dLThptPerUE" targetCondition: "IS_WITHIN_RANGE" targetValueRange: A pair of integers to represent the guaranteed data rate and the maximum data rate. |
| uLThptPerUE | targetName: "uLThptPerUE" targetCondition: "IS_WITHIN_RANGE" targetValueRange: A pair of integers to represent the guaranteed data rate and the maximum data rate. |

Editor’s Note: Whether any of the remaining attributes (e.g., maxNumberofUEs, activityFactor, maxNumberofPDUSessions) in ServiceProfile can be  used as input for modelling IntentExpectation is FFS

##### 6.2.3.4	ExpectationContexts

Following attributes in ServiceProfile can be modelled as ExpectationContexts, these attributes used to represent additional requirements for communication service (e.g., operational requirements), radio service or core network service.

| Attributes in ServiceProfile | ExpectationContext |
| --- | --- |

##### 6.2.3.5	Extensions

A vendor may map additional extensions in their implementation.

### 6.2.4	Alternative 3: Communication service expectation

##### 6.2.4.1	Description

In this alternative an MnS producer can handle the following:

- provision a communication service and assure its performance when consumer conveys a CommunicationService expectation;

##### 6.2.4.2	ExpectationObjects

Following are the specific allowed values when implementing the IntentExpectation for CommunicationService, Expectation.

Table 6.2.2.1.2.1-1 Expectation object allowed values

| Attribute | Allowed Values |
| --- | --- |
| ObjectType (CM) | CommunicationService |
| objectInstance (CM) | DN of the CommunicationService |

Following attributes in ServiceProfile can be used as input for modelling ObjectContexts, these attributes represent the functional requirements for the CommunicationService.

| Attributes in ServiceProfile | Object Context |
| --- | --- |
| coverageArea | contextAttribute: "coverageArea" contextCondition: "IS_WITHIN_RANGE" contextValueRange: coverageArea defined in clause 6.4.1 of 3GPP TS 28.541 [2]. |
| radioSpectrum | contextAttribute: "radioSpectrum" contextCondition: "IS_WITHIN_RANGE" contextValueRange: radioSpectrum defined in clause 6.3.31 of 3GPP TS 28.541 [2]. |

##### 6.2.4.3	ExpectationTargets

Following attributes in ServiceProfile can be modelled as ExpectationTargets, these attributes represent the capacity and performance requirements for the communication service.

| Attributes in ServiceProfile | Expectation Target |
| --- | --- |
| dLLatency | targetName: "dLLatency" targetCondition: "IS_LESS_THAN " targetValueRange: integer |
| uLLatency | targetName: "uLLatency" targetCondition: "IS_LESS_THAN" targetValueRange: integer |
| dLThptPerUE | targetName: "dLThptPerUE" targetCondition: "IS_WITHIN_RANGE" targetValueRange: A pair of integers to represent the guaranteed data rate and the maximum data rate. |
| uLThptPerUE | targetName: "uLThptPerUE" targetCondition: "IS_WITHIN_RANGE" targetValueRange: A pair of integers to represent the guaranteed data rate and the maximum data rate. |

Editor’s Note: Whether any of the remaining attributes (e.g., maxNumberofUEs, activityFactor, maxNumberofPDUSessions) in ServiceProfile can be  used as input for modelling IntentExpectation is FFS

##### 6.2.4.4	ExpectationContexts

Following attributes in ServiceProfile can be modelled as ExpectationContexts, these attributes used to represent additional requirements for communication service (e.g., operational requirements).

| Attributes in ServiceProfile | ExpectationContext |
| --- | --- |

##### 6.2.4.5	Extensions

A vendor may map additional extensions in their implementation

### 6.2.5	Alternative 4: Network slice expectation

##### 6.2.5.1	Description

In this alternative the network slice expectation is modelled based on the requirements which are the same as those modelled in a ServiceProfile. The expectation object in this alternative can be a TopSliceSubnet or a RANSliceSubnet or a CNSliceSubnet. In this alternative an MnS producer can handle the following:

- provision a TopSliceSubnet and assure its performance when consumer conveys a TopSliceSubnet expectation;

- provision a RANSliceSubnet and assure its performance when consumer conveys the RANSliceSubnet subset of a NetworkSlice expectation;

- provision a CNSliceSubnet and assure its performance when consumer conveys the CNSliceSubnet subset of a NetworkSlice expectation.

##### 6.2.5.2	ExpectationObjects

Following are the specific allowed values when implementing the IntentExpectation for NetworkSlice.

Table 6.2.2.1.2.1-1 Expectation object allowed values

| Attribute | Allowed Values |
| --- | --- |
| ObjectType (CM) | NetworkSlice |
| objectInstance (CM) | DN of the NetworkSlice |

Following attributes in ServiceProfile can be used as input for modelling ObjectContexts, these attributes represent the functional requirements for the network slice.

| Attributes in ServiceProfile | Object Context |
| --- | --- |
| coverageArea | contextAttribute: "coverageArea" contextCondition: "IS_WITHIN_RANGE" contextValueRange: coverageArea defined in clause 6.4.1 of 3GPP TS 28.541 [2]. |
| radioSpectrum | contextAttribute: "radioSpectrum" contextCondition: "IS_WITHIN_RANGE" contextValueRange: radioSpectrum defined in clause 6.3.31 of 3GPP TS 28.541 [2]. |

##### 6.2.5.3	ExpectationTargets

Following attributes in ServiceProfile can be modelled as ExpectationTargets, these attributes represent the capacity and performance requirements for the networkslice.

| Attributes in ServiceProfile | Expectation Target |
| --- | --- |
| dLLatency | targetName: "dLLatency" targetCondition: "IS_LESS_THAN " targetValueRange: integer |
| uLLatency | targetName: "uLLatency" targetCondition: "IS_LESS_THAN" targetValueRange: integer |
| dLThptPerUE | targetName: "dLThptPerUE" targetCondition: "IS_WITHIN_RANGE" targetValueRange: A pair of integers to represent the guaranteed data rate and the maximum data rate. |
| uLThptPerUE | targetName: "uLThptPerUE" targetCondition: "IS_WITHIN_RANGE" targetValueRange: A pair of integers to represent the guaranteed data rate and the maximum data rate. |

Editor’s Note: Whether any of the remaining attributes (e.g., maxNumberofUEs, activityFactor, maxNumberofPDUSessions) in ServiceProfile can be  used as input for modelling IntentExpectation is FFS

##### 6.2.5.4	ExpectationContexts

Following attributes in ServiceProfile can be modelled as ExpectationContexts, these attributes used to represent additional requirements for network slice (e.g., operational requirements).

| Attributes in ServiceProfile | ExpectationContext |
| --- | --- |

##### 6.2.5.5	Extensions

A vendor may map additional extensions in their implementation.

### 6.2.6	Evaluation of alternatives

#### 6.2.6.1	Overview of alternatives

The alternatives described in 6.2.2/3/4/5 are evaluated based on what information elements must be specified or enhanced for a standard based solution.

Table 6.2.6-1 Overview of identified new information needed for each alternative

| Alt. | Expectation on | To be defined in TS 28.312 | Examples of objects that may be allocated as result of an expectation (TS 28.541) |
| --- | --- | --- | --- |
| 1 | NetworkSlice RANSliceSubnet CNSliceSubnet | NetworkSliceExpectation RANSliceSubnetExpectation CNSliceSubnetExpectation ObjectType=NetworkSlice or NetworkSliceSubnet | NetworkSliceSubnet <<IOC>> with TopSliceSubnetProfile NetworkSliceSubnet <<IOC>> with RANSliceSubnetProfile NetworkSliceSubnet <<IOC>> with CNSliceSubnetProfile |
| 2 | CommunicationService RadioService CNService | CommunicationServiceExpectation RadioServiceExpectation CNServiceExpectation ObjectType=CommunicationService or RadioService or CNService | NetworkSliceSubnet <<IOC>> with TopSliceSubnetProfile NetworkSliceSubnet <<IOC>> with RANSliceSubnetProfile NetworkSliceSubnet <<IOC>> with CNSliceSubnetProfile |
| 3 | CommunicationService | CommunicationServiceExpectation ObjectType= CommunicationService | NetworkSliceSubnet <<IOC>> with TopSliceSubnetProfile NetworkSliceSubnet <<IOC>> with RANSliceSubnetProfile NetworkSliceSubnet <<IOC>> with CNSliceSubnetProfile |
| 4 | NetworkSlice | NetworkSliceExpectation ObjectType=NetworkSlice | NetworkSliceSubnet <<IOC>> with TopSliceSubnetProfile NetworkSliceSubnet <<IOC>> with RANSliceSubnetProfile NetworkSliceSubnet <<IOC>> with CNSliceSubnetProfile |
| NOTE 1: Information imported from TS 28.541 is specified and only needs to be referenced to from TS 28.312. NOTE 2: Expectation on Radio Service is described in [7] similarly, a consumer may have expectations on 5GC services which for this report is called CNService. |  |  |  |

#### 6.2.6.2	Evaluation

Alternative 1: Requires specification of three new dataTypes; NetworkSliceExpectation, RadioServiceExpectation and CNServiceExpectation. The mapping from ServiceProfile to the three NetworkSliceExpectations is straightforward and can be done in normative work.

Alternative 2: Requires specification three new dataTypes CommunicationServiceExpectation, RadioServiceExpectation and CNServiceExpectation. The generic study for enhancements to IDMS [7] also identified that the RadioServiceExpectation and CNServiceExpectation are not defined. The mapping from ServiceProfile to the three ServiceExpectations is straightforward and can be done in normative work.

Alternative 3: Requires specification of new dataType for CommunicationServiceExpectation. The CommunicationService is a good abstraction level for a network slicing solution as it expresses what the network slice is used for. The MnS producer logic determines what objects may be allocated to fulfil the expectation. The mapping from ServiceProfile to the CommunicationServiceExpectation is straightforward and can be done in normative work.

Alternative 4: Requires specification of new dataType for NetworkSliceExpectation. The abstraction level for network slice expectation is limited to network slicing while communication service provides the next level of abstraction and may be more generic and better understood by an MnS consumer.  The mapping from ServiceProfile to NetworkSliceExpectation is straightforward and can be done in normative work.

### 6.2.7	Examples

This clause gives some examples of the alternatives described in 6.2.2/3/4/5 using the intent object model defined in figure 6.2.4-1 which can be found in TS 28.312, see reference [1].

Figure 6.2.7-1 Intent object model, TS 28.312 [1]

Following the modelling used for Intent expectation a few examples are shown using the object classes and attributes  that have been defined for network slicing.

In all the given examples the name of the expectation is defined by the user, there is no standardized way of identifying what the intent is about or how it relates to the actual network without additional documentation.

#### Alternative 1, example

Operator specifies an intent expectation and names it “NetworkSliceExpectation”. An instance of the dataType IntentExpectation could look as follows:

| NetworkSliceExpectation | user assigned name |
| --- | --- |
| networkSlice.coverageAreaTAList networkSlice.nROperatingBands networkSlice.plmnIdInfoList networkSlice.sST | expectation object with object contexts |
| dLLatency.condition.range uLLatency.condition.range dLThptPerUE.condition.range uLThptPerUE.condition.range | expectation target with target contexts |
| maxNumberofUEs maxDLDataVolume maxULDataVolume maxNumberofPDUSessions dLMaxPktSize uLMaxPktSize | expectation contexts (in this example the dimensioning information is the expectation context) |

Operator specifies an intent expectation and names it “RANSliceSubnetExpectation”. An instance of the dataType IntentExpectation could look as follows:

| SliceSubnetExpectation | user assigned name |
| --- | --- |
| networkSliceSubnet.coverageAreaTAList networkSliceSubnet.nROperatingBands networkSliceSubnet.plmnInfoList | expectation object with object contexts |
| dLLatency.condition.range uLLatency.condition.range dLThptPerUE.condition.range uLThptPerUE.condition.range | expectation target with target contexts |
| maxNumberofUEs dLMaxPktSize uLMaxPktSize | expectation contexts (in this example the dimensioning information is the expectation context) |

#### Alternative 2 example

Operator specifies an intent expectation and names it “CommunicationServiceExpectation”. An instance of the dataType IntentExpectation could look as follows:

| CommunicationServiceExpectation | user assigned name |
| --- | --- |
| communicationService.coverageAreaTAList communicationService.nROperatingBands communicationService.plmnIdInfoList communicationService.sST | expectation object with object contexts |
| dLLatency.condition.range uLLatency.condition.range dLThptPerUE.condition.range uLThptPerUE.condition.range | expectation target with target contexts |
| maxNumberofUEs maxDLDataVolume maxULDataVolume maxNumberofPDUSessions dLMaxPktSize uLMaxPktSize | expectation contexts (in this example the dimensioning information is the expectation context) |

Operator specifies an intent expectation and names it “RadioServiceExpectation”. An instance of the dataType IntentExpectation could look as follows:

| RadioServiceExpectation | user assigned name |
| --- | --- |
| radioService.coverageAreaTAList radioService.nROperatingBands radioService.plmnInfoList | expectation object with object contexts |
| dLLatency.condition.range uLLatency.condition.range dLThptPerUE.condition.range uLThptPerUE.condition.range | expectation target with target contexts |
| maxNumberofUEs dLMaxPktSize uLMaxPktSize | expectation contexts (in this example the dimensioning information is the expectation context) |

Operator specifies an intent expectation and names it “CNServiceExpectation”. An instance of the dataType IntentExpectation could look as follows:

| CNServiceExpectation | user assigned name |
| --- | --- |
| cNService.coverageAreaTAList cNService.plmnInfoList | expectation object with object contexts |
| dLLatency.condition.range uLLatency.condition.range dLThptPerUE.condition.range uLThptPerUE.condition.range | expectation target with target contexts |
| maxNumberofUEs maxNumberofPDUSessions dLMaxPktSize uLMaxPktSize | expectation contexts (in this example the dimensioning information is the expectation context) |

#### Alternative 3 example

Operator specifies an intent expectation and names it “CommunicationServiceExpectation”. An instance of the dataType IntentExpectation could look as follows:

| CommunicationServiceExpectation | user assigned name |
| --- | --- |
| communicationService.coverageAreaTAList communicationService.nROperatingBands communicationService.plmnIdInfoList communicationService.sST | expectation object with object contexts |
| dLLatency.condition.range uLLatency.condition.range dLThptPerUE.condition.range uLThptPerUE.condition.range | expectation target with target contexts |
| maxNumberofUEs maxDLDataVolume maxULDataVolume maxNumberofPDUSessions dLMaxPktSize uLMaxPktSize | expectation contexts (in this example the dimensioning information is the expectation context) |

#### Alternative 4, example

Operator specifies an intent expectation and names it “NetworkSliceExpectation”. An instance of the dataType IntentExpectation could look as follows:

| NetworkSliceExpectation | user assigned name |
| --- | --- |
| networkSlice.coverageAreaTAList networkSlice.radioSpectrum networkSlice.plmnInfoList | expectation object with object contexts |
| dLLatency.condition.range uLLatency.condition.range dLThptPerUE.condition.range uLThptPerUE.condition.range | expectation target with target contexts |
| maxNumberofUEs maxDLDataVolume maxULDataVolume maxNumberofPDUSessions dLMaxPktSize uLMaxPktSize | expectation contexts (in this example the dimensioning information is the expectation context) |

End of changes.