3GPP TSG-SA5 Meeting #149 	S5-234790

Berlin, Germany, 22 - 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0179 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Align figure with description in Procedure of NF instance creation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The procedure in clause 7.10 refers to createMOI operations, but the figure 7.10-1 shows CreateNf operation which is undefined. The figure 7.10-1 is not aligned with the description. In the procedure in clause 7.10, it is described that NFMS_C sends createMOI request in step 1), and NFMS_P sends response in step 7). createMOI response does not contain “identifier of NFMS_P which actually maintains the MOI for ManagedFunction.” As described in step 7). Hence this needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update figure 7.10-1 to use createMOI to align with the description. Update step 7) to indicate that the NFMS_P sends the createMOI response to NFMS_C. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification leads to confusion and incorrect implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - S5-234042 is merged into this CR |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - S5-233904 is revised to S5-234790 |  |  |  |  |  |  |  |  |

1st Change

## 7.10	Procedure of NF instance creation

The Figure 7.10-1 illustrates the procedure of creating a new network function instance to satisfy the required network function related requirements.

Figure 7.10-1: Network Function Instance Creation procedure

1)	Network Function Management Service Provider (NFMS_P) receives a createMOI, for ManagedFunction IOC, request from Network Function Management Service Consumer (NFMS_C) with network function related requirements.

Note:	The network function related requirements see information model definition for NR NRM in clause 4 and information model definition for 5GC NRM in clause 5 in TS 28.541[6].

2)	If NF instance to be created contains virtualized part, NFMS_P derives the requirements for VNF instance based on the network function related requirements.

3)	If corresponding VNF Package needs to be on-boarded or changed, the NFMS_P invoke corresponding VNF Package management procedure as described in clause 4.3 in TS 28.526 [7].

4)	The NFMS_P invokes VNF lifecycle management with requirements for VNF instance as descried in clause 4.2.2.2 in TS 28.526 [7].

5)	The NFMS_P creates the MOI for the ManagedFunction to be created. If the ManagedFunction contains virtualized part, the NFMS_P may send the request of creating the MOI to the NFMS_P in the ManagedFunction.

6)	The NFMS_P configures the new created MOI with corresponding configuration information (see information model definition for NR NRM in clause 4 and information model definition for 5GC NRM in clause 5 in TS 28.541[6]).

7) The NFMS_P sends the createMOI response to NFMS_C with DN of MOI for ManagedFunction.

End of change