# 3GPP TSG-SA5 Meeting #150	S5-235711

# Goteborg, Sweden, August 21-25, 2023

# Source:	Nokia

# Title:	Rel-18 pCR 28.318 clause 3 Definitions and Acronyms

# Document for:	Approval

Agenda Item:	6.6.4.2 (NSOEU_WoP#1) General

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

None specific to this pCR.

# 3	Rationale

The term Energy Supply ID must be well understood for proper use in the TS.

# 4	Detailed proposal

It is proposed to agree to the following change to TS 28.318, 0.0.0.

Begin Change

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

Energy Supply ID: A unique identity that is assigned by the DSO to every point where electric energy supply is provided to an energy consumer. Each physical component of a 5G network may be associated with one Energy Supply ID. Every MOI in a MIB that collectively represents the physical component may be associated with that same Energy Supply ID.

Multiple physical components of the 5G network may share the same Energy Supply ID (e.g. co-located components such as CU, DU and RU may have the same Energy Supply ID) although it cannot be assumed that different physical components share the same Energy Supply ID (a specific physical implementation cannot be mandated). The site operator has authority to associate an Energy Supply ID with a specific physical 5G network component and by extension this Energy Supply ID could optionally be associated with every MOI that collectively represents the physical component.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ESI	Energy Supply ID

Next Change