3GPP TSG-SA5 Meeting #151	S5-236349

# Xiamen, China, 9th Oct 2023 - 13th Oct 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0285 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS28.532 Rel18 correction to eventTime description for NotifyMoiDeletion & NotifyMoiAttributeValueChanges |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, REST_SS |  |  |  |  |  | Date: |  |  | 2023-09-27 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The descriptions for both eventTime for are " Event (MOI creation) occurrence time", which are not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification leads to incorrect implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.1.4.1a.6, 12.1.1.4.1a.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Mirror of S5-236347 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Modified Section

###### 12.1.1.4.1a.6	Type NotifyMoiDeletion

Table 12.1.1.4.1a.6 -1: Definition of type NotifyMoiDeletion

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type ("notifyMOIDeletion") | M |
| eventTime | DateTime | Event (MOI deletion) occurrence time | M |
| systemDN | SystemDN | System DN | M |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| sourceIndicator | SourceIndicator | Indicates the source of the operation that led to the generation of this notification. | O |
| attributeList | AttributeNameValuePairSet | Attributes (name/value pairs) of the deleted MOI. | O |

The following example demonstrates the deletion of an object. The message body includes the name/value pairs representing the attributes of the deleted object.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.com Content-Type: application/json { "href": "http://example.com/ManagedElement=ME1/ClassA=CA1", "notificationId": 123456789, "notificationType": "notifyMOIDeletion", "eventTime": "Tue, 06 Aug 2019 16:50:26 GMT", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1" }

The message body may include the name/value pairs representing the attributes of the deleted object.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.com Content-Type: application/json { "href": "http://example.com/ManagedElement=ME1/ClassA=CA1", "notificationId": 123456789, "notificationType": "notifyMOIDeletion", "eventTime": "Tue, 06 Aug 2019 16:50:26 GMT", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "attributeList": { "attrA": "valueAttrA", "attrB": "valueAttrB" } }

###### 12.1.1.4.1a.7	Type NotifyMoiAttributeValueChanges

Table 12.1.1.4.1a.7 -1: Definition of type NotifyMoiAttributeValueChanges

| Attribute name | Data type | Description | S |
| --- | --- | --- | --- |
| href | Uri | URI of the resource where the event (alarm) occurred | M |
| notificationId | NotificationId | Notification identifier as defined in ITU-T Rec. X. 733 [4] | M |
| notificationType | NotificationType | Notification type ("notifyMOIAttributeValueChanges") | M |
| eventTime | DateTime | Event (MOI attribute value changes) occurrence time | M |
| systemDN | SystemDN | System DN | M |
| correlatedNotifications | array(CorrelatedNotification) | Set of all notifications to which this notification is considered to be correlated as defined in ITU-T Rec. X. 733 [4] | O |
| additionalText | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] | O |
| sourceIndicator | SourceIndicator | Indicates the source of the operation that led to the generation of this notification. | O |
| attributeListValueChanges | AttributeValueChangeSet | List with names of changed attributes, together with new value and optionally old value | M |

The following example notification reports the modification of the attribute values for "attrA" and "attrB".

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.org Content-Type: application/json { "href": "http://example.com/ManagedElement=ME1/ClassA=CA1", "notificationId": 123456789, "notificationType": "notifyMOIAttributeValueChanges", "eventTime": "Tue, 06 Aug 2019 16:50:26 GMT", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "attributeListValueChanges": [ { "attrA": "newValueAttrA", "attrB": "newValueAttrB" } ] }

In addition to the new values, the old values may be included in the notification.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.org Content-Type: application/json { "href": "http://example.com/ManagedElement=ME1/ClassA=CA1", "notificationId": 123456789, "notificationType": "notifyMOIAttributeValueChanges", "eventTime": "Tue, 06 Aug 2019 16:50:26 GMT", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "attributeListValueChanges": [ { "attrA": "newValueAttrA", "attrB": "newValueAttrB" }, { "attrA": "oldValueAttrA", "attrB": "oldValueAttrB" } ] }

In the example above the attribute values are of simple type. For attributes of structured type, the question is if all attribute fields of an attribute, where at least one attribute field changed its value, need to be reported or only those attribute fields that changed value. Assume an attribute field that did not change value is not included in the notification. In this case the MnS consumer receiving the notification cannot tell if the attribute field was deleted or if the attribute field did not change value. It is not possible to distinguish these two cases based on the information in the notification. For that reason, always all attribute fields of an attribute need to be included in the notification. For structured attributes with many attribute fields this may not be very efficient.

In the next example "attrA" is a structured attribute with the attribute fields "attrFieldAA" and "attrFieldAB". The attribute field "attrFieldAA" changed value, the attribute field "attrFieldAB" did not change value. The attribute value change notification reporting this change may look as follows.

POST /3gpp-management/cm-notification-sink HTTP/1.1 Host: example.org Content-Type: application/json { "href": "http://example.com/ManagedElement=ME1/ClassA=CA1", "notificationId": 123456789, "notificationType": "notifyMOIAttributeValueChanges", "eventTime": "Tue, 06 Aug 2019 16:50:26 GMT", "systemDN":"DC=example.com,ManagedElement=ME1,MnsAgent=MA1", "attributeListValueChanges": [ { "attrA": { "attrFieldAA": "newValueAttrFieldAA", "attrFieldAB": "oldValueAttrFieldAB" } }, { "attrA": { "attrFieldAA": "oldValueAttrFieldAA", "attrFieldAB": "oldValueAttrFieldAB" } } ] }

Note also that for multi-valued attributes all attribute elements of the new value need to be reported. It is not possible to report only added, deleted, or modified attribute elements. Furthermore, the notification does not allow reporting of deleted attributes. When this is required the MnS producer needs to include always all attributes of the object in the notification. It is outside the present document how the MnS producer signals to the MnS consumer if all attributes or only the changed ones are included in the attribute value change notification. The notification itself does not include this information.

End of Modified Section