3GPP TSG-SA5 Meeting #151 	S5-236418

Xiamen, China, 9 - 13 October 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.310 | CR | 0037 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 28.310 Update on energy saving for UPFs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | EE5GPLUS |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use case of switching off edge UPFs during off-peak traffic hours is omitted unintentionally in the clause of energy saving concepts and general clause of energy saving use cases while other energy saving use cases are reflected in those. It is recommended to fix this issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to reflect the use case of switching off edge UPFs during off-peak traffic hours in the clause of energy saving concepts and general clause of energy saving use cases to give a better general view for SA5 energy saving work. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Use case of switching off edge UPFs during off-peak traffic hours missing in the clause of energy saving concepts and general clause of energy saving use cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.2, 5.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 4.3.2	Concepts

Two energy saving states can be conceptually identified for cells, NEs and NFs.

Conceptually, a cell or a network element or network function may be on one of these two states with respect to energy saving:

-	notEnergySaving state

-	energySaving state

Based on the above energy saving states, a full energy saving solution includes two elementary procedures:

-	Energy saving activation (change from notEnergySaving state to energySaving state)

-	Energy saving deactivation (change from energySaving state to notEnergySaving state)

When a cell is in energy saving state, it may need candidate cells to pick up the load. However, a cell in energySaving state should not cause coverage holes or create undue load on the surrounding cells. All traffic on that cell is expected to be drained to other overlaid/umbrella candidate cells before the cell moves to energySaving state.

Similarly, when a network element or network function is in energy saving state it may need candidate network elements or network functions to pick up the load. For example, during off-peak traffic periods, one or more edge UPFs in energySaving state should not cause undue load on the other UPFs. All traffic on those edge UPFs is expected to be redirected from those edge UPFs to other UPFs before those edge UPFs move into energySaving state.

A cell in energySaving state is not considered as a cell outage or a fault condition. No alarms should be raised for any condition that is a consequence of a subject cell or network element or network function moving into energySaving state.

Next change

### 5.1.3	Energy saving use cases

#### 5.1.3.1	General

The objective of energy saving is to lower OPEX for mobile operators, through the reduction of power consumption in the mobile networks that is becoming more urgent and challenging, as there are much more network elements in NR (e.g., small cells with massive MIMO in higher frequency bands) than those used in LTE (TR 37.816 [14], TS 38.300 [13]). One typical scenario of energy saving is to switch off capacity boosters when the traffic demand is low, and re-activated them on a need basis (see clause 5.6 in TR 37.816 [14]).

For NG-RAN, the energy saving consists of two scenarios where the capacity booster cell - gNB is fully or partially overlaid by the candidate cell(s). For 5GC, the energy saving consists of scenarios where 5GC NFs may be switched off during off-peak traffic time, e.g. some UPFs deployed at the edge of core network may be switched off during off-peak traffic time.

End of changes