3GPP TSG-SA5 Meeting #151	S5-236430

# Xiamen, China, 9-13 October 2023

# Source:	Huawei

# Title:	pCR 28.865 Update solultion of service assurance for video monitoring

# Document for:	Approval

Agenda Item:	6.9.2.4

1	Decision/action requested

The group is asked to discuss and approve the proposal.

2	References

[1]	SP-211442: "New SID on deterministic communication service assurance"

[2]	S5-234729: "draft TR 28.865 Study on deterministic communication service assurance"; v0.5.0

3	Rationale

This tdoc addresses the solution update of service assurance for video monitoring.

It is proposed to add some abnormal issue analysis output for service assurance of video monitoring in draft TR 28.865. It is helpful for the MnS consumer to identify problems and find means to solve them.

4	Detailed proposal

This document proposes the following changes in TR 28.865.

1st Change

5	Issues and potential solutions

5.Y	Issue #2: Service assurance for video monitoring

5.Y.1	Description

5.Y.2	Potential solutions

5.Y.2.a	Potential solution #1: Video monitoring service

5.Y.2.a.1	Introduction

In the deterministic communication service requirement analysis phase, network requirements (data rate, delay, and reliability) are analyzed. Network deployment requirements of different areas need to be considered for network preparation. Deterministic communication services have high SLS requirements. For example, the planning of video monitoring services focus on uplink coverage, uplink capacity, uplink data rate, and E2E latency etc. It demands on high delay stability of video frame transmission. The network requirements need to be derived based on the application characteristics of specific services. For example, network preparation information, such as network capacity, coverage, reliability, and data rate estimation, needs to be output based on the three-layer service requirement model of the video monitoring service.

5.Y.2.a.2	Description

1. Data collection

Collect service experience data and network performance data of video monitoring services based on the three-layer service requirement modeling. For example, the following data may collected: latency and throughput related performance data defined in TS 28.552 and TS 28.554. Abnormal information such as large round-trip delay, large jitter, low data rate distribution, high packet loss rate, and service quality deterioration etc.

Taking the video monitoring services in a large port as an example, it requires high uplink bandwidth, low latency, high availability, mobility, and high reliability. Network preparation is performed to meet these requirements, e.g., deployment and provisioning of the necessary network functions.

2. Analytics and demarcation

Service and network analysis of deterministic communication services includes monitoring, poor-QoE identification, demarcation, and root cause analysis. Identify abnormal issues, determine the impact scope, and restore services.

Monitoring and the poor-QoE identification requires two steps:

- Builds the three-layer service requirement model based on service characteristics, including poor experience information.

- Collects experience, service quality, and network-related indicators and abnormal incidents to identify poor-QoE users and services.

Fault demarcation is used to generate service interruption/ poor experience incident when a service anomaly occurs. The E2E network KPI, alarm information, terminal exceptions are correlated and analzed. Demarcation results for video monitoring services are provided accordingly.

For deterministic communication services, the stability and deadline of latency are defined at data packet level from end to end perspective according to TS 22.104. The abnormal issues should be identified and means should be taken to solve the problems. To support the assurance of video monitoring services, some abnormal issue analysis outputs are as in the following table:

| Attribute Name | Description |
| --- | --- |
| Communicationcation service interruption | The reception failure of an uplink data packet upon the deadline of the survival time. |
| Excessive RTT | The times that the RTT is greater than a threshold exceed a predefined value within a detection period. |
| High packet loss rate | The uplink packet loss rate is greater than a threshold within a detection period. |
| Abnormal data rate | The uplink traffic is greater than a traffic threshold and the number of consecutive occurrence periods reaches the threshold within a detection period. |

Some performance measurements are currently available in TS 28.552, e.g. packet drop rate, average packet delay, IP latency measurements etc, which may be used as the enabling data for the above analysis.

3. Optimization and verification

Optimization:

For video monitoring services, delay-related optimization and uplink data rate optimization are considered. To optimize delay-related problems, it is needed to analyze the average delay, delay jitter, and delay reliability. It can analyzed the delay-related factors of the air interface, such as the uplink and downlink slot assignment proportion, air interface resource capacity, and scheduling policy.

Uplink data rate optimization: The coverage, interference, resource allocation policy, and scheduling policy are optimized to make the uplink rate better match the SLS requirement.

Verification:

According to the service assurance objective of deterministic communication, that is, the quality of experience indicator of each service meets the deterministic requirement after optimization, it is further verified that the network capability meets the deterministic requirement. Select indicators that reflect video monitoring service experience, such as stalling and interruption, or indicators that reflect network performance, such as packet loss rate, rate, and delay.

5.Y.3	Conclusion - Impact on normative work

Editor's Note:	This clause provides the conclusion from the aspect of impact on normative work for issue#2.

End of change