3GPP TSG-SA5 Meeting #151 	S5-236508

# Xiamen, China, 9 Oct 2023 – 13 Oct 2023

# Source:	Amdocs, Nokia, Nokia Shanghai Bell

# Title:	Update CHF Selection By Charging Domains

# Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

Include the proposed changes in TR 28.840

# 2	References

[1]		3GPP TR 28.840: "Study on CHF Segmentation"

# Rationale

This pCR is to update the study topic for CHF selection by Charging Domain and bring it in alignment with the TR structure for other topics under consideration.

# 4	Detailed proposal

First change

## 5.4	Topic #4 CHF Discovery by Charging Domains

### 5.4.1 	General description and assumptions

This topic is for investigating how to support the discovery of a Charging Function segregated by one or more charging domains / subsystem / service. For example, there can be separate CHF NFs deployed:

- 5G connection and mobility domain charging

- IP Multimedia System (IMS) domain charging

- Edge Computing domain charging

- Proximity-based Services (ProSe) charging

- Exposure function Northbound Application Program Interfaces (APIs) charging.

Sometimes a CHF NF instance could support a set of related domains: e.g.

- IMS and MMTel domains

- Network Slice Management, Network Slice Admission Control, and Network Slice Performance and Analytics domains.

Therefore, it should be possible to discover and select a CHF instance based on the charging domain(s) / subsystem(s) / service(s) that the charging is required for.

### 5.4.2	Potential charging requirements

The following are potential high-level charging requirements, derived from the requirements in TS 32.255 [4]

REQ-CH_ SELCHF_CD -01: The 5GS should be able to provide a CHF instance based on the functional domain, subsystem or service that requires charging.

### 5.4.3 	Key Issues

The following key issues are identified:

- Key Issue #4a: Definition of charging domain(s), subsystem(s) or service(s) for distributed charging deployments

- Key Issue #4b: Determination of methods and procedures by which a CHF service producer can indicate the supported charging domain(s) / subsystem(s) or service(s)

- Key Issue #4c: Determination of methods and procedures by which a CHF service consumer can indicate the charging domain(s) / subsystem(s) or service(s) for which it requests charging

- Key Issue #4d: Determination of methods and procedures by which a CHF service consumer can discover a CHF service producer providing charging service for one or more charging domain(s), subsystem(s) or service(s)

### 5.4.4	Possible Solutions

### 5.4.5	Evaluation

### 5.4.6 	Conclusion

End of changes