3GPP TSG-SA5 Meeting #151	S5-236550

# Xiamen, China, 9th Oct 2023 - 13th Oct 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0275 | rev | 1 | Current version: | 16.16.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR 28.532 Clarify Heartbeat MnS capability definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The definition of Heartbeat in 28.532 is problematic as its inclusion in section 11 (“Management services – stage 2”) could be misread to mean that Heartbeat is its own MnS. Heartbeat is instead a capability implemented by other MnS and the TS should be clearer on that. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update definitions of Heartbeat to more clearly state that it is implemented by other MnS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Potential confusion about this could cause implementation and mutli-vendor interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.0 (NEW),11.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The new section 11.0 is intentionally numbered to position it before the existing 11.x clauses. This is to provide a common location for further generic capabilities to be defined. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 11	Management services – Stage 2

## 11.0	Generic management service capabilities

### 11.0.1	Introduction

This clause provides the definitions of generic capabilities which may be implemented by a management service to provide a generic capability in a consistent manner.  The definitions in this clause are not distinct MnS and are implemented as part of another MnS.

2nd Change

## 11.0.x		Heartbeat Capability

#### 11.0.x.1	Introduction

### This clause provides the stage 2 definition to enable any MnS Producer to support the emission of periodic heartbeat notifications.  This can be used by any MnS optting to provide heartbeat support between its MnS Producer and its MnS Consumer(s).

### 11.0.x.2	Operations and notifications

#### 11.0.x.2.1	Notification notifyHeartbeat

##### 11.0.x.2.1	Definition

This notification notifies the subscribed consumer(s) that the MnS producer heartbeat period has expired or that a MnS consumer requested the emission of an immediate heartbeat notification.

The emission of heartbeat notifications is controlled by the HeartbeatControl IOC (3GPP TS 28.622 [11]).

##### 11.0.x.2.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | HeartbeatControl.objectClass |  |
| objectInstance | M | HeartbeatControl.objectInstance | Instance controlling the emission of this notifyHeartbeat notification. |
| notificationId | M | -- |  |
| notificationType | M | "notifyHeartbeat" |  |
| eventTime | M | -- | Time at which the notification is emitted. The semantics of Generalised Time specified by RFC 3339 [50] shall be used here. |
| systemDN | M | MnSAgent.objectInstance | Identifies the MnS which emitted the heartbeat notification. |
| heartbeatNtfPeriod | M | HeartbeatControl.heartbeatNtfPeriod |  |

##### 11.0.x.2.3	Triggering event

###### 11.0.x.2.3.1	From-state

stateBeforeHeartbeatNotification1 OR stateBeforeHeartbeatNotification2.

| Assertion Name | Definition |
| --- | --- |
| stateBeforeHeartbeatNotification1 | The internal countdown timer of the MOI emitting the notifyHeartbeat notification has reached the value ‘0’ (zero). |
| stateBeforeHeartbeatNotification2 | The value of the attribute triggerHeartbeatNtf of the MOI emitting the notifyHeartbeat notification is TRUE. |

###### 11.0.x.2.0.2	To-state

stateAfterOHeartbeatNotification1 OR stateAfterOHeartbeatNotification2.

| Assertion Name | Definition |
| --- | --- |
| stateAfterHeartbeatNotification1 | If From-state is stateBeforeHeartbeatNotification1 then: the internal countdown timer of the MOI is reset to the value of its heartbeatNtfPeriod attribute. |
| stateAfterHeartbeatNotification2 | If From-state is stateBeforeHeartbeatNotification2 then: the value of the internal countdown timer of the MOI is not affected. |

3rd Change

## 11.4	VOID