3GPP TSG-SA5 Meeting #151	S5-236559

# Xiamen, China, 9th Oct 2023 - 13th Oct 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0290 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 28.532 Correct URI definition inconsistencies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 32.158 defines URIs as values based on MnS info however 28.532 contradicts with fixed values. This could cause inconsistency between the URI in the MnS Registry, MnS Info and the actual URI for the MnS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update 28.532 to be consistent with 32.158 and use variables in the URI construction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The inconsistency could cause confusion and interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.1.1.1, 12.1.1.3, 12.2.1.3, 12.5.1.3,12.6.1.3, A.1.1, A.2.1, A.6.2, A.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/813 at commit e50b3a34a452053cb0c91a62d541349232d668f5 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 12.1.1	RESTful HTTP-based solution set

#### 12.1.1.1	Mapping of operations

##### 12.1.1.1.1	Introduction

The IS operations are mapped to SS equivalents according to table 12.1.1.1.1-1.

Table 12.1.1.1.1-1: Mapping of IS operations to SS equivalents

| IS operation | HTTP Method | Resource URI | S |
| --- | --- | --- | --- |
| createMOI | PUT | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{className}={id} | M |
| getMOIAttributes | GET | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{className}={id} | M |
| modifyMOIAttributes | PUT PATCH | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{className}={id} | M |
| deleteMOI | DELETE | {MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}/{className}={id} | M |

2nd Change

#### 12.1.1.3	Resources

##### 12.1.1.3.1	Resource structure

12.1.1.3.1.1	Resource structure on the MnS producer

Figure 12.1.1.3.1.1-1 shows the resource structure of the Provisioning MnS on the MnS producer.

{MnSRoot}/{MnsType}/{MnSLabel}/{MnSVersion}/{URI-LDN-first-part}

/{

className

}={id}

Figure 12.1.1.3.1.1-1: Resource URI structure of the Provisioning MnS on the MnS producer

Table 12.1.1.3.1.1-1 provides an overview of the resources and applicable HTTP methods.

Table 12.1.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| MOI | …/{className}={id} | PUT | Create a resource representing a managed object instance |
| MOI | …/{className}={id} | GET | Retrieve one or multiple resources representing managed object instances |
| MOI | …/{className}={id} | PATCH | Modifiy one or multiple resources representing managed object instances |
| MOI | …/{className}={id} | DELETE | Delete one or multiple resources representing managed object instances |

###### 12.1.1.3.1.2	Resource structure on the MnS consumer

Figure 12.1.1.3.1.2-1 shows the resource structure of the Provisioning MnS on the MnS consumer.

Figure 12.1.1.3.1.2-1: Resource URI structure of the Provisioning MnS on the MnS consumer

Table 12.1.1.3.1.2-1 provides an overview of the resources and applicable HTTP methods.

Table 12.1.1.3.1.2-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Notification Target | {notificationTarget} | POST | Send a notification to the notification target |

##### 12.1.1.3.2	Resource definitions

###### 12.1.1.3.2.1	Resource "…/{className}={id}"

12.1.1.3.2.1.1	Description

This resource represents a managed object instance.

12.1.1.3.2.1.2	URI

Resource URI: {MnSRoot}/{MnSType}/{MnSLabel}/{MnSVersion}/{URI-LDN-first-part}/{className}={id}

The resource URI variables are defined in table 12.1.1.3.2.1.2-1.

Table 12.1.1.3.2.1.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.2 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.2 of TS 32.158 [15] |
| URI-LDN-first-part | See clause 4.4.2 of TS 32.158 [15] |
| className | Class name of the targeted resource |
| id | Identifier of the targeted resource |

12.1.1.3.2.1.3	HTTP methods

12.1.1.3.2.1.3.1	HTTP PUT

This method shall support the URI query parameters specified in the following table.

Table 12.1.1.3.2.1.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 12.1.1.3.2.1.3.1-2: Data structures supported by the PUT request body on this resource

| Data type | Description | S |
| --- | --- | --- |
| Resource | Resource representation of the resource to be created or replaced | M |

Table 12.1.1.3.2.1.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| Resource | 200 OK | Status code returned when the resource is replaced, and when the replaced resource representation is not identical to the resource representation in the request. This status code may be retourned when the resource is updated and when the updated resource representation is identical to the resource representation in the request. The representation of the updated resource is returned in the response message body. | M |
| Resource | 201 Created | Status code returned when the resource is created. The representation of the created resource is returned in the response message body. | M |
| n/a | 204 No Content | Status code that may be returned only when the replaced resource representation is identical to the representation in the request. The response has no message body. | M |
| ErrorResponse | 4xx/5xx | Returned in case of an error | O |

12.1.1.3.2.1.3.2	HTTP GET

This method shall support the URI query parameters specified in the following table.

Table 12.1.1.3.2.1.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| scope | Scope style: form explode: true | Extends the set of targeted resources beyond the base resource identified with the authority and path component of the URI. | O |
| filter | Filter | Reduces the targeted set of resources by applying a filter to the scoped set of resource representations. Only resources representations for which the filter construct evaluates to "true" are targeted. | O |
| attributes | array(string) style: form explode: false | Attributes of the scoped resources to be returned. The value is a comma-separated list of attribute names. | O |
| fields | array(string) style: form explode: false | Attribute fields of the scoped resources to be returned. The value is a comma-separated list of JSON pointers to the attribute fields. | O |

This method shall support the request data structures, the response data structures and response codes specified in the following tables.

Table 12.1.1.3.2.1.3.2-2: Data structures supported by the GET request body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.1.1.3.2.1.3.2-3: Data structures supported by the GET response body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| Resource | 200 OK | Resources identified in the request for retrieval. In case the attributes or fields query parameters are used, only the selected attributes or sub-attributes are returned. The response message body is constructed according to the hierarchical response construction method (TS 32.158 [15]) | M |
| ErrorResponse | 4xx/5xx | Returned in case of an error | M |

12.1.1.3.2.1.3.3	HTTP PATCH

This method shall support the URI query parameters specified in the following table.

Table 12.1.1.3.2.1.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures, the response data structures and response codes specified in the following tables.

Table 12.1.1.3.2.1.3.3-2: Data structures supported by the PATCH request body on this resource

| Data type | Description | S |
| --- | --- | --- |
| Resource, or array(object) | Patch document describing the set of modifications to be applied to the targeted resources. The following patch media types are available: - "application/merge-patch+json" (RFC 7396 [37]) - "application/3gpp-merge-patch+json" (TS 32.158 [15]) - "application/json-patch+json" (RFC 6902 [36]) - "application/3gpp-json-patch+json" (TS 32.158 [15]) | M |

Table 12.1.1.2.1.1.3.3-3: Data structures supported by the PATCH response body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| ErrorResponse | 4xx/5xx | Returned in case of an error | M |

12.1.1.3.2.1.3.4	HTTP DELETE

This method shall support the URI query parameters specified in the following table.

Table 12.1.1.3.2.1.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| scope | Scope style: form explode: true | Extends the set of targeted resources beyond the base resource identified with the authority and path component of the URI. | O |
| filter | Filter | Reduces the targeted set of resources by applying a filter to the scoped set of resource representations. Only resources representations for which the filter construct evaluates to "true" are targeted. | O |

This method shall support the request data structures, the response data structures and response codes specified in the following tables.

Table 12.1.1.3.2.1.3.4-2: Data structures supported by the DELETE request body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.1.1.3.2.1.3.4-3: Data structures supported by the DELETE response body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| array(Uri) | 200 OK | Status code returned, when query parameters are present in the request and one or multiple resources are deleted. The URIs of the deleted resources are returned in the response message body. | M |
| n/a | 204 No Content | Status code returned, when no query parameters are present in the request and only one resource is deleted. The message body is empty. | M |
| ErrorResponse | 4xx/5xx | Returned in case of an error | M |

3rd Change

## 12.2	Generic fault supervision management service

### 12.2.1	RESTful HTTP-based solution set

#### 12.2.1.3	Resources

##### 12.2.1.3.1	Resource structure

12.2.1.3.1.1	Resource structure on the MnS producer

Figure 12.2.1.3.1.1-1 shows the resource structure of the Fault Supervision MnS on the MnS producer.

/{

alarmId

}

/alarms

/comments

/subscriptions

/{

commentId

}

{MnSRoot}/{MnSType}{MnSLabel}/{MnSVersion}/

/{

subscriptionId

}

/

alarmCount

Figure 12.2.1.3.1-1: Resource URI structure of the Fault Supervision MnS on the MnS producer

Table 12.2.1.3.1.1-1 provides an overview of the resources and applicable HTTP methods.

Table 12.2.1.3.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Alarms | .../alarms | GET | Retrieve all alarms or a filtered subset |
|  |  | PATCH | Clear, acknowledge or unacknowledge multiple alarms |
| Alarm Count | .../alarms/alarmCount | GET | Retrieve the alarm count per perceived severity |
| Alarm | .../alarms/{alarmId} | PATCH | Clear, acknowledge or unacknowledge an alarm |
| Comments | .../alarms/{alarmId}/comments | POST | Add a comment to an alarm |
| Subscriptions | .../subscriptions | POST | Create a subscription |
| Subscription | .../subscriptions/{subscriptionId} | DELETE | Delete a subscription |

###### 12.2.1.3.1.2	Resource structure on the MnS consumer

Figure 12.2.1.3.1.2-1 shows the resource structure of the Fault Supervision MnS on the MnS consumer.

Figure 12.2.1.3.1.2-1: Resource URI structure of the Fault Supervision MnS on the MnS consumer

Table 12.2.1.3.1.2-1 provides an overview of the resources and applicable HTTP methods.

Table 12.2.1.3.1.2-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Notification Target | {notificationTarget} | POST | Send a notification to the notification target |

##### 12.2.1.3.2	Resource definitions

###### 12.2.1.3.2.1	Resource "…/alarms"

12.2.1.3.2.1.1	Description

This resource represents a collection of alarms.

12.2.1.3.2.1.2	URI

Resource URI: {MnSRoot}/{MnSType}/{MnSLabel}/{MnSVersion}/alarms

The resource URI variables are defined in table 12.2.1.3.2.1.2-1.

Table 12.2.1.3.2.1.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.3 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.3 of TS 32.158 [15] |

12.2.1.3.2.1.3	HTTP methods

12.2.1.3.2.1.3.1	HTTP GET

This method shall support the URI query parameters specified in the following table.

Table 12.2.1.3.2.1.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| alarmAckState | AlarmAckState |  | O |
| href | Dn |  | O |
| filter | string |  | O |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 12.2.1.3.2.1.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.2.1.3.2.1.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| GetAlarmsResponse | 200 OK | The alarms returned. | M |
| ErrorResponse | 4xx/5xx | Returned in case of an error | O |

12.2.1.3.2.1.3.2	Void

12.2.1.3.2.1.3.3	HTTP PATCH

This method shall support the URI query parameters specified in the following table.

Table 12.2.1.3.2.1.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 12.2.1.3.2.1.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| map(MergePatchAcknowledgeAlarm) | Patch document for acknowledging one or multiple alarms | M |
| map(MergePatchClearAlarm) | Patch document for clearing one or multiple alarms | M |

Table 12.2.1.3.2.1.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success the response body shall be empty. | M |
| ErrorResponse | 4xx/5xx | In case of failure, the response body shall be described by "ErrorResponse". | M |

###### 12.2.1.3.2.2	Resource "…/alarms /{alarmId}"

12.2.1.3.2.2.1	Description

This resource represents an alarm.

12.2.1.3.2.2.2	URI

Resource URI: {MnSRoot}/{MnSType}/{MnSLabel}/{MnSVersion}/alarms/{alarmId}

The resource URI variables are defined in table 12.2.1.3.2.2.2-1.

Table 12.2.1.3.2.2.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.3 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.3 of TS 32.158 [15] |
| alarmId | Alarm identifier |

12.2.1.3.2.2.3	HTTP methods

12.2.1.3.2.2.3.1	HTTP PATCH

This method shall support the URI query parameters specified in the following table.

Table 12.2.1.3.2.2.3.1-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 12.2.1.3.2.2.3.1-2: Data structures supported by the PATCH Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| MergePatchAcknowledgeAlarm | Patch document for acknowledging an alarm | M |
| MergePatchClearAlarm | Patch document for clearing an alarm | M |

Table 12.2.1.3.2.2.3.1-3: Data structures supported by the PATCH Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| n/a | 200 OK | In case of success the response body shall be empty. |  |
| ErrorResponse | 4xx/5xx | In case of failure, the response body shall carry a JSON object described by "ErrorResponse". |  |

###### 12.2.1.3.2.3	Resource "…/alarms/alarmCount"

12.2.1.3.2.3.1	Definition

This resource holds metadata about the /alarms collection resource like the alarm count per perceived severity.

12.2.1.3.2.3.2	URI

Resource URI: {MnSRoot}/{MnSType}/{MnSLabel}/{MnSVersion}/alarms/alarmCount

The resource URI variables are defined in table 12.2.1.3.2.3.2-1.

Table 12.2.1.3.2.3.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.3 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.3 of TS 32.158 [15] |

12.2.1.3.2.3.3	HTTP methods

12.2.1.3.2.3.3.1	GET

This method shall support the URI query parameters specified in table 12.2.1.3.2.3.3.1-1.

Table 12.2.1.3.2.3.3.3-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| alarmAckState | AlarmAckState | Allows to control which alarms are counted based on acknowledgement state | O |
| filter | string | Allows to control which alarms are counted based on a general filter applied to the alarm records. | O |

This method shall support the request data structures specified in table 12.2.1.3.2.3.3.1-2 and the response data structures and response codes specified in table 12.2.1.3.2.3.3.1-3.

Table 12.2.1.3.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.2.1.3.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| AlarmsCount | 200 OK | The alarm count per severity level returned. | M |
| ErrorResponse | 4xx/5xx | Returned in case of an error | O |

###### 12.2.1.3.2.4	Resource "…/alarms/{alarmId}/comments"

12.2.1.3.2.4.1	Definition

This resource is a collection resource for comments attached to an alarm.

12.2.1.3.2.4.2	URI

Resource URI: {MnSRoot}/{MnSType}/{MnSLabel}/{MnSVersion}/alarms/{alarmId}/comments

The resource URI variables are defined in table 12.2.1.3.2.4.2-1.

Table 12.2.1.3.2.4.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.3 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.3 of TS 32.158 [15] |
| alarmId | Alarm identifier |

12.2.1.3.2.4.3	HTTP methods

12.2.1.3.2.4.3.1	POST

This method shall support the URI query parameters specified in the following table.

Table 12.2.1.3.2.4.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures, and the response data structures and response codes specified in the following tables.

Table 12.2.1.3.2.4.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| Comment | The representation of the comment to be added to an alarm. | M |

Table 12.2.1.3.2.4.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| Comment | 201 Created | In case of success, the response body shall carry the representation of a comment. The "commentTime" shall be set by the MnS producer. | M |
| ErrorResponse | 4xx/5xx | In case of failure, the response body shall be described by "ErrorResponse". | M |

###### 12.2.1.3.2.5	Resource "…/comments/{commentId}"

12.2.1.3.2.5.1	Definition

This resource represents a comment attached to an alarm.

12.2.1.3.2.5.2	URI

Resource URI: {MnSRoot}/{MnSType}/{MnSLabel}/{MnSVersion}/alarms/{alarmId}/comments/{commentId}

The resource URI variables are defined in table 12.2.1.3.2.4.5-1.

Table 12.2.1.3.2.4.5-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.3 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.3 of TS 32.158 [15] |
| alarmId | Alarm identifier |
| commentId | Comment identifier |

12.2.1.3.2.5.3	HTTP methods

None.

###### 12.2.1.3.2.6	Resource "…/subscriptions"

12.2.1.3.2.6.1	Description

This resource is a container resource for individual subscriptions.

12.2.1.3.2.6.2	URI

Resource URI: {MnSRoot}/FaultSupervisionMnS/{MnSVersion}/subscriptions

The resource URI variables are defined in table 12.2.1.3.2.6.2-1.

Table 12.2.1.3.2.6.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.3 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.3 of TS 32.158 [15] |

12.2.1.3.2.6.3	HTTP methods

12.2.1.3.2.6.3.1	POST

This method shall support the URI query parameters specified in table 12.2.1.3.2.6.3.1-1.

Table 12.2.1.3.2.6.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures specified in table 12.2.1.3.2.6.3.1-2 and the response data structures and response codes specified in table 12.2.1.3.2.6.3.1-3.

Table 12.2.1.3.2.6.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| Subscription | Details of the subscription to be created | M |

Table 12.2.1.3.2.6.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| Subscription | 201 Created | In case of success the representation of the created subscription is returned. | M |
| ErrorResponse | 4xx/5xx | In case of failure the error object is returned. | M |

12.2.1.3.2.6.3.2	Void

###### 12.2.1.3.2.7	Resource "…/subscriptions/{subscriptionId}"

12.2.1.3.2.7.1	Description

This resource represents a subscription.

12.2.1.3.2.7.2	URI

Resource URI: {MnSRoot}/{MnSType}/{MnSLabel}/{MnSVersion}/ subscriptions/{subscriptionId}

The resource URI variables are defined in table 12.2.1.3.2.7.2-1.

Table 12.2.1.3.2.7.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.3 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.3 of TS 32.158 [15] |
| subscriptionId | Subscription identifier |

12.2.1.3.2.7.3	HTTP methods

12.2.1.3.2.7.3.1	DELETE

This method shall support the URI query parameters specified in table 12.2.1.3.2.7.3.1-1.

Table 12.2.1.3.2.7.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures specified in table 12.2.1.3.2.7.3.1-2 and the response data structures and response codes specified in table 12.2.1.3.2.7.3.1-3.

Table 12.2.1.3.2.7.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.2.1.3.2.7.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success no message body is returned | M |
| ErrorResponse | 4xx/5xx | In case of failure the error object is returned. | M |

###### 12.2.1.3.2.8	Resource "{notificationTarget}"

12.2.1.3.2.8.1	Description

This resource represents a notification target on the MnS consumer.

12.2.1.3.2.8.2	URI

Resource URI: {notificationTarget}

The resource URI variables are defined in table 12.2.1.3.2.8.2-1.

Table 12.2.1.3.2.8.2-1: URI variables

| Name | Definition |
| --- | --- |
| notificationTarget | URI of the notification target on the MnS consumer, contained in the notification subscription |

12.2.1.3.2.8.3	HTTP methods

12.2.1.3.2.8.3.1	POST

This method shall support the URI query parameters specified in table 12.2.1.3.2.8.3.1-1.

Table 12.2.1.3.2.8.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures specified in table 12.2.1.3.2.8.3.1-2 and the response data structures and response codes specified in table 12.2.1.3.2.8.3.1-3.

Table 12.2.1.3.2.8.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| NotifyNewAlarm | Type for a notifyNewAlarm notification (non-security alarm) | M |
| NotifyNewSecAlarm | Type for a notifyNewAlarm notification (security alarm) | M |
| NotifyAckStateChanged | Type for a notifyAckStateChanged notification | M |
| NotifyClearedAlarm | Type for a notifyClearedAlarm notification | M |
| NotifyAlarmListRebuilt | Type for a notifyAlarmListRebuilt notification | M |
| NotifyChangedAlarm | Type for a notifyChangedAlarm notification | M |
| NotifyComments | Type for a notifyComments notification | M |
| NotifyPotentialFaultyAlarmList | Type for a notifyPotentialFaultyAlarmList notification | M |
| NotifyCorrelatedNotificationChanged | Type for a notifyCorrelatedNotificationChanged notification | M |
| NotifyChangedAlarmGeneral | Type for a notifyChangedAlarmGeneral notification (non-security alarm) | M |
| NotifyChangedSecAlarmGeneral | Type for a notifyChangedAlarmGeneral notification (security alarm) | M |

Table 12.2.1.3.2.8.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success no message body is returned | M |
| ErrorResponse | 4xx/5xx | In case of failure the error object is returned. | M |

4th Change

## 12.5	Streaming data reporting service

### 12.5.1	RESTful HTTP-based solution set

#### 12.5.1.2	Mapping of notifications

Not applicable (no notifications defined in IS).

#### 12.5.1.3	Resources

##### 12.5.1.3.2	Resources definitions

12.5.1.3.2.1	Resource "…/connections"

12.5.1.3.2.1.1	Description

This resource represents a collection of connections and can be used to establish new connections or to obtain information about existing connections.

12.5.1.3.2.1.2	URI

The resource URI is: {MnSRroot}/StreamingDataReportingMnS/{MnSVersion}/connections

This resource shall support the resource URI variables defined in the table 12.5.1.3.2.1.2-1.

Table 12.5.1.3.2.1.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.3 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.3 of TS 32.158 [15] |

12.5.1.3.2.1.3	HTTP methods

12.5.1.3.2.1.3.1	HTTP POST

This method shall support the URI query parameters specified in the following table.

Table 12.5.1.3.2.1.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| none supported |  |  |  |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 12.5.1.3.2.1.3.1-2: Data structures supported by the POST request body on this resource

| Data type | Description | S |
| --- | --- | --- |
| producerId | String representing the DN of the streaming data reporting MnS producer. | M |
| array(streamInfo-Type) | List of meta-data about each reporting stream. Where each reporting stream is represented by a streamInfo. |  |

Table 12.5.1.3.2.1.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |
| uri-Type | 201 Posted | Connection identifier assigned by the MnS consumer | M |

12.5.1.3.2.1.3.2	HTTP GET

This method shall support the URI query parameters specified in the following table.

Table 12.5.1.3.2.1.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| connectionIdList | array(uri-Type) | The list of connectionId for which the connection information is to be returned. | O |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 12.5.1.3.2.1.3.2-2: Data structures supported by the GET request body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.5.1.3.2.1.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |
| array(uri-Type, streamReporter-Type, streamIdList-Type) | 200 OK | In case of success the representation of the retrieved information is returned. | M |
|  | 202 Partially retrieved | In case of partial success the representation of the retrieved information is returned. | M |

12.5.1.3.2.2	Resource "…/connections/{connectionId}"

12.5.1.3.2.2.1	Description

This resource represents an individual connection and can be used for an "upgrade" to WebSocket as part of the connection establishment, or to obtain information about an existing connection, or to terminate an existing connection, or to send a unit of streaming data.

12.5.1.3.2.2.2	URI

The resource URI is: {MnSRoot}/StreamingDataReportingMnS/{MnSVersion}/connections/{connectionId}

This resource shall support the resource URI variables defined in the table 12.5.1.3.2.2.2-1.

Table 12.5.1.3.2.2.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.3 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.3 of TS 32.158 [15] |
| connectionId | Represents identifier of an individual connection assigned by the MnS consumer during connection establishment |

12.5.1.3.2.2.3	HTTP methods

12.5.1.3.2.2.3.1	HTTP GET (Upgrade)

This method shall support the URI header parameters specified in the following table.

Table 12.5.1.3.2.2.3.2-1: Header parameters supported by the GET request on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| connectionId | uri-Type | To indicate the ID (URI) of the connection being upgraded to WebSocket | M |
| Upgrade | Upgrade-HeaderType | To indicate the HTTP GET operation is to upgrade the connection to WebSocket protocol | M |
| Connection | Connection-HeaderType | To indicate the HTTP GET operation is to upgrade the connection to another protocol | M |
| Sec-WebSocket-Key | Sec-WebSocket-Key-HeaderType | The Sec-WebSocket-Key needed for establishing the WebSocket connection. | M |
| Sec-WebSocket-Version | Sec-WebSocket-Version-HeaderType | The Sec-WebSocket-Version needed for establishing the WebSocket connection. | M |

This method shall support the URI query parameters specified in the following table.

Table 12.5.1.3.2.2.3.2-2: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| none supported |  |  |  |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 12.5.1.3.2.2.3.2-3: Data structures supported by the GET request body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.5.1.3.2.2.3.2-4: Header parameters supported by the GET response on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| Upgrade | Upgrade-HeaderType | To indicate the HTTP GET operation is to upgrade the connection to WebSocket protocol | M |
| Connection | Connection-HeaderType | To indicate the HTTP GET operation is to upgrade the connection to another protocol | M |
| Sec-WebSocket-Accept | Sec-WebSocket-Accept-HeaderType | The Sec-WebSocket-Accept responded when establishing the WebSocket connection. | M |

Table 12.5.1.3.2.2.3.2-5: Data structures supported by the GET response body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| n/a | 101 Switching Protocols | The status code indicating the connection has been successfully upgraded to WebSocket. | M |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |

12.5.1.3.2.2.3.2	HTTP GET

This method shall support the URI query parameters specified in the following table.

Table 12.5.1.3.2.1.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| none supported |  |  |  |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 12.5.1.3.2.1.3.2-2: Data structures supported by the GET request body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.5.1.3.2.1.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |
| uri-Type | 200 OK | In case of success the representation of the connectionId is returned. | M |
| streamReporter-Type | 200 OK | In case of success the representation of the streamReporter is returned. | M |
| streamIdList-Type | 200 OK | In case of success the representation of the streamIdList is returned. | M |

12.5.1.3.2.3	Resource "…/connections/{connectionId}/streams"

12.5.1.3.2.3.1	Description

This resource represents a collection of reporting streams on a particular connection and can be used to add a new reporting stream to an existing connection, or to remove a reporting stream from an existing connection, or to obtain information about reporting streams.

12.5.1.3.2.3.2	URI

The resource URI is: {MnSRoot}/StreamingDataReportingMnS/{MnSVersion}/connections/{connectionId}/streams

This resource shall support the resource URI variables defined in the table 12.5.1.3.2.3.2-1.

Table 12.5.1.3.2.3.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.3 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.3 of TS 32.158 [15] |
| connectionId | See table 12.5.1.3.2.2.2-1 |

12.5.1.3.2.3.3	HTTP methods

12.5.1.3.2.3.3.1	HTTP POST

This method shall support the URI query parameters specified in the following table.

Table 12.5.1.3.2.3.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| none supported |  |  |  |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 12.5.1.3.2.3.3.1-2: Data structures supported by the POST request body on this resource

| Data type | Description | S |
| --- | --- | --- |
| array(streamInfo-Type) | The resource representation of the set of information about streams to be posted. | M |

Table 12.5.1.3.2.3.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| array(streamInfo-Type) | 201 Posted | In case of success the representation of the posted information about streams is returned. | M |
|  | 202 Partially posted | In case of partial success the representation of the posted information about streams is returned. | M |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |

12.5.1.3.2.3.3.2	HTTP DELETE

This method shall support the URI query parameters specified in the following table.

Table 12.5.1.3.2.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| streamIdList | array(streamId-Type) | The list of streamId for the stream(s) to be deleted. | M |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 12.5.1.3.2.3.3.2: Data structures supported by the DELETE request body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.5.1.3.2.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success no message body is returned | M |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |

12.5.1.3.2.3.3.3	HTTP GET

This method shall support the URI query parameters specified in the following table.

Table 12.5.1.3.2.3.3.3-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| streamIdList | array(streamId-Type) | The list of streamId for which the stream information are to be returned. | O |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 12.5.1.3.2.3.3.3-2: Data structures supported by the GET request body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.5.1.3.2.3.3.3-3: Data structures supported by the GET Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| array(streamInfo-Type, streamReporters-Type) | 200 OK | In case of success the representation of the retrieved stream information is returned. | M |
|  | 202 Partially retrieved | In case of partial success the representation of the retrieved stream information is returned. | M |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |

12.5.1.3.2.4	Resource "…/connections/{connectionId}/streams/{streamId}"

12.5.1.3.2.4.1	Description

This resource represents an individual reporting stream on an existing connection and can be used to obtain information about reporting stream.

12.5.1.3.2.4.2	URI

The resource URI is: {MnSRoot}/StreamingDataReportingMnS/{MnSVersion}/connections/{connectionId}/streams/{streamId}

This resource shall support the resource URI variables defined in the table 12.5.1.3.2.4.2-1.

Table 12.5.1.3.2.4.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.3 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.3 of TS 32.158 [15] |
| connectionId | See table 12.5.1.3.2.2.2-1 |
| streamId | Represents identifier of an individual stream. For Streaming Trace reporting, the Trace Reference (see clause 5.6 of TS 32.422 [38]) is used as stream identifier |

12.5.1.3.2.4.3	HTTP methods

12.5.1.3.2.4.3.1	HTTP GET

This method shall support the URI query parameters specified in the following table.

Table 12.5.1.3.2.4.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| none supported |  |  |  |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 12.5.1.3.2.4.3.1-2: Data structures supported by the GET request body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.5.1.3.2.4.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| streamInfo-Type | 200 OK | In case of success the representation of the retrieved stream information is returned. | M |
| streamReporters-Type | 200 OK | In case of success the representation of the retrieved stream reporters information is returned. | M |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |

5th Change

## 12.6	File data reporting service

### 12.6.1	RESTful HTTP-based solution set

#### 12.6.1.3	Resources

##### 12.6.1.3.1	Resource structure

###### 12.6.1.3.1.1	Resource structure on the MnS producer

Figure 12.6.1.3.1.1-1 shows the resource structure of the File Data Reporting MnS on the MnS producer.

/files

/subscriptions

{

MnSRoot}/{MnSType}/{MnSLabel}/{MnSVersion}

/{

subscriptionId

}

Figure 12.6.1.3.1.1-1: Resource URI structure of the File Data Reporting MnS on the MnS producer

Table 12.2.1.3.1.1-1 provides an overview of the resources and applicable HTTP methods.

Table 12.2.1.3.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Files | …/files | GET | Retrieve the information of the available files |
| Subscriptions | …/subscriptions | POST | Create a subscription |
| Subscription | …/subscriptions/{subscriptionId} | DELETE | Delete a single subscription |
| Notification Target | {notificationTarget} | POST | Send a notification to the notification target |

###### 12.6.1.3.1.2	Resource structure on the MnS consumer

Figure 12.6.1.3.1.2-1 shows the resource structure of the File Data Reporting MnS on the MnS consumer.

Figure 12.6.1.3.1.2-1: Resource URI structure of the File Data Reporting MnS on the MnS consumer

Table 12.6.1.3.1.2-1 provides an overview of the resources and applicable HTTP methods.

Table 12.6.1.3.1.2-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Notification Target | {notificationTarget} | POST | Send a notification to the notification target |

##### 12.6.1.3.2	Resource definitions

12.6.1.3.2.1	Resource "…/files"

12.6.1.3.2.1.1	Description

This resource represents the information about a collection of available files.

12.6.1.3.2.1.2	URI

Resource URI = {MnSRoot}/FileDataReportingMnS/{MnSVersion}/files

The resource URI variables are defined in table 12.6.1.3.2.1.1-1.

Table 12.6.1.3.2.1.1-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.3 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.3 of TS 32.158 [15] |

12.6.1.3.2.1.3	HTTP methods

12.6.1.3.2.1.3.1	HTTP GET

This method shall support the URI query parameters specified in the following table.

Table 12.6.1.3.2.1.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| fileDataType | FileDataType | Selects files based on the file data type. | M |
| beginTime | DateTime | Selects files based on the earliest time they became available | M |
| endTime | DateTime | Selects files based on the latest time they became available | M |

This method shall support the request data structures, the response data structures and response codes specified in the following tables.

Table 12.6.1.3.2.1.3.1-2: Data structures supported by the GET request body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.6.1.3.2.1.3.1-3: Data structures supported by the GET response body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| array(FileInfo) | 200 OK | Information about the files identified in the request | M |
| ErrorResponse | 4xx/5xx | Returned in case of an error | M |

12.6.1.3.2.2	Resource "…/subscriptions"

12.6.1.3.2.2.1	Description

This resource is a container resource for individual subscriptions.

12.6.1.3.2.2.2	URI

Resource URI: {MnSRoot}/FileDataReportingMnS/{MnSVersion}/subscriptions

The resource URI variables are defined in table 12.6.1.3.3.2.2.2-1:

Table 12.6.1.3.3.2.2.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.3 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.3 of TS 32.158 [15] |

12.6.1.3.2.2.3	HTTP methods

12.6.1.3.2.2.3.1	POST

This method shall support the URI query parameters specified in table 12.6.1.3.2.2.3.1-1.

Table 12.6.1.3.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures specified in table 12.6.1.3.2.2.3.1-2 and the response data structures and response codes specified in table 12.6.1.3.2.2.3.1-3.

Table 12.6.1.3.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| Subscription | Details of the subscription to be created | M |

Table 12.6.1.3.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| Subscription | 201 Created | In case of success the representation of the created subscription is returned. | M |
| ErrorResponse | 4xx/5xx | In case of failure the error object is returned. | M |

12.6.1.3.2.2.3.2	Void

12.6.1.3.2.3	Resource ".../subscriptions/{subscriptionId}"

12.6.1.3.2.3.1	Description

This resource represents a subscription.

12.6.1.3.2.3.2	URI

Resource URI: {MnSRoot}/FileDataReportingMnS/{MnSVersion}/subscriptions/{subscriptionId}

The resource URI variables are defined in table 12.6.1.3.2.3.2-1.

Table 12.6.1.3.2.3.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.3 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.3 of TS 32.158 [15] |
| subscriptionId | Subscription identifier |

12.6.1.3.2.3.3	HTTP methods

12.6.1.3.2.3.3.1	DELETE

This method shall support the URI query parameters specified in table 12.6.1.3.2.3.3-1.

Table 12.6.1.3.2.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures specified in table 12.6.1.3.2.3.3-2 and the response data structures and response codes specified in table 12.6.1.3.2.3.3-3.

Table 12.6.1.3.2.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.6.1.3.2.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success no message body is returned | M |
| ErrorResponse | 4xx/5xx | In case of failure the error object is returned. | M |

12.6.1.3.2.4	Resource "/notificationTarget"

12.6.1.3.2.4.1	Description

This resource represents a notification target on the MnS consumer.

12.6.1.3.2.4.2	URI

Resource URI: {notificationTarget}

The resource URI variables are defined in table 12.6.1.3.2.4.2-1.

Table 12.6.1.3.2.4.2-1: URI variables

| Name | Definition |
| --- | --- |
| notificationTarget | URI of the notification target on the MnS consumer, contained in the notification subscription |

12.6.1.3.2.4.3	HTTP methods

12.6.1.3.2.4.3.1	POST

This method shall support the URI query parameters specified in table 12.6.1.3.2.4.3.1-1.

Table 12.6.1.3.2.4.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures specified in table 12.6.1.3.2.4.3.1-2 and the response data structures and response codes specified in table 12.6.1.3.2.4.3.1-3.

Table 12.6.1.3.2.4.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| NotifyFileReady | Type in case a notifyFileReady notification is sent | M |
| NotifyFilePreparationError | Type in case a notifyFilePreparationError notification is sent | M |

Table 12.6.1.3.2.4.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success no message body is returned | M |
| ErrorResponse | 4xx/5xx | In case of failure the error object is returned. | M |

6th Change

## A.1.1	OpenAPI document "TS28532_ProvMnS.yaml"

<CODE BEGINS>

openapi: 3.0.1

info:

title: Provisioning MnS

version: 17.5.0

description: >-

OAS 3.0.1 definition of the Provisioning MnS

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.532; Generic management services

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.532/

servers:

- url: '{MnSRoot}/ProvMnS/{MnSLabel}/{MnSVersion}/{URI-LDN-first-part}'

variables:

MnSRoot:

description: See clause 4.4.2 of TS 32.158

default: http://example.com/3GPPManagement

MnSLabel:

description: Label of the MnS

default: 3GPP

MnSVersion:

description: Version number of the OpenAPI definition

default: XXX

URI-LDN-first-part:

description: See clause 4.4.2 of TS 32.158

default: ''

paths:

'/{className}={id}':

parameters:

- name: className

in: path

required: true

schema:

type: string

- name: id

in: path

required: true

schema:

type: string

put:

summary: Replaces a complete single resource or creates it if it does not exist

description: >-

With HTTP PUT a complete resource is replaced or created if it does not

exist. The target resource is identified by the target URI.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

responses:

'200':

description: >-

Success case ("200 OK").

This status code shall be returned when the resource is replaced, and

when the replaced resource representation is not identical to the resource

representation in the request.

This status code may be retourned when the resource is updated and when the

updated resource representation is identical to the resource representation

in the request.

The representation of the updated resource is returned in the response

message body.

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

'201':

description: >-

Success case ("201 Created").

This status code shall be returned when the resource is created.

The representation of the created resource is returned in the response

message body.

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

'204':

description: >-

Success case ("204 No Content").

This status code may be returned only when the replaced resource

representation is identical to the representation in the request.

The response has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

callbacks:

notifyMOICreation:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyMoiCreation'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyMOIDeletion:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyMoiDeletion'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyMOIAttributeValueChanges:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyMoiAttributeValueChanges'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyMOIChanges:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyMoiChanges'

application/yang-data+json:

schema:

$ref: '#/components/schemas/NotifyMoiChanges'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

get:

summary: Reads one or multiple resources

description: >-

With HTTP GET resources are read. The resources to be retrieved are

identified with the target URI. The attributes and fields parameter

of the query components allow to select the resource properties to be returned.

parameters:

- name: scope

in: query

description: >-

This parameter extends the set of targeted resources beyond the base

resource identified with the path component of the URI. No scoping

mechanism is specified in the present document.

required: false

schema:

$ref: '#/components/schemas/Scope'

style: form

explode: true

- name: filter

in: query

description: >-

This parameter reduces the targeted set of resources by applying a

filter to the scoped set of resource representations. Only resource

representations for which the filter construct evaluates to "true"

are targeted. No filter language is specified in the present

document.

required: false

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Filter'

- name: attributes

in: query

description: >-

This parameter specifies the attributes of the scoped resources that

are returned.

required: true

schema:

type: array

items:

type: string

style: form

explode: false

- name: fields

in: query

description: >-

This parameter specifies the attribute field of the scoped resources

that are returned.

required: false

schema:

type: array

items:

type: string

style: form

explode: false

responses:

'200':

description: >-

Success case ("200 OK").

The resources identified in the request for retrieval are returned

in the response message body. In case the attributes or fields query

parameters are used, only the selected attributes or sub-attributes are

returned. The response message body is constructed according to the

hierarchical response construction method (TS 32.158 [15]).

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

application/vnd.3gpp.object-tree-hierarchical+json:

schema:

$ref: '#/components/schemas/Resource'

application/vnd.3gpp.object-tree-flat+json:

schema:

type: array

items:

$ref: '#/components/schemas/Resource'

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

patch:

summary: Patches one or multiple resources

description: >-

With HTTP PATCH resources are created, updated or deleted. The resources

to be modified are identified with the target URI (base resource) and

the patch document included in the request message body.

requestBody:

description: >-

The request body describes changes to be made to the target resources.

The following patch media types are available

- "application/merge-patch+json" (RFC 7396)

- "application/3gpp-merge-patch+json" (TS 32.158)

- "application/json-patch+json" (RFC 6902)

- "application/3gpp-json-patch+json" (TS 32.158)

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/Resource'

application/3gpp-merge-patch+json:

schema:

$ref: '#/components/schemas/Resource'

application/json-patch+json:

schema:

type: array

items:

$ref: '#/components/schemas/PatchItem'

application/3gpp-json-patch+json:

schema:

type: array

items:

$ref: '#/components/schemas/PatchItem'

responses:

'200':

description: >-

Success case ("200 OK").

This status code is returned when the updated the resource representations

shall be returned for some reason.

The resource representations are returned in the response message body. The

response message body is constructed according to the hierarchical response

construction method (TS 32.158 [15])

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

'204':

description: >-

Success case ("204 No Content").

This status code is returned when there is no need to return the updated

resource representations.

The response message body is empty.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

delete:

summary: Deletes one resource

description: >-

With HTTP DELETE one resource is deleted. The resources to be deleted is

identified with the target URI.

responses:

'200':

description: >-

Success case ("200 OK").

This status code is returned, when the resource has been successfully deleted.

The response body is empty.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

components:

schemas:

CmNotificationTypes:

type: string

enum:

- notifyMOICreation

- notifyMOIDeletion

- notifyMOIAttributeValueChanges

- notifyMOIChanges

SourceIndicator:

type: string

enum:

- RESOURCE_OPERATION

- MANAGEMENT_OPERATION

- SON_OPERATION

- UNKNOWN

ScopeType:

type: string

enum:

- BASE_ONLY

- BASE_NTH_LEVEL

- BASE_SUBTREE

- BASE_ALL

Operation:

type: string

enum:

- add

- remove

- replace

Insert:

type: string

enum:

- before

- after

PatchOperation:

type: string

enum:

- add

- replace

- remove

- copy

- move

- test

Resource:

oneOf:

- type: object

properties:

id:

type: string

objectClass:

type: string

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

attributes:

type: object

additionalProperties:

type: array

items:

type: object

required:

- id

- anyOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/resources-genericNrm'

- $ref: 'TS28541_NrNrm.yaml#/components/schemas/resources-nrNrm'

- $ref: 'TS28541_5GcNrm.yaml#/components/schemas/resources-5gcNrm'

- $ref: 'TS28541_SliceNrm.yaml#/components/schemas/resources-sliceNrm'

- $ref: 'TS28536_CoslaNrm.yaml#/components/schemas/resources-coslaNrm'

- $ref: 'TS28312_IntentNrm.yaml#/components/schemas/resources-intentNrm'

- $ref: 'TS28104_MdaNrm.yaml#/components/schemas/resources-mdaNrm'

- $ref: 'TS28105_AiMlNrm.yaml#/components/schemas/resources-AiMlNrm'

- $ref: 'TS28538_EdgeNrm.yaml#/components/schemas/resources-edgeNrm'

Scope:

type: object

properties:

scopeType:

$ref: '#/components/schemas/ScopeType'

scopeLevel:

type: integer

CorrelatedNotification:

type: object

properties:

source:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

notificationIds:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationId'

required:

- source

- notificationIds

MoiChange:

type: object

properties:

notificationId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationId'

correlatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

additionalText:

type: string

sourceIndicator:

$ref: '#/components/schemas/SourceIndicator'

op:

$ref: '#/components/schemas/Operation'

path:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

insert:

$ref: '#/components/schemas/Insert'

value: {}

oldValue: {}

required:

- notificationId

- op

- path

NotifyMoiCreation:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

correlatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

additionalText:

type: string

sourceIndicator:

$ref: '#/components/schemas/SourceIndicator'

attributeList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyMoiDeletion:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

correlatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

additionalText:

type: string

sourceIndicator:

$ref: '#/components/schemas/SourceIndicator'

attributeList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyMoiAttributeValueChanges:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

correlatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

additionalText:

type: string

sourceIndicator:

$ref: '#/components/schemas/SourceIndicator'

attributeListValueChanges:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeValueChangeSet'

required:

- attributeListValueChanges

NotifyMoiChanges:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

moiChanges:

type: array

items:

$ref: '#/components/schemas/MoiChange'

required:

- moiChanges

PatchItem:

type: object

properties:

op:

$ref: '#/components/schemas/PatchOperation'

from:

type: string

path:

type: string

value:

nullable: true

required:

- op

- path

<CODE ENDS>

7th Change

## A.2.1	OpenAPI document "TS28532_FaultMnS.yaml"

<CODE BEGINS>

openapi: 3.0.1

info:

title: Fault Supervision MnS

version: 17.2.0

description: >-

OAS 3.0.1 definition of the Fault Supervision MnS

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.532; Generic management services

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.532/

servers:

- url: '{MnSRoot}/FaultSupervisionMnS/{MnSLabel}/{MnSversion}'

variables:

MnSRoot:

description: See subclause 4.4.3 of TS 32.158

default: http://example.com/3GPPManagement

MnSLabel:

description: Label of the MnS

default: 3GPP

MnSversion:

description: Version number of the OpenAPI definition

default: XXX

paths:

/alarms:

get:

summary: Retrieve multiple alarms

description: >-

Retrieves the alarms identified by alarmAckState, baseObjectInstance

and filter.

parameters:

- name: alarmAckState

in: query

required: false

schema:

$ref: '#/components/schemas/AlarmAckState'

- name: baseObjectInstance

in: query

required: false

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

- name: filter

in: query

required: false

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Filter'

responses:

'200':

description: >-

Success case ("200 OK").

Returns the alarms identified in the request. The alarmId is the key

of the map.

content:

application/json:

schema:

type: object

additionalProperties:

type: object

allOf:

- type: object

properties:

lastNotificationHeader:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- $ref: '#/components/schemas/AlarmRecord'

- type: object

properties:

comments:

$ref: '#/components/schemas/Comments'

default:

description: Response in case of error.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

patch:

summary: 'Clear, acknowledge or unacknowledge multiple alarms'

description: >-

Clears, acknowledges or unacknowledges multiple alarms using patch. Depending

on which action is to be performed, different merge patch documents need

to be used.

requestBody:

description: >-

Patch documents for acknowledging and unacknowledging, or clearing multiple

alarms. The keys in the map are the alarmIds to be patched.

content:

application/merge-patch+json:

schema:

oneOf:

- type: object

additionalProperties:

$ref: '#/components/schemas/MergePatchAcknowledgeAlarm'

- type: object

additionalProperties:

$ref: '#/components/schemas/MergePatchClearAlarm'

responses:

'204':

description: >-

Success case ("204 No content").

The response message body is empty.

default:

description: Response in case of error.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/FailedAlarm'

/alarms/alarmCount:

get:

summary: Get the alarm count per perceived severity

parameters:

- name: alarmAckState

in: query

required: false

schema:

$ref: '#/components/schemas/AlarmAckState'

- name: filter

in: query

required: false

schema:

type: string

responses:

'200':

description: >-

Success case ("200 OK").

The alarm count per perceived severity is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/AlarmCount'

default:

description: Response in case of error. The error case needs rework.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

/alarms/{alarmId}:

patch:

summary: 'Clear, acknowledge or unacknowledge a single alarm'

description: >-

Clears, acknowledges or uncknowldeges a single alarm by patching the alarm

information. A conditional acknowledge request based on the perceived

severity is not supported.

parameters:

- name: alarmId

in: path

description: Identifies the alarm to be patched.

required: true

schema:

type: string

requestBody:

required: true

content:

application/merge-patch+json:

schema:

oneOf:

- $ref: '#/components/schemas/MergePatchAcknowledgeAlarm'

- $ref: '#/components/schemas/MergePatchClearAlarm'

responses:

'204':

description: >-

Success case (204 No content).

The response message body is absent.

default:

description: Response in case of error.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

/alarms/{alarmId}/comments:

post:

summary: Add a comment to a single alarm

description: >-

Adds a comment to an alarm identified by alarmId. The id of the new comment

is allocated by the producer.

parameters:

- name: alarmId

in: path

description: Identifies the alarm to which the comment shall be added.

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/Comment'

responses:

'201':

description: >-

Success case (201 Created).

The representation of the newly created comment resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/Comment'

headers:

Location:

description: URI of the newly created comment resource.

required: true

schema:

type: string

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

/subscriptions:

post:

summary: Create a subscription

description: >-

To create a subscription the representation of the subscription is

POSTed on the /subscriptions collection resource.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/Subscription'

responses:

'201':

description: >-

Success case ("201 Created").

The representation of the newly created subscription resource shall

be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/Subscription'

headers:

Location:

description: URI of the newly created subscription resource

required: true

schema:

type: string

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

callbacks:

notifyNewAlarm:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

oneOf:

- $ref: '#/components/schemas/NotifyNewAlarm'

- $ref: '#/components/schemas/NotifyNewSecAlarm'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyClearedAlarm:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyClearedAlarm'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyChangedAlarm:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyChangedAlarm'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyChangedAlarmGeneral:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

oneOf:

- $ref: '#/components/schemas/NotifyChangedAlarmGeneral'

- $ref: '#/components/schemas/NotifyChangedSecAlarmGeneral'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyCorrelatedNotificationChanged:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyCorrelatedNotificationChanged'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyAckStateChanged:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyAckStateChanged'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyComments:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyComments'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyPotentialFaultyAlarmList:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyPotentialFaultyAlarmList'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyAlarmListRebuilt:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyAlarmListRebuilt'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

/subscriptions/{subscriptionId}:

delete:

summary: Delete a subscription

description: >-

The subscription is deleted by deleting the corresponding subscription

resource. The resource to be deleted is identified with the path

component of the URI.

parameters:

- name: subscriptionId

in: path

description: Identifies the subscription to be deleted.

required: true

schema:

type: string

responses:

'204':

description: >-

Success case ("204 No Content").

The subscription resource has been deleted. The response message body

is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

components:

schemas:

#---- Definition of AlarmRecord ----------------------------------------------------#

AlarmId:

type: string

AlarmType:

type: string

enum:

- COMMUNICATIONS_ALARM

- QUALITY_OF_SERVICE_ALARM

- PROCESSING_ERROR_ALARM

- EQUIPMENT_ALARM

- ENVIRONMENTAL_ALARM

- INTEGRITY_VIOLATION

- OPERATIONAL_VIOLATION

- PHYSICAL_VIOLATION

- SECURITY_SERVICE_OR_MECHANISM_VIOLATION

- TIME_DOMAIN_VIOLATION

ProbableCause:

description: >-

The value of the probable cause may be a specific standardized string, or any

vendor provided string. Probable cause strings are not standardized in the

present document. They may be added in a future version. Up to then the

mapping of the generic probable cause strings "PROBABLE_CAUSE_001" to

"PROBABLE_CAUSE_005" is vendor specific.

The value of the probable cause may also be an integer. The mapping of integer

values to probable causes is vendor specific.

oneOf:

- anyOf:

- type: string

enum:

- PROBABLE_CAUSE_001

- PROBABLE_CAUSE_002

- PROBABLE_CAUSE_003

- PROBABLE_CAUSE_004

- PROBABLE_CAUSE_005

- type: string

- type: integer

SpecificProblem:

oneOf:

- type: string

- type: integer

PerceivedSeverity:

type: string

enum:

- INDETERMINATE

- CRITICAL

- MAJOR

- MINOR

- WARNING

- CLEARED

TrendIndication:

type: string

enum:

- MORE_SEVERE

- NO_CHANGE

- LESS_SEVERE

ThresholdHysteresis:

type: object

required:

- high

properties:

high:

oneOf:

- type: integer

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/Float'

low:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Float'

ThresholdLevelInd:

oneOf:

- type: object

properties:

up:

$ref: '#/components/schemas/ThresholdHysteresis'

- type: object

properties:

down:

$ref: '#/components/schemas/ThresholdHysteresis'

ThresholdInfo:

type: object

properties:

observedMeasurement:

type: string

observedValue:

type: number

thresholdLevel:

$ref: '#/components/schemas/ThresholdLevelInd'

armTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

required:

- observedMeasurement

- observedValue

CorrelatedNotification:

type: object

properties:

sourceObjectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

notificationIds:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationId'

required:

- sourceObjectInstance

- notificationIds

CorrelatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

AckState:

type: string

enum:

- ACKNOWLEDGED

- UNACKNOWLEDGED

AlarmRecord:

description: >-

The alarmId is not a property of an alarm record. It is used as key

in the map of alarm records instead.

type: object

properties:

# alarmId:

#  $ref: '#/components/schemas/AlarmId'

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

notificationId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationId'

alarmRaisedTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

alarmChangedTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

alarmClearedTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

backedUpStatus:

type: boolean

backUpObject:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

trendIndication:

$ref: '#/components/schemas/TrendIndication'

thresholdinfo:

$ref: '#/components/schemas/ThresholdInfo'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

stateChangeDefinition:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeValueChangeSet'

monitoredAttributes:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

proposedRepairActions:

type: string

additionalText:

type: string

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

ackTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

ackUserId:

type: string

ackSystemId:

type: string

ackState:

$ref: '#/components/schemas/AckState'

clearUserId:

type: string

clearSystemId:

type: string

serviceUser:

type: string

serviceProvider:

type: string

securityAlarmDetector:

type: string

#---- Definition of alarm notifications --------------------------------------------#

AlarmNotificationTypes:

type: string

enum:

- notifyNewAlarm

- notifyChangedAlarm

- notifyChangedAlarmGeneral

- notifyAckStateChanged

- notifyCorrelatedNotificationChanged

- notifyComments

- notifyClearedAlarm

- notifyAlarmListRebuilt

- notifyPotentialFaultyAlarmList

AlarmListAlignmentRequirement:

type: string

enum:

- ALIGNMENT_REQUIRED

- ALIGNMENT_NOT_REQUIRED

NotifyNewAlarm:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

backedUpStatus:

type: boolean

backUpObject:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

trendIndication:

$ref: '#/components/schemas/TrendIndication'

thresholdInfo:

$ref: '#/components/schemas/ThresholdInfo'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

stateChangeDefinition:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeValueChangeSet'

monitoredAttributes:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

proposedRepairActions:

type: string

additionalText:

type: string

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

NotifyNewSecAlarm:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

- serviceUser

- serviceProvider

- securityAlarmDetector

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

additionalText:

type: string

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

serviceUser:

type: string

serviceProvider:

type: string

securityAlarmDetector:

type: string

NotifyClearedAlarm:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

clearUserId:

type: string

clearSystemId:

type: string

NotifyChangedAlarm:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

NotifyChangedAlarmGeneral:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

specificProblem:

$ref: '#/components/schemas/SpecificProblem'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

backedUpStatus:

type: boolean

backUpObject:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

trendIndication:

$ref: '#/components/schemas/TrendIndication'

thresholdInfo:

$ref: '#/components/schemas/ThresholdInfo'

stateChangeDefinition:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeValueChangeSet'

monitoredAttributes:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

proposedRepairActions:

type: string

additionalText:

type: string

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

changedAlarmAttributes:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyChangedSecAlarmGeneral:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- serviceUser

- serviceProvider

- securityAlarmDetector

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

additionalText:

type: string

additionalInformation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

rootCauseIndicator:

type: boolean

serviceUser:

type: string

serviceProvider:

type: string

securityAlarmDetector:

type: string

changedAlarmAttributes:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyCorrelatedNotificationChanged:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- correlatedNotifications

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

correlatedNotifications:

$ref: '#/components/schemas/CorrelatedNotifications'

rootCauseIndicator:

type: boolean

NotifyAckStateChanged:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

- ackState

- ackUserId

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

ackState:

$ref: '#/components/schemas/AckState'

ackUserId:

type: string

ackSystemId:

type: string

NotifyComments:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- alarmId

- alarmType

- probableCause

- perceivedSeverity

- comments

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

alarmType:

$ref: '#/components/schemas/AlarmType'

probableCause:

$ref: '#/components/schemas/ProbableCause'

perceivedSeverity:

$ref: '#/components/schemas/PerceivedSeverity'

comments:

$ref: '#/components/schemas/Comments'

NotifyPotentialFaultyAlarmList:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- reason

properties:

reason:

type: string

NotifyAlarmListRebuilt:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

required:

- reason

properties:

reason:

type: string

alarmListAlignmentRequirement:

$ref: '#/components/schemas/AlarmListAlignmentRequirement'

#---- Definition of query parameters -----------------------------------------------#

AlarmAckState:

type: string

enum:

- ALL_ALARMS

- ALL_ACTIVE_ALARMS

- ALL_ACTIVE_AND_ACKNOWLEDGED_ALARMS

- ALL_ACTIVE_AND_UNACKNOWLEDGED_ALARMS

- ALL_CLEARED_AND_UNACKNOWLEDGED_ALARMS

- ALL_UNACKNOWLEDGED_ALARMS

#---- Definition of patch documents ------------------------------------------------#

MergePatchAcknowledgeAlarm:

description: >-

Patch document acknowledging or unacknowledging a single alarm. For

acknowleding an alarm the value of ackState is ACKNOWLEDGED, for unacknowleding

an alarm the value of ackState is UNACKNOWLEDGED.

type: object

required:

- ackUserId

- ackState

properties:

ackUserId:

type: string

ackSystemId:

type: string

ackState:

$ref: '#/components/schemas/AckState'

MergePatchClearAlarm:

description: Patch document for clearing a single alarm

type: object

required:

- clearUserId

- perceivedSeverity

properties:

clearUserId:

type: string

clearSystemId:

type: string

perceivedSeverity:

type: string

enum:

- CLEARED

#---- Definition of method responses -----------------------------------------------#

FailedAlarm:

type: object

required:

- alarmId

- failureReason

properties:

alarmId:

$ref: '#/components/schemas/AlarmId'

failureReason:

type: string

#---- Definition of resources ------------------------------------------------------#

AlarmCount:

type: object

required:

- criticalCount

- majorCount

- minorCount

- warningCount

- indeterminateCount

- clearedCount

properties:

criticalCount:

type: integer

majorCount:

type: integer

minorCount:

type: integer

warningCount:

type: integer

indeterminateCount:

type: integer

clearedCount:

type: integer

Comment:

type: object

properties:

commentTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

commentUserId:

type: string

commentSystemId:

type: string

commentText:

type: string

Comments:

description: >-

Collection of comments. The comment identifiers are allocated by the

MnS producer and used as key in the map.

type: object

additionalProperties:

$ref: '#/components/schemas/Comment'

Subscription:

type: object

properties:

consumerReference:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

timeTick:

type: integer

filter:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Filter'

<CODE ENDS>

8th Change

## A.6.2	OpenAPI document "TS28532_StreamingDataMnS.yaml"

<CODE BEGINS>

openapi: 3.0.1

info:

title: TS 28.532 Streaming data reporting service

version: 17.1.0

description: >-

OAS 3.0.1 specification for the Streaming data reporting service (Streaming MnS)

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

servers:

- url: '{MnSRoot}/StreamingDataReportingMnS/{MnSLabel}/{MnSVersion}'

variables:

MnSRoot:

description: See clause 4.4.3 of TS 32.158.

default: https://example.com/3GPPManagement

MnSLabel:

description: Label of the MnS

default: 3GPP

MnSVersion:

description: See clause 4.4.3 of TS 32.158.

default: ''

paths:

'/connections':

post:

summary: Inform consumer about reporting streams to be carried by the new connection and receive a new connection id.

description: Exchange of meta-data (producer informs consumer about its own identity and the nature of the data to be reported via streaming) phase of the connection establishement by streaming data reporting producer to the streaming data reporting consumer (i.e. streaming target).

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/connectionRequest-Type'

responses:

'201':

description: Success case (201 Created).

headers:

Location:

description: Location of the created connection resource.

schema:

$ref: '#/components/schemas/connectionId-Type'

default:

description: Error case.

content:

application/json:

schema:

$ref: '#/components/schemas/failedConnectionResponse-Type'

get:

summary: Obtain information about connections.

description: Enables the streaming data reporting service producer to obtain information about one or more streaming connections.

parameters:

- name: connectionIdList

in: query

description: The list of connectionId for which the connection information is to be returned.

required: false

schema:

type: array

items:

$ref: '#/components/schemas/connectionId-Type'

responses:

'200':

description: Success case (200 OK). The resources identified in the request for retrieval are returned in the response message body. In case the fields query parameter is used, the selected resources are returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/connectionInfo-Type'

'202':

description: Partial success case (202 Partially retrieved). Subset of the resources identified in the request for retrieval are returned in the response message body.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/connectionInfo-Type'

default:

description: Error case.

content:

application/json:

schema:

$ref: '#/components/schemas/errorResponse-Type'

'/connections/{connectionId}':

get:

summary: Obtain information about a connection.

description: Enables the streaming data reporting service producer to obtain information about one streaming connection.

parameters:

- name: connectionId

in: path

description: Indicate the ID (URI) of the connection for which the information is being retrieved

required: true

schema:

$ref: '#/components/schemas/connectionId-Type'

- name: Connection

in: header

schema:

$ref: '#/components/schemas/websocketHeaderConnection-Type'

- name: Sec-WebSocket-Extensions

in: header

schema:

$ref: '#/components/schemas/websocketHeader-Sec-WebSocket-Extensions-Type'

- name: Sec-WebSocket-Key

in: header

schema:

$ref: '#/components/schemas/websocketHeader-Sec-WebSocket-Key-Type'

- name: Sec-WebSocket-Protocol

in: header

schema:

$ref: '#/components/schemas/websocketHeader-Sec-WebSocket-Protocol-Type'

- name: Sec-WebSocket-Version

in: header

schema:

$ref: '#/components/schemas/websocketHeader-Sec-WebSocket-Version-Type'

responses:

'101':

description: Success case (101 Switching Protocols). The connection has been successfully switched to WebSocket. The response message body is absent.

headers:

Upgrade:

schema:

$ref: '#/components/schemas/websocketHeaderUpgrade-Type'

Connection:

schema:

$ref: '#/components/schemas/websocketHeaderConnection-Type'

Sec-WebSocket-Accept:

schema:

$ref: '#/components/schemas/websocketHeader-Sec-WebSocket-Accept-Type'

'200':

description: Success case (200 OK). The resource identified in the request for retrieval returned in the response message body.

content:

application/json:

schema:

$ref: '#/components/schemas/connectionInfo-Type'

default:

description: Error case.

content:

application/json:

schema:

$ref: '#/components/schemas/errorResponse-Type'

'/connections/{connectionId}/streams':

post:

summary: Inform consumer about new reporting streams on an existing connection.

description: Allows the producer to add one or more reporting streams to an already established streaming connection.

parameters:

- name: connectionId

in: path

description: Indicate the ID (URI) of the connection for which the reporting stream information is being added.

required: true

schema:

$ref: '#/components/schemas/connectionId-Type'

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/streamInfo-Type'

responses:

'201':

description: Success case (201 Posted).

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/streamInfo-Type'

'202':

description: Partial success case (202 Posted).

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/streamInfo-Type'

default:

description: Error case.

content:

application/json:

schema:

$ref: '#/components/schemas/errorResponse-Type'

delete:

summary: Remove reporting streams from an existing connection

description: Allows the producer to remove one or more reporting streams from an already established streaming connection.

parameters:

- name: connectionId

in: path

description: Indicate the ID (URI) of the connection for which the reporting stream information is being removed.

required: true

schema:

$ref: '#/components/schemas/connectionId-Type'

- name: streamIds

in: query

description: The list of streamId for the stream(s) to be deleted.

required: true

schema:

type: array

items:

$ref: '#/components/schemas/streamId-Type'

responses:

'204':

description: Success case (204 No Content). The stream information resource has been deleted. The response message body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: '#/components/schemas/errorResponse-Type'

get:

summary: Obtain information about streams.

description: Enables the streaming data reporting service producer to obtain information about one or more reporting streams.

parameters:

- name: connectionId

in: path

description: Indicate the ID (URI) of the connection for which the information is being retrieved

required: true

schema:

$ref: '#/components/schemas/connectionId-Type'

- name: streamIds

in: query

description: The list of streamId for which the stream information is to be retrieved.

required: true

schema:

type: array

items:

$ref: '#/components/schemas/streamId-Type'

responses:

'200':

description: Success case (200 OK).

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/streamInfoWithReporters-Type'

'202':

description: Partial success case (202 Partially retrieved).

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/streamInfoWithReporters-Type'

default:

description: Error case.

content:

application/json:

schema:

$ref: '#/components/schemas/errorResponse-Type'

'/connections/{connectionId}/streams/{streamId}':

get:

summary: Obtain information about stream

description: Enables the streaming data reporting service producer to obtain information about a reporting stream.

parameters:

- name: connectionId

in: path

description: Indicate the ID (URI) of the connection for which the information is being retrieved

required: true

schema:

$ref: '#/components/schemas/connectionId-Type'

- name: streamId

in: path

description: Indicate the ID of the reporting stream for which the information is being retrieved

required: true

schema:

$ref: '#/components/schemas/streamId-Type'

responses:

'200':

description: Success case (200 OK).

content:

application/json:

schema:

$ref: '#/components/schemas/streamInfoWithReporters-Type'

default:

description: Error case.

content:

application/json:

schema:

$ref: '#/components/schemas/errorResponse-Type'

components:

schemas:

analyticsInfo-Type:

description: Information specific to analytics reporting.

type: object

properties:

activityDetails:

type: string

connectionId-Type:

$ref: '#/components/schemas/uri-Type'

connectionInfo-Type:

type: object

properties:

connection:

$ref: '#/components/schemas/connectionId-Type'

producer:

$ref: '#/components/schemas/producerId-Type'

streams:

type: array

items:

$ref: '#/components/schemas/streamId-Type'

connectionRequest-Type:

type: object

properties:

producer:

$ref: '#/components/schemas/producerId-Type'

streams:

type: array

items:

$ref: '#/components/schemas/streamInfo-Type'

errorResponse-Type:

type: object

properties:

error:

type: object

properties:

errorInfo:

type: string

failedConnectionResponse-Type:

type: object

properties:

error:

type: array

items:

type: object

properties:

streamId:

$ref: '#/components/schemas/streamId-Type'

errorReason:

type: string

measObjDn-Type:

description: DN of the measured object instance (see 3GPP TS 28.550)

allOf:

- $ref: '#/components/schemas/systemDN-Type'

performanceMetrics-Type:

description: an ordered list of performance metric names (see clause 4.4.1 of 3GPP TS 28.622[11]) whose values are to be reported by the Performance Data Stream Units (see Annex C of TS 28.550 [42]) via this stream. Performance metrics include measurement and KPI

type: array

items:

type: string

performanceInfo-Type:

description: Information specific to performance data reporting

type: object

properties:

measObjDn:

$ref: '#/components/schemas/measObjDn-Type'

performanceMetrics:

$ref: '#/components/schemas/performanceMetrics-Type'

jobId:

type: string

required:

- measObjDn

- performanceMetrics

producerId-Type:

description: DN of the streaming data reporting MnS producer.

allOf:

- $ref: '#/components/schemas/systemDN-Type'

serializationFormat-Type:

type: string

enum:

- GPB

- ASN1

streamId-Type:

description: globally unique stream identifier

type: string

example: '26F452550021'

streamInfo-Type:

description: Reporting stream meta-data.

type: object

properties:

streamType:

$ref: '#/components/schemas/streamType-Type'

serializationFormat:

$ref: '#/components/schemas/serializationFormat-Type'

streamId:

oneOf:

- $ref: '#/components/schemas/streamId-Type'

- $ref: '#/components/schemas/traceReference-Type'

additionalInfo:

oneOf:

- $ref: '#/components/schemas/traceInfo-Type'

- $ref: '#/components/schemas/performanceInfo-Type'

- $ref: '#/components/schemas/analyticsInfo-Type'

- $ref: '#/components/schemas/vsDataContainer-Type'

required:

- streamType

- serializationFormat

- streamId

streamInfoWithReporters-Type:

description: Reporting stream meta-data with added information about reporters.

type: object

properties:

streamInfo:

$ref: '#/components/schemas/streamInfo-Type'

reporters:

type: array

items:

$ref: '#/components/schemas/producerId-Type'

systemDN-Type:

description: See 3GPP TS 32.300 for details

type: string

example: 'SubNetwork=ABCNetwork,SubNetwork=MUC01,GNBDUFunction=XYZ0100'

streamType-Type:

type: string

enum:

- TRACE

- PERFORMANCE

- ANALYTICS

- PROPRIETARY

traceInfo-Type:

description: Information specific to trace data reporting

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/TraceJob-Attr'

traceReference-Type:

description: Trace Reference (see clause 5.6 of 3GPP TS 32.422) as stream identifier for streaming trace data reporting

type: string

example: '4358070034D7'

uri-Type:

description: Resource URI

type: string

vsDataContainer-Type:

description: container for vendor specific data (see 3GPP TS 28.622)

type: object

properties:

vsDataType:

type: string

vsData:

type: string

vsDataFormatVersion:

type: string

websocketHeaderConnection-Type:

description: Header value for the upgrade request and response.

type: string

enum:

- Upgrade

websocketHeaderUpgrade-Type:

description: Header value for the upgrade to WebSocket request and response.

type: string

enum:

- websocket

websocketHeader-Sec-WebSocket-Accept-Type:

description: Header value for secure WebSocket response. Carries hash.

type: string

websocketHeader-Sec-WebSocket-Extensions-Type:

description: Header value for secure WebSocket request. Carries protocol extensions.

type: string

websocketHeader-Sec-WebSocket-Key-Type:

description: Header value for secure WebSocket request. Provides information to the server which is needed in order to confirm that the client is entitled to request an upgrade to WebSocket.

type: string

websocketHeader-Sec-WebSocket-Protocol-Type:

description: Header value for secure WebSocket request. Carries a comma-separated list of subprotocol names, in the order of preference.

type: string

websocketHeader-Sec-WebSocket-Version-Type:

description: Header value for secure WebSocket request and response. Carries the WebSocket protocol version to be used.

type: string

<CODE ENDS>

9th Change

## A.7.2	OpenAPI document "TS 28532_FileDataReportingMnS.yaml"

<CODE BEGINS>

openapi: 3.0.1

info:

title: File Data Reporting MnS

version: 17.1.0

description: >-

OAS 3.0.1 definition of the File Data Reporting MnS

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.532; Generic management services

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.532/

servers:

- url: '{MnSRoot}/fileDataReportingMnS/{MnSLabel}/{MnSVersion}'

variables:

MnSRoot:

description: See clause 4.4.3 of TS 32.158

default: http://example.com/3GPPManagement

MnSLabel:

description: Label of the MnS

default: 3GPP

MnSVersion:

description: Version number of the OpenAPI definition

default: XXX

paths:

/files:

get:

summary: Read information about available files

description: >-

Information about available files is read with HTTP GET. The files for

which information shall be returned are identified with the path

component (base resource) and the query component (fileDataType, beginTime,

endTime) of the URI.

parameters:

- name: fileDataType

in: query

description: >-

This parameter selects files based on the file data type.

required: true

schema:

$ref: '#/components/schemas/FileDataType'

- name: beginTime

in: query

description: >-

This parameter selects files based on the earliest time they

became available

required: false

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

- name: endTime

in: query

description: >-

This parameter selects files based on the latest time they

became available

required: false

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

responses:

'200':

description: >-

'Success case ("200 OK").

The resources identified in the request for retrieval are returned

in the response message body.'

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/FileInfo'

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

/subscriptions:

post:

summary: Create a subscription

description: >-

To create a subscription the representation of the subscription is

POSTed on the /subscriptions collection resource.

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS28532_FaultMnS.yaml#/components/schemas/Subscription'

responses:

'201':

description: >-

Success case ("201 Created").

The representation of the newly created subscription resource shall

be returned.

content:

application/json:

schema:

$ref: 'TS28532_FaultMnS.yaml#/components/schemas/Subscription'

headers:

Location:

description: URI of the newly created subscription resource

required: true

schema:

type: string

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

callbacks:

notifyFileReady:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyFileReady'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyFilePreparationError:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyFilePreparationError'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

/subscriptions/{subscriptionId}:

delete:

summary: Delete a subscription

description: >-

The subscription is deleted by deleting the corresponding subscription

resource. The resource to be deleted is identified with the path

component of the URI.

parameters:

- name: subscriptionId

in: path

description: Identifies the subscription to be deleted.

required: true

schema:

type: string

responses:

'204':

description: >-

Success case ("204 No Content").

The subscription resource has been deleted. The response message body

is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

components:

schemas:

FileDataType:

type: string

enum:

- Performance

- Trace

- Anatytics

- Proprietary

FileNotificationTypes:

type: string

enum:

- notifyFileReady

- notifyFilePreparationError

FileInfo:

type: object

properties:

fileLocation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

fileSize:

type: integer

fileReadyTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

fileExpirationTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

fileCompression:

type: string

fileFormat:

type: string

fileDataType:

$ref: '#/components/schemas/FileDataType'

NotifyFileReady:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

fileInfoList:

type: array

items:

$ref: '#/components/schemas/FileInfo'

additionalText:

type: string

NotifyFilePreparationError:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

fileInfoList:

type: array

items:

$ref: '#/components/schemas/FileInfo'

reason:

type: string

additionalText:

type: string

<CODE ENDS>

*** END OF CHANGE 2 ***

*** START OF CHANGE 3 ***

*** OpenAPI/TS28532_ProvMnS.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Provisioning MnS

version: 17.5.0

description: >-

OAS 3.0.1 definition of the Provisioning MnS

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.532; Generic management services

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.532/

servers:

- url: '{MnSRoot}/ProvMnS/{MnSLabel}/{MnSVersion}/{URI-LDN-first-part}'

variables:

MnSRoot:

description: See clause 4.4.2 of TS 32.158

default: http://example.com/3GPPManagement

MnSLabel:

description: Label of the MnS

default: 3GPP

MnSVersion:

description: Version number of the OpenAPI definition

default: XXX

URI-LDN-first-part:

description: See clause 4.4.2 of TS 32.158

default: ''

paths:

'/{className}={id}':

parameters:

- name: className

in: path

required: true

schema:

type: string

- name: id

in: path

required: true

schema:

type: string

put:

summary: Replaces a complete single resource or creates it if it does not exist

description: >-

With HTTP PUT a complete resource is replaced or created if it does not

exist. The target resource is identified by the target URI.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

responses:

'200':

description: >-

Success case ("200 OK").

This status code shall be returned when the resource is replaced, and

when the replaced resource representation is not identical to the resource

representation in the request.

This status code may be retourned when the resource is updated and when the

updated resource representation is identical to the resource representation

in the request.

The representation of the updated resource is returned in the response

message body.

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

'201':

description: >-

Success case ("201 Created").

This status code shall be returned when the resource is created.

The representation of the created resource is returned in the response

message body.

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

'204':

description: >-

Success case ("204 No Content").

This status code may be returned only when the replaced resource

representation is identical to the representation in the request.

The response has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

callbacks:

notifyMOICreation:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyMoiCreation'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyMOIDeletion:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyMoiDeletion'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyMOIAttributeValueChanges:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyMoiAttributeValueChanges'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

notifyMOIChanges:

'{request.body#/notificationRecipientAddress}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyMoiChanges'

application/yang-data+json:

schema:

$ref: '#/components/schemas/NotifyMoiChanges'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response

has no message body.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

get:

summary: Reads one or multiple resources

description: >-

With HTTP GET resources are read. The resources to be retrieved are

identified with the target URI. The attributes and fields parameter

of the query components allow to select the resource properties to be returned.

parameters:

- name: scope

in: query

description: >-

This parameter extends the set of targeted resources beyond the base

resource identified with the path component of the URI. No scoping

mechanism is specified in the present document.

required: false

schema:

$ref: '#/components/schemas/Scope'

style: form

explode: true

- name: filter

in: query

description: >-

This parameter reduces the targeted set of resources by applying a

filter to the scoped set of resource representations. Only resource

representations for which the filter construct evaluates to "true"

are targeted. No filter language is specified in the present

document.

required: false

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Filter'

- name: attributes

in: query

description: >-

This parameter specifies the attributes of the scoped resources that

are returned.

required: true

schema:

type: array

items:

type: string

style: form

explode: false

- name: fields

in: query

description: >-

This parameter specifies the attribute field of the scoped resources

that are returned.

required: false

schema:

type: array

items:

type: string

style: form

explode: false

responses:

'200':

description: >-

Success case ("200 OK").

The resources identified in the request for retrieval are returned

in the response message body. In case the attributes or fields query

parameters are used, only the selected attributes or sub-attributes are

returned. The response message body is constructed according to the

hierarchical response construction method (TS 32.158 [15]).

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

application/vnd.3gpp.object-tree-hierarchical+json:

schema:

$ref: '#/components/schemas/Resource'

application/vnd.3gpp.object-tree-flat+json:

schema:

type: array

items:

$ref: '#/components/schemas/Resource'

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

patch:

summary: Patches one or multiple resources

description: >-

With HTTP PATCH resources are created, updated or deleted. The resources

to be modified are identified with the target URI (base resource) and

the patch document included in the request message body.

requestBody:

description: >-

The request body describes changes to be made to the target resources.

The following patch media types are available

- "application/merge-patch+json" (RFC 7396)

- "application/3gpp-merge-patch+json" (TS 32.158)

- "application/json-patch+json" (RFC 6902)

- "application/3gpp-json-patch+json" (TS 32.158)

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/Resource'

application/3gpp-merge-patch+json:

schema:

$ref: '#/components/schemas/Resource'

application/json-patch+json:

schema:

type: array

items:

$ref: '#/components/schemas/PatchItem'

application/3gpp-json-patch+json:

schema:

type: array

items:

$ref: '#/components/schemas/PatchItem'

responses:

'200':

description: >-

Success case ("200 OK").

This status code is returned when the updated the resource representations

shall be returned for some reason.

The resource representations are returned in the response message body. The

response message body is constructed according to the hierarchical response

construction method (TS 32.158 [15])

content:

application/json:

schema:

$ref: '#/components/schemas/Resource'

'204':

description: >-

Success case ("204 No Content").

This status code is returned when there is no need to return the updated

resource representations.

The response message body is empty.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

delete:

summary: Deletes one resource

description: >-

With HTTP DELETE one resource is deleted. The resources to be deleted is

identified with the target URI.

responses:

'200':

description: >-

Success case ("200 OK").

This status code is returned, when the resource has been successfully deleted.

The response body is empty.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

components:

schemas:

CmNotificationTypes:

type: string

enum:

- notifyMOICreation

- notifyMOIDeletion

- notifyMOIAttributeValueChanges

- notifyMOIChanges

SourceIndicator:

type: string

enum:

- RESOURCE_OPERATION

- MANAGEMENT_OPERATION

- SON_OPERATION

- UNKNOWN

ScopeType:

type: string

enum:

- BASE_ONLY

- BASE_NTH_LEVEL

- BASE_SUBTREE

- BASE_ALL

Operation:

type: string

enum:

- add

- remove

- replace

Insert:

type: string

enum:

- before

- after

PatchOperation:

type: string

enum:

- add

- replace

- remove

- copy

- move

- test

Resource:

oneOf:

- type: object

properties:

id:

type: string

objectClass:

type: string

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

attributes:

type: object

additionalProperties:

type: array

items:

type: object

required:

- id

- anyOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/resources-genericNrm'

- $ref: 'TS28541_NrNrm.yaml#/components/schemas/resources-nrNrm'

- $ref: 'TS28541_5GcNrm.yaml#/components/schemas/resources-5gcNrm'

- $ref: 'TS28541_SliceNrm.yaml#/components/schemas/resources-sliceNrm'

- $ref: 'TS28536_CoslaNrm.yaml#/components/schemas/resources-coslaNrm'

- $ref: 'TS28312_IntentNrm.yaml#/components/schemas/resources-intentNrm'

- $ref: 'TS28104_MdaNrm.yaml#/components/schemas/resources-mdaNrm'

- $ref: 'TS28105_AiMlNrm.yaml#/components/schemas/resources-AiMlNrm'

- $ref: 'TS28538_EdgeNrm.yaml#/components/schemas/resources-edgeNrm'

Scope:

type: object

properties:

scopeType:

$ref: '#/components/schemas/ScopeType'

scopeLevel:

type: integer

CorrelatedNotification:

type: object

properties:

source:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

notificationIds:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationId'

required:

- source

- notificationIds

MoiChange:

type: object

properties:

notificationId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationId'

correlatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

additionalText:

type: string

sourceIndicator:

$ref: '#/components/schemas/SourceIndicator'

op:

$ref: '#/components/schemas/Operation'

path:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

insert:

$ref: '#/components/schemas/Insert'

value: {}

oldValue: {}

required:

- notificationId

- op

- path

NotifyMoiCreation:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

correlatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

additionalText:

type: string

sourceIndicator:

$ref: '#/components/schemas/SourceIndicator'

attributeList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyMoiDeletion:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

correlatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

additionalText:

type: string

sourceIndicator:

$ref: '#/components/schemas/SourceIndicator'

attributeList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

NotifyMoiAttributeValueChanges:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

correlatedNotifications:

type: array

items:

$ref: '#/components/schemas/CorrelatedNotification'

additionalText:

type: string

sourceIndicator:

$ref: '#/components/schemas/SourceIndicator'

attributeListValueChanges:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeValueChangeSet'

required:

- attributeListValueChanges

NotifyMoiChanges:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

moiChanges:

type: array

items:

$ref: '#/components/schemas/MoiChange'

required:

- moiChanges

PatchItem:

type: object

properties:

op:

$ref: '#/components/schemas/PatchOperation'

from:

type: string

path:

type: string

value:

nullable: true

required:

- op

- path

<CODE ENDS>