3GPP TSG-SA5 Meeting #150	S5-236594

# Xiamen, China, 9th Oct 2023 to - 13th Oct 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.538 | CR | - | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 InputToDraftCR 28.538 Federated ECS Management Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics France SA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eECM |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The EDGEAPP architecture for federation support requires sharing ECS information belonging to participating operator with Leading operator. It is expected to configure leading operator ECS with infomration related with partner operator ECS. An OAM configuration is expected for the same. The following is the snippet from TS 23.558 “ECS profile of partner ECSs can be preconfigured in the ECS or can be configured by the OAM system. For cases where the preconfigured and OAM configured information is not sufficient or not available, the ECS may communicate with other ECSs, e.g. ECS with repository functionality, to obtain the information.” This configuration is used in “EAS discovery via leading ECSP” procedure to discovery appropiate EAS on partner OP (refer to clause 8.18.2.3.2 of 23.558) The following is the snippet from TS 23.558 “If the ECS does not identify any suitable EES(s) based on EDN configuration available at the ECS and UE’s location, the ECS determines a partner ECS that may satisfy the requirements. Based on ECSP policy, the ECS may use preconfigured or OAM configured information about the partner ECSs or ECS discovery via ECS-ER as specified in clause 8.17.2.3 or both." The partner ECS info can very well be made available to Leading operator ECS using Federation Management procedures. The leading operator can then do the required configuration in its own ECS. The configured ECS information is updated as and when the federation relationship is updated. It is suggested to agree on the normative requirement for this functionality. It is also suggested to send an LS to SA6 clarifying why the assumption of OAM configuration not being sufficient was made. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UC and requirements for ECS information sharing between operators under federation relationship. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In sufficient SA5 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Section

## 5.x	Edge Federation Management

### 5.x.1	Description

This clause contains use cases associated with federation management.

### 5.x.x	Federated ECS management

In federation, the EAS requested by UE may only be available with the federated operator. The EAS discovery will fail at leading operator resulting in the initiation of discovering target EES and ECS belonging to partner operator. See clause 8.18.2.3.2 [2]. This will require configuring leading operator ECS with federated ECS information belonging to partner operator. The information may include ECS Profile (clause 8.2.12[2]), related EES and EAS etc.

The partner operator provides information related with its ECS as part of federation establishment. Based on the provided information required configurations can be done in leading operator ECS.

### 5.x.y	Requirements

Table 5.x.x-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-FECS-MGMT-FUN-1 | Generic Provisioning MnS shall enable federation relationship to include information on PO ECS including (not limited to) ECS Profile, served EES and served EAS. | Federated ECS management |
| REQ-FECS-MGMT-FUN-2 | Generic Provisioning MnS producer shall have a capability to update ECS based on established federation relationship. | Federated ECS management |

End Of Modified Section