3GPP TSG-SA5 Meeting #151	S5-236668

# Xiamen , China 9-13 October 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.533 | CR | draftCR | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 InputToDraftCR 28.533 on Entities and interactions for Access control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MSAC |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The access control inputs have evolved and when we look at the end to end picture we miss a few concepts bridging the details that have been approved. Hence clarity on the concept of access control elaborating the entities and interactions in the first section before the deep dive of identifying information IOCs and relationships |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 4.9.x has been extended to explain the entities and interactions required for access control. Section X.2 has been modified to bridge the concept with the IOCs that are designed for access control related to information unlike the others NRMs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No clarity as to why the IOCs in section X.2 were created and designed. No clarity as to what is the overarching concept when reading the stage 1 and stage 2 together |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.x,X.x, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | New TS is yet to be planned |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is input to approved DraftCR S5-235998 DraftCR S5-235998 is a revision of S5-234845 S5-234845 is a revision of S5-233137 S5-233137 is a revision of S5-227070 S5-227070 is a revision of S5-226544 |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[xx]	IETF RFC 6749: " The OAuth 2.0 Authorization Framework".

[yy]	IETF RFC 8341: " Network Configuration Access Control Model".

## 4.9.x	Identity and Access Control

Network Management systems are becoming a challenge to manage in terms of the users of the system as well as the access control that needs to be applied continuously on a need-to-know basis.

Flexibility and agility to adapt to these growing needs is the key for a sustainable identity and access control implementation.

The service based architecture needs to factor seamless integration to any system for authentication and authorization of Management Service (MnS) consumers.

The need to make continuous access control related changes should be supported with ease and simplicity.

Today we have various human users, machine users and various resources that are continuously growing with complexities below:

- Human users seem to require various levels of access control.

- Machine type of communication in the direction towards automated systems require another type of access and may not necessarily be user name password based.

- Resources and modules themselves require various level of clearance depending on the sensitivity of the data being accessed.

The identity and access control system should be designed for role based access control. Also the authorization rules may support the fine grained permissions i.e. based on MnS component A, B and C.

#### 4.9.x.1 	Role based access control

This is based upon the concept of assigning the appropriate permissions and privileges to authorized users. The combinations of the permissions and privileges make up a role. The users who belong to a role should be assigned to resources based on a least privilege principle and access rules associated to the resource. The principle of least privileges states that the users should be granted access only to the data and the operations that are required to perform the job. This minimizes the possibility of a security breach.

The management system should be setup for access control by a system administrator who will have the know-how of creating the required users and roles. Roles will use the various access rules. Additionally, the system administrator will also need to setup the access rules for a MnS producer.

Roles could be in various categories like full access and restricted access. The access rules will be defined on a MnS producer which could relate to all or combinations of component A, component B and component C.

Figure 4.9.x.1.1-y - role based access

Figure 4.9.x.1.1-y - role based access

The Figure 4.9.x.1.1-y - role based access shows how a user is assigned to a role. The roles in turn are able to act upon a resource with the required operation on a MnS Producer. The actions that can be performed are defined by the access rules.

Start of change

#### 4.9.x.2 	Entities and interactions

In a distributed system the responsibilities of authentication and authorization is split between several entities (Figure 4.9.x.1.1-z):

Figure 4.9.x.2.1-z: Entities and interactions.

The above figure shows the interactions between various entities that need to be involved in setting up and using access control

- Provisioning of access control data into the authentication and authorization function. These functions could be collocated or distributed. The authentication function mainly contains the ‘who’ part of the MnS consumers. The authorization function contains the information of the resource and the action of the MnS producer associated to the ‘who’ of the MnS consumers.

- The MnS consumer is associated to ‘who’ is carrying out the operation and has to be known to the resource owner who will provision this information. The MnS consumer interacts with an authentication function to identify itself.

- The MnS consumer interacts with the authorization function whether it can carry out the action on the resource. The resource owner also provisions the information associating various resources and corresponding actions to valid MnS consumers.

- The MnS consumer interacts with the MnS producer after getting responses from the authentication and authorization functions. The MnS producer further does a validation if the action on its resource can be allowed or not.

The above interactions can be realized in any implementation.

For example in oAUTH we have the possibility of the authentication and authorization function integrated

For example in Netconf we have the possibility of authentication taking place separately on the transport layer and the authorization separately on the Netconf server.

For security reasons not all involved parties shall have the same knowledge, as well as the different interfaces shall not transfer all security-related information. Nevertheless, the different entities need to have a common notion of the security-related parameters. E.g. Authentication and authorization function and potential MnS consumer need to use the same notion of user, as well as authentication and authorization function and potential MnS producer need to use the same notion of resource. Therefore the involved entities need to be based on an overarching security-related information model (Figure 4.9.x.1.1-y), which is the basis for the concrete data models of the security-related interfaces that are needed to fulfil the security-related use cases.

### 4.9.y     Usecases

#### 4.9.y.1 Provisioning identities, roles, and associated resources to the authentication and authorization function

Authorization function and MnS producer need to have a common understanding of resources and refer to the same resources and corresponding actions. The access control information needs to be provisioned in the authentication and authorization function.

#### 4.9.y.2 Provisioning identities to the management function acting as MnS

#### Consumer

Identity must be available to the MnS consumers and assigned by a network administrator for example. As a possible option a human operator might provision his/her credentials by entering user identity (and probably a pass word) via a UI to the MnS consumer.

#### 4.9.y.3	 Authentication and authorization

- To carry out authentication and authorization based on role-based access the following tasks needs to be in place.

- Pre-deployment task – This relates to the identification of the resources represented by the MnS component B and C and it associated operations represented by MnS component A. This is typically done by a Network Equipment Provider (NEP) during the design phase.

- Post deployment task – this relates to the set of administrative tasks which are requires to enable role-based access control typically caried out by a network operator (NOP). This is done once the system is up and running and access control needs to be administered.

NOTE:     The NOP may have a system administration department in charge of defining and configuring role-based access control.

Post the above tasks we have the possibility to have role-based access in operation with every service call being authenticated and authorized.

When an MnS is invoked, the MnS consumer authenticates towards an authentication service producer. This is then followed by checking the access rights with respect to the role-based access. This takes place in the authorization service producer which is invoked from the MnS producer to check if its resources can be accessed with respect to the related operation.

#### 4.9.y.4	 Identity to roles association

The identity or user of the system needs to be assigned the permissions to carry out the management operations. A set of permissions is defined as a role. An efficient and flexible means is to define various roles by a network operator. In a second step, association of the roles to an identity (entity or human user) is carried out.

#### 4.9.y.5	 Roles to associated to resources (consisting of IOCs(static),MOIs(dynamic) and corresponding operations

Role-based access control defines roles. Roles are associated to access rules which are combinations of resources and operations (e.g., CRUD operations). The resources can be represented by IOCs(static) and/or MOIs (dynamic). Hence the resources in context are Component B and Component C and the operations are Component A.

### 4.9.z	Requirements

- UC-MSAC-0x - 3GPPP management system shall support role-based access control for the resources represented by the MnS component A, B and C.

- UC-MSAC-0x.01- 3GPP management system shall support authentication and authorisation for management services.

- UC-MSAC-0x.02 - 3GPP management system shall support identity to role assignment.

- UC-MSAC-0x.03 - 3GPP management system shall support roles which are associated to resources (IOCs(static) and/or MOIs(dynamic)) and the corresponding operations.

## X.2	Class diagrams

The entities and interactions described in the previous section translate to the information IOCs proposed in the figure below.

During the operational phase information has to be passed on to the authorization service producer who will understand the request and send a response to the MnS producer to validate and further carry out the next steps in the operation. In order to enable this we need to have the identity and permission information modelled in an authorisation server.

Figure X.2.1-y Information architecture between different entities

The above diagram depicts the flow of information on a high level and where the information related to access control resides. The main aspects are depicted are below:

- The access control related information has to be first provisioned in the authorization server during the commissioning phase. The operations for provisioning are related to the IOCs that are designed and created for storing the identity, role and permissions for various MnS consumers.

- During the operational phase the MnS consumer will interact with the authorization server in order to check for access control before carrying out the normal operations. Here the normal operations refer to the component A interacting with component B or component C.

- After the authorization server sends the information to the MnS producer the validation related to access control takes place and the operations goes forward if valid else is failed due to an access control issue.

The remaining of the document depicts the IOCs responsible for access control and the possible operations executed on them by the MnS consumer.

End of changes

### X.2.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for the concept of the role based access control. This clause provides an overview of the relationships between relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

In order to realise the use cases for authentication and authorization, the need is to model the required IOCs and the relevant operations which utilise the IOCs. The operations supported on the IOCs are those supported by the generic management services.

As mentioned in the usecases the IOCs are provisioned during the integration time by a network operator. The data is sent to an authentication and authorization service producer.

Post this during integration time when a MnS consumer invokes an operation on the MnS producer, the authentication and authorization service producer validates the action on the resource to enable the decision for the MnS producer. The decision could be that the MnS producer allows or disallows the action on the resource.

Below are the IOCs that need to interact in order to store the data required for the authentication and authorization operations with respect to role based access control.

Figure X.2.1-1 NRM for role based access control name containment

The IOCs related to Identity, Role,  PermissionForMnSs are name contained within the Subnetwork or ManagedFunction or ManagedElement.

@startuml for S5-235513

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class Identity <<InformationObjectClass>>

class Role <<InformationObjectClass>>

class PermissionForMnSs <<InformationObjectClass>>

class Permission <<DataType>>

class ComponentBAttributePermission <<DataType>>

Identity "1" o-down-> "*" Role

Role "1" o-down-> "*"  PermissionForMnSs

PermissionForMnSs "1" o-down-> "*"  Permission

Permission "1" o-down-> "*" ComponentBAttributePermission

@enduml

Figure X.2.1-1 NRM for role based access control relationship

Editor’s note: access right, for particular consumer, pertaining to attributes of Component B will differ. How to model this functionality is FFS

### X.2.2	Inheritance

This subclause depicts the inheritance relationships. The IOCs are derived from the TOP class

Figure X.2.2-1 Inheritance diagram for role based access control

## X.3	Class definitions

### X.3.1	Identity

#### X.3.1.1	Definition

This IOC represents an identity of a MnS consumer.

It is used for authentication and authorization. It can be name-contained by SubNetwork or Managed Function or a Managed Element.

The MnS consumer can be a human or a machine user. This IOC enables the creation and storage of an identity of a MnS consumer. The information in this IOC is the starting point for a MnS consumer to identity who it is. This is validated against an authentication service producer.

For the authentication operation to take place the identity related information has to be provisioned into the system by a network operator who could be an administrator. The administrator adds the identityType attribute and identityName attribute which characterizes a machine user or human user respectively. For example, we could have an identity like a tenant mapped to the relevant list of roles.

Attribute credential is used to provide information for the credential used together with identity when requesting authentication. The examples of credential are password, certificate, biometric, etc.

The roleRefLis attribute defines the role names associated to a particular user.

The IOC stores the details of the expected tasks to be performed by an identity. The tasks are what is to be done on the network management system. To ease the administration on the system, the tasks are organised as roles. The user can be associated to one or more roles.

#### X.3.1.2	Attribute

The Identity IOC includes the attributes inherited from Top IOC (as defined in Generic resource Model) and the following attributes:

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| identityType | M | T | F | F | T |
| identityName | M | T | F | F | F |
| credential | O | T | T | F | T |
| Attributes related to role |  |  |  |  |  |
| roleRefList | M | T | F | F | T |

#### X.3.1.3	Attribute constraints

None

#### X.3.1.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

### X.3.2	Role

#### X.3.2.1	Definition

The Role IOC represents a task or collection of tasks in a network management system.

The Role can be name contained under the Subnetwork or Managed Function or a Managed Element.

The Role IOC enables the storage of information as to what resources and actions an identity can work upon. This IOC maintains the static resources that are known to the management system during integration time. This contains all the granular level resources and the corresponding actions.

The permissionsForMnSs attribute contains a list of PermissionForMnSs to contain the list of granular permission sets.

#### X.3.2.2	Attribute

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| roleName | M | T | F | F | T |
| Attribute related to role |  |  |  |  |  |
| permissionsForMnSsList | M | T | F | F | T |

#### X.3.2.3	Attribute constraints

None

#### X.3.2.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

### X.3.3	PermissionForMnSs

#### X.3.3.1	Definition

The PermissionForMnSs IOC represents the granular resource and actions in a network management system.

The PermissionForMnSs can be name contained under the Subnetwork or Managed Function or a Managed Element

This IOC enables the storage of the resource types in the system and the possible actions that are allowed on it. The permutations and combinations of these permissions are assigned to a role.

The mnsType attribute is the type of the management service as defined TS 28.622.

The permissionList attribute contains a list of permissions.

#### X.3.3.2	Attribute

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mnsType | M | T | T | F | F |
| Attribute related to role |  |  |  |  |  |
| permissionList | M | T | T | F | F |

#### X.3.3.3	Attribute constraints

None

#### X.3.3.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

### X.3.4	Permission <<dataType>>

#### X.3.4.1	Definition

The Permission data type represents the object level and granular attribute level resource and corresponding actions in a network management system.

The attribute componentA specifies an operation on a management service or an NRM.

The attribute ComponentBObjectClass specifies a NRM which is the protected resource for access control. Further attribute level access control would be specified by the attribute componentBAttributePermissionList which contains the list of permissions for the attributes.

The attribute componentC specifies the resource related to the alarm or performance. The attribute componentC specifies for example the performance management data or KPI.

#### X.3.4.2	Attribute

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| componentA | M | T | F | F | F |
| componentBObjectClass | M | T | F | F | T |
| componentBAttributePermissionList | O | T | F | F | T |
| componentC | O | T | F | F | T |

#### X.3.4.3	Attribute constraints

None

#### X.3.4.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

### X.3.5	ComponentBAttributePermission <<dataType>>

#### X.3.4.1	Definition

The ComponentBAttributePermission data type represents the attribute level resource and corresponding actions in a network management system.

The attribute componentBAttributeName specifies an attribute name related to the NRM.

The attribute componentBAttributeOperation specifies the operation on the attribute of the NRM this is part of. For example the operation could be read only or writable.

#### X.3.4.2	Attribute

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| componentBAttributeName | M | T | F | F | F |
| componentBAttributeOperation | O | T | F | F | T |

#### X.3.4.3	Attribute constraints

None

#### X.3.4.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

## X.4	Attribute definitions

### X.4.1	Attribute properties

The following table defines the properties of attributes specified in the present document.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| identityType | This defines a readable string to uniquely represent an identity AllowedValues: username, email address, phone number, IP address, machineuser | type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| identityName | This indicates a type of identifier, AllowedValues: NA | type: String multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| credential | The credential of an MnS consumer or producer used for authentication with authentication service producer. It could be password, certificate, key, pass phrase, etc., based on authentication protocol and factor. AllowedValues: NA | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: No value isNullable: False |
| roleRefList | This defines the list of roles associated with an identity AllowedValues: NA | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| roleName | This string defines the name of a role AllowedValues: NA | type: String multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| mnsType | This attribute defines the type of management service AllowedValues: ProvMnS, FaultSupervisionMnS, StreamingDataReportMnS, FileDataReportingMnS | type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| permissionsForMnSsList | This defines the list of permissions for MnSs which constitue the resources and corresponding actions AllowedValues: NA | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| permissionList | This defines the list permissions for resources and corresponding actions for static objects AllowedValues: NA | type: Permission multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| componentA | This defines the operation of the MnS as defined in 28.532 AllowedValues: NA | type: string multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| componentBObjectClass | This attribute defines a MnS component type B represented by information models of managed entities. A MnS component type B is also called Network Resource Model (NRM). MnS component type B examples are: 1) Network resource models as defined in TS 28.622 2) Network resource models as defined in TS 28.541 AllowedValues: eg: ManagedNFService, HeartbeatControl, NtfSubscriptionControl, AlarmList, TraceJob, PerfMetricJob, | type: ENUM multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| componentBAttributePermissionList | This defines the list of componentBAttributePermissions which contains the IOC attribute and corresponding actions AllowedValues: NA | type: componentBAttributePermissions multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| componentC | This attribute defines a MnS component type C which is the performance management data, KPIs of the managed entity and fault information of the managed entity. The following are examples of Management service component type C: 1. Alarm information as defined in TS 28.532 and TS 28.545 2. Performance data as defined in TS 28.552 , TS 28.554 and TS 32.425. AllowedValues: NA | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| componentBAttributeName | This attribute defines a attribute of a MnS component type B represented by information models of managed entities. A MnS component type B is also called Network Resource Model (NRM). Attributes of MnS component type B examples are: 1) Network resource models as defined in TS 28.622 2) Network resource models as defined in TS 28.541 AllowedValues: NA | type: string multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| componentBAttributeOperation | This indicates which operation is specified on the particular component B attribute AllowedValues: Read-only, Writable | type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |

## X.5	Common Notifications

### X.5.1	Access control notifications

The below CM notifications are valid for the access control related IOCs.

### 4.5.2	Configuration notifications

This clause presents a list of notifications, defined inTS 28.532, that a MnS consumer can receive. The notification header attribute objectClass/objectInstance, defined in TS 32.302, captures the DN of an instance of an IOC defined in the present document.

| Name | S | Notes |
| --- | --- | --- |
| notifyMOIObjectCreation | O |  |
| notifyMOIObjectDeletion | O |  |
| notifyMOIAttributeValueChanges | O |  |
| notifyMOIChanges | O |  |
| notifyEvent | O |  |

## Y.x	Access Control management service

### Y.x.1	Operations

#### Y.x.1.1	authentication operation (CM)

##### Y.x.1.1.1	Description

This operation is invoked by MnS consumer to request the authentication service producer to authenticate MnS consumer. MnS consumer and authentication service producer shall use one of the following methods for authentication:

-	Mutual authentication between MnS consumer and authentication service producer provided by the transport layer protection solution.

-	Client credentials assertion based authentication.

When client credentials assertion based authentication is used, the operation is invoked by a MnS consumer to request authentication by authentication service producer. The authentication service producer gets the identifier and credential of the MnS consumer for validation. Based on identifier in the request, authentication service producer gets identity information, e.g. credential of the identity, associated role(s) of the identity, etc., from local data store (e.g., a directory). The authentication service producer authenticates the MnS consumer by validating the identity information. associated to the role(s) the MnS consumer belongs to.

If authentication is successful, the authentication service producer sends successful response with assertion to the MnS consumer. MnS producer validates the assertion to authenticate a MnS consumer.

If fail for authentication, the authentication service producer sends failure response to the MnS consumer.

##### Y.x.1.1.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| identityName | M | String | This parameter specifies the identity of the MnS consumer that is to be authenticate. |
| attributeListIn | M | LIST OF SEQUENCE< attribute name, attribute value> | This parameter may have a null value. When this parameter is supplied, it contains a list of name/value pairs specifying attribute identifiers and their values to be assigned to the identity to be authenticated. |

##### Y.x.1.1.3	Output parameters

| Parameter name | S | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListOut | M | LIST OF SEQUENCE< attribute name, attribute value> | This list of name/value pairs contains the attributes of the authenticated MnS consumer and the actual value assigned to each. |
| Status | M | ENUM (OperationSucceeded, OperationFailed) |  |

###### Y.x.1.1.4	Exceptions and constraints

| Exception Name | Definition |
| --- | --- |
| operation_failed | Condition: Operation is failed Returned Information: The output parameter status Exit state: Entry State |

#### Y.x.1.2	authorization operation (M)

##### Y.x.1.2.1	Definition

The operation is used to authorize a MnS consumer by the authorization service producer.

MnS consumer, MnS producer and authorization service producer shall use one of the following methods for authorization:

-

-    Token based authorization framework including various grant modes (e.g., as specified in RFC 6749 [xx])

-	Static authorization as specified (e.g.,. in RFC 8341 for NACM [yy])

When token based authorization framework is used, authorization service producer receives authorization request, gets the assertion of the MnS consumer related to the authentication and probably other context information (e.g. resource , action) from the request. If access token is supported by the MnS producer and consumer:

-  The MnS consumer gets access token from authorization service producer by providing the assertion from authentication session.

-  The MnS consumer starts normal operation with MnS producer with the access token. The MnS producer validates the token. If the token is valid, the MnS producer performs the action(operation) on the resource authorized and returns result to the MnS consumer.

If access token is not supported by the MnS producer and consumer:

-  When normal operation starts, the MnS producer validate the authentication assertion and check permission of the MnS consumer with authorization service producer.

-  The MnS producer performs the operation on the targeted resource which is part of the authorized scope and returns result to the consumer if the MnS request is allowed according to permissions.

If fail for authorization, the authorization service producer sends failure response to the consumer.

##### Y.x.1.2.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListIn | M | LIST OF SEQUENCE< attribute name, attribute value> | This parameter may have a null value. When this parameter is supplied, it contains a list of name/value pairs specifying attribute identifiers and their values to be assigned to the identity to be authorized. |

##### Y.x.1.2.3	Output parameters

| Parameter name | S | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListOut | M | LIST OF SEQUENCE< attribute name, attribute value> | This list of name/value pairs contains the attributes of the authorization for the MnS consumer and the actual value assigned to each. |
| Status | M | ENUM (OperationSucceeded, OperationFailed) |  |

###### Y.x.1.1.4	Exceptions and constraints

| Exception Name | Definition |
| --- | --- |
| operation_failed | Condition: Operation is failed Returned Information: The output parameter status Exit state: Entry State |

# Annex Z (Informative):

# Z.1	Informative sequence diagram: Commissioning flow to be carried out by an administrator:

The sequence diagram below explains the IOCs to be used as a pre-requisite for enabling the role based access control. The notes above in the diagram also depict an example of each of the tasks.

The sequence diagram below explains the operations and the interactions with the authentication and authorization service which typically should suit any implementation be in oAUTH or a NACM implementation.

The notes along the flows show the examples related to the IOCs provisioning operations.

Figure Z.1 - Sequence of commissioning for role based access control

## Commissioning usecases to be carried out by an administrator

- To carry out authentication and authorization based on role based access a set of tasks needs to be in place at the commissioning phase.

The network equipment vendor will typically take care of the below:

    - The granularity level at which the management objects need to be authorized

    - The actions that need to be permitted on these granular level objects

This is typically carried out by an administrator who has to setup the system for operational purposes which included authentication and authorization for every operation. Hence the administrator will need to have a good view of the permissions of the devices being integrated and managed. The administrator will need to know the possible scope of action for various users in the system on the instances that are created. The administrator will need to know the roles that need to be created in the system as well as the mapping of the same to the users. Below is the detail of every task.

The below elaborates the sequence flow to depict the IOCs involved for provisioning.

### a. Permissions with resources and actions

- Permissions are combinations of static resources and actions. The network equipment vendor can introduce the resources and actions at various levels of granularity as well as in the language that can be suitable for the vendor. When the commissioning phase happens it is important to translate these permissions to the authentication and authorization system so that it is aware of the static resources and corresponding actions on them for access control.

Hence the IOC for PermissionForMnSs is required to create the necessary data.

### b. Role creation with association to permissions and conditions

The meaningful task is the role which defines the responsibility realm for the user. Hence the role contains the overall distribution of resources and actions it can work on by resolving the condition and permissions. The static resources identified need to be combined with the dynamically created instances of the resource in order to be assigned to a meaningful task in the system.

The roles are typically created according to the convenience and hierarchy of the organisation.  Coarse grained or fine grained permissions can be created with the help of roles.

The management system should be setup for access control by a system administrator who will have the know how of creating the required users and roles. Roles will use the various possibilities to map resources and actions.

Hence the IOC for Role is required to create the necessary data related to various roles in the system.

### c. Identity to role assignment

The identity or user of the system needs to be assigned the responsibilities to carry out the OAM operations. Hence the set of responsibilities are defined in various roles. Assignment of the roles to an identity is required to make the administrative task more efficient and flexible. This also shields the administrator from getting into the details of the know how of granular resources and actions relevant to it.

Hence the IOC for Identity is required to create the necessary data.

# Z.2 Informative sequence diagram: Authentication and Authorization with role based access control:

Post the above tasks we have the possibility to have role-based access in operation with every service call being authenticated and authorized.

The below sequence diagram shows how the operations are invoked from a consumer towards an authentication and authorization service producer.

When an MnS is invoked, the MnS consumer authenticates towards an authentication service producer. This is then followed by checking the access rights with respect to the role-based access. This takes place in the authorization service producer which is invoked from the MnS producer to check if its resources can be accessed with respect to the related operation.

Hence then need of a authentication and authorization service producer which will validate a user who want to access the resources as well as validates what the user can access.

Figure Z.2 - Sequence diagram for operations of authentication and authorization

End of changes