3GPP TSG-SA5 Meeting #151 	S5-236774

Xiamen, China, 9 - 13 October 2023	(revision of xx-yyxxxx)

# Source:	China Telecom

# Title:	New WID on management aspects of 5G system with satellite backhaul

# Document for:	Approval

# Agenda Item:	6.2.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Management aspects of 5G system with satellite backhaul

Acronym: OAM_5GSATB

Unique identifier:

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
| X | Normative – Stage 1 |
| X | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 20035 | 5G system with satellite backhaul | SA1 Rel-18 WID on requirements of supporting satellite backhaul in 5GS. |
| 890034 | Integration of satellite components in the 5G architecture | SA2 Rel-17 WID for 5GS architecture using satellite access, which supports satellite backhaul in 5GS. |
| 940060 | Study on 5G System with Satellite Backhaul | SA2 Rel-18 SID for 5G System using different backhaul connections and the potential enhanced support for UPF deployed on GEO satellites. (TR 23.700-27) |
| 970056 | 5G system with satellite backhaul | SA2 Rel-18 WID for the support of satellite backhaul |

3	Justification

Since satellites can provide mobile broadband access coverage, they are used by MNOs to provide backhaul service for gNBs in fringe areas (e.g. remote rural areas) and in cases of emergency or temporary measure (e.g. a disaster area or in place of a microwave link whilst waiting for licence approval).

As indicated by GSMA, the satellite backhaul is used in developing markets and as a complementary role in developed markets.

In SA1, the requirements related to 5G system with satellite backhaul are documented in TS 22.261 [1], which have been described as:

The 5G system with satellite access shall support the use of satellite links between the radio access network and core network, by enhancing the 3GPP system to handle the latencies introduced by satellite backhaul.

In SA2 Rel-17 and Rel-18, the architecture enhancements related to 5G system with satellite backhaul and UPF onboard have been addressed the following scenarios, including:

-	Reporting of satellite backhaul

-	QoS monitoring when dynamic Satellite Backhaul is used

-	Edge computing via UPF deployed on satellite

-	Local switch for UE-Satellite-UE communications via UPF deployed on satellite

And SA5 has not begun related studies so far. Although the previous SA5 study FS_5GSAT_MO investigated the orchestration of satellite transport network, the related TR 28.808 has not yet been developed into TS. So in SA5, it does not include the management of satellite backhaul or the UPF onboard.

4	Objective

The objective of the work item are as follows:

WoP#1: Specify the concept, use cases, requirements and solutions for 3GPP management system to support 5G system with satellite backhaul, especially when UPF is deployed on satellite.

WoP#2: Specify potential new configuration that support 5G system with satellite backhaul architecture:

-	Dynamic configuration of satellite backhaul type, e.g., AMFFunction extension

-	Configuration of UPF deployed on satellite, e.g., UPFFunction extension

-	Configuration that support edge computing via UPF deployed on satellite, e.g., DNAI extension

-	Configuration that support local switch via UPF deployed on satellite, e.g., allocating/releasing N19 tunnel resources

WoP#3: Specifying potential new configuration and performance measurements related to Qos control, especially when dynamic Satellite Backhaul is used.

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | SA Dependency (Yes/No/Maybe) |
| --- | --- | --- | --- | --- |
| WoP#1 | 0 | 0.2 | No | No |
| WoP#2 | 0 | 0.6 | No | No |
| WoP#3 | 0 | 0.2 | No | No |

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 28.541 | Enhancement of 5G NRMs for 5G system with satellite backhaul | SA#104 Jun 2024 |  |
| 28.552 | Update performance measurements for 5G system with satellite backhaul | SA#104 Jun 2024 |  |
| 28.533 | Enhancement of 5G management architecture framework for 5G system with satellite backhaul | SA#104 Jun 2024 |  |

6	Work item Rapporteur(s)

7	Work item leadership

SA5

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

China Telecom