# 3GPP TSG-SA5 Meeting #151	S5-236841

# Xiamen, China, October 9-13, 2023

# Source:	Nokia

# Title:	Rel-18 pCR 28.318 Annex A Procedures

# Document for:	Approval

Agenda Item:	6.6.4.1 (NSOEU_WoP#1) General

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

This pCR is motivated by reference to preceding work relevant to the feature.

[1] 3GPP TR 28.829, "Study on network and service operations for energy utilities".

# 3	Rationale

The method of using PerfMetricJob, ThresholdMonitor & NtfSubscriptionControl has the critical problem of knowing how to scope MOIs for cells in a given geographic location. ManagementDataCollection gracefully handles this problem.

# 4	Detailed proposal

It is proposed to agree to the following change to TS 28.318, 0.0.0.

Begin Change

# Annex	<Y> (informative):	Procedures

# A.2	ManagementDataCollection: Exposing performance and prediction information for high availability

Y.2.1	DSO is trusted but not supported by network operator

Y.2.1.1	General

In this basic scenario the DSO consumer, is recognized by the network operator as a trusted entity that is allowed to read from and write to the management system.

Y.2.1.2	Signal flow

The DSO consumer wants to receive performance measurement reports from one or more specific cell(s). The DSO consumer is in possession of the following information:

the identity of the cell site(s): base station ID, or

the estimated location(s) of the cell site antenna(s)

Figure 2.1.2-1

- ManagementDataCollection

- The DSO consumer sends createMOI to the producer with the following parameters: managedObjectClass = ManagementDataCollection, managedObjectInstance = {DN of the producer}, attributeListIn as defined in the following table:

| Attribute name | Value |
| --- | --- |
| managementData | DRB.AirIfDelayDL_{plmnid}, DRB.AirIfDelayUl_{plmnid}, DRB.UEThpDL, DRB.UEThpDL.QOS, DRB.UEThpDLSNSSAI, DRB.UEThpDL.PLMN, DRB.UEThpUL, DRB.UEThpUL.QOS, DRB.UEThpULSNSSAI, DRB.UEThpUL.PLMN, DRB.PacketLossRateUL, DRB.PacketLossRateUL.QOS, DRB.PacketLossRateUL.SNSSAI cell/network availability |
| targetNodeFilter | - |
| areaOfInterest | 3..N polygon points OR list of NR Cell IDs |
| networkDomain | RAN |
| cpUpType | UP |
| sst | eMBB |
| collectionTimeWindow | startTime, endTime |
| reportingCtrl | fileReportingPeriod = <integer, multiple of granularity period, chosen by DSO> |
| dataScope | 5QI |

targetNodeFilter
The area of interest is defined by the area inside a convex polygon. Performance measurements shall be collected from cells having the antenna inside the polygon. The corners of the polygon are defined by latitude, longitude and altitude. Alternatively, the DSO my provide a list of NR Cell IDs that identify the specific cell which will provide the performance measurements.
The networkDomain is RAN, specifying that performance measurements are collected from the radio access network.
The cpUpType is UP, specifying that performance measurements are collected from user plane traffic.
The sst is eMBB, specifying that performance measurements are collected from eMBB traffic.

collectionTimeWindow
Specifies the start time and end time for the collection of performance measurements. This is selected by the DSO.

In this example, report files shall be stored on MnS producer and, on condition that an appropriate subscription is in place, inform the MnS consumer about the availability of new files and the file location using the notifyFileReady notification. Other reporting configurations are possible according to attribute settings.

dataScope
Specifies whether the required data is reported per S-NSSAI or per 5QI.

- The producer creates one or more “PerfMetricJob” data collection job(s) to collect the performance measurements.

- The producer sends createMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| attributeListOut | name / value pairs of the attributes of the new object |
| status | OperationSucceeded ¦ OperationFailed |

The DSO consumer stores the attribute name/value pairs.

Metric production is now active on the in-scope object instances whose object class matches the object class associated to the specified performance metrics.

Repeate steps 1 - 3 as needed for each required geographic area OR list of NR Cell IDs where performance metric information is wanted.

- ThresholdMonitor

- The DSO consumer sends createMOI for ThresholdMonitor to check for threshold crossings of performance measurements including measurement for latency, packet loss, throughput and cell/network availability, using the follows parameters: managedObjectClass = ThresholdMonitor, managedObjectInstance = {DN of the producer}, attributeListIn as defined in the following table:

| Attribute name | Value |
| --- | --- |
| administrativeState | - |
| operationalState | - |
| thresholdInfoList | list of ThesholdInfo <<datatype>>, one for each performance metric. chosen by DSO |
| monitorGranularityPeriod | <integer, defined in seconds, multiple of granularity period from PMJ MOI, chosen by DSO> |
| objectInstances | - |
| rootObjectInstances | - |

In this example, a ThresholdInfo datatype will be included for each performance metric that was included in the creation of the corresponding ManagementDataCollection MOI.

ThresholdInfo will also contain an attribute for the location (Lat/long, TAC, cellid). This is used to scope the object instance to be monitored.

- The producer creates “ThresholdMonitor” data collection job.

- The producer sends createMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| attributeListOut | name / value pairs of the attributes of the new object |
| status | OperationSucceeded ¦ OperationFailed |

The DSO consumer stores the attribute name/value pairs.

- NtfSubscriptionControl

- The DSO consumer sends createMOI for NtfSubscriptionControl to subscribe for the required notifcations with the following parameters: managedObjectClass = NtfSubscriptionControl, managedObjectInstance = {DN of the producer}, attributeListIn as defined in the following table:

| Attribute name | Value |
| --- | --- |
| notificationRecipientAddress | <string, address of the notification recipient>. |
| notificationTypes | notifyFileReady, notifyFilePreparationError, notifyThresholdCrossing |
| scope | included if supplied by the supporting site operator |
| notificationFilter | - |

The address of the notification recipient is provided by the DSO. In this example, three notification types are allowed and none are filtered.

- The producer creates “NtfSubscriptionControl” job.

- The producer sends createMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| attributeListOut | name / value pairs of the attributes of the new object |
| status | OperationSucceeded ¦ OperationFailed |

The DSO consumer stores the attribute name/value pairs.

Repeate steps 7 - 9 for each required MOI where threshold monitoring is required.

End of Changes