3GPP TSG SA WG5 Meeting 151		S5-237043

# Xiamen, China, 9th Oct 2023 - 13th Oct 2023                                   revision of S5-236258

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.622 | CR | - | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 dCR 28.622 Remove FM related parts |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBMA |  |  |  |  |  | Date: |  |  | 2023-09-25 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As the definition of the fault management Mns is moved to the new TS 28.111, FM related parts should be removed from this specification as endorsed in S5-234507 DP on FM restructuring of TS 28-545. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove - 2 References – Reference only used by FM - 4.1 Imported information entities and local labels – AlarmRecord removed other imports updated to new TS number - 4.2 Class diagrams – AlarmList IOC - 4.3.26 AlarmList IOC - 4.3.27 AlarmRecord datatype - 4.4.1 Remove alarm related attrbutes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Duplicate and slightly conflicting definition of fault management in multiple specification would lead to confusion and interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.1, 4.2, 4.2.1, 4.2.2, 4.3.26, 4.3.27, 4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS/TR ... CR ... TS 28.111, CR for 28.623 |  |  |
| Other comments: |  | A real CR based on this draftCR should be agreed/approved together with the TS 28.111. An associated draftCR and later CR for 28.623 is planned to remove the stage 3 definitions from the same items. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 32.302: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP): Information Service (IS)".

[4]	3GPP TS 32.150: "Telecommunication management; Integration Reference Point (IRP) Concept and Definitions".

[5]	3GPP TS 23.003: "Technical Specification Group Core Network and Terminals; Numbering, addressing and identification"

[6]	 Void

[7]	ITU-T Recommendation X.710 (1991): "Common Management Information Service Definition for CCITT Applications".

[8]	TS 32.107: "Telecommunication management; Fixed Mobile Convergence (FMC) Federated Network Information Model (FNIM)"

[9]	TS 28.620: "Telecommunication management; Fixed Mobile Convergence (FMC) Federated Network Information Model (FNIM) Umbrella Information Model (UIM)"

[10]	TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) Model Repertoire"

[11]	 Void

[12]	 Void

[13]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[14]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[15]	ETSI GS NFV 003 V1.1.1: "Network Functions Virtualisation (NFV); Terminology for Main Concepts in NFV".

[16] 	ETSI GS NFV-IFA 008 v2.1.1: "Network Functions Virtualisation (NFV); Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

[17]	ETSI GS NFV-IFA 015 v2.1.2: "Network Functions Virtualisation (NFV); Management and Orchestration; Report on NFV Information Model".

[18]	ETSI ES 202 336-12 V1.1.1: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[19]	ITU-T Recommendation X.731: "Information technology - Open Systems Interconnection - Systems Management: State management function".

[20]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[21]	3GPP TS 28.625: "State Management Data Definition Integration Reference Point (IRP); Information Service (IS) ".

[22]	3GPP TS 23.501: "System Architecture for the 5G System".

[23]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[24]	IETF RFC 791: "Internet Protocol".

[25]	IETF RFC 2373: "IP Version 6 Addressing Architecture".

[26]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[27]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[28]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[29]	3GPP TS 32.421: "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements".

[30]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[31]	Void

[32]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[33]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[34]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[35]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[36]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[37]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[38]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[39]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[40]	3GPP TS 25.321: "Medium Access Control (MAC) protocol specification".

[41]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol specification".

[42]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[43]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[44]	3GPP TS 28.705: "Telecommunication management; IP Multimedia Subsystem (IMS) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[45]	3GPP TS 28.702: "Telecommunication management; Core Network (CN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[46]	3GPP TS 28.652: "Telecommunication management; Universal Terrestrial Radio Access Network (UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[47]	3GPP TS 28.708: "Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[48]	3GPP TS 28.541: " Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[49]	IETF RFC 8089: "The "file" URI Scheme".

[50]	3GPP TS 28.405: "Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration".

[51]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[52]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[53]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[X]	3GPP TS 28.111: "Fault management"

Next change

## 4.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| 3GPP TS 28.532 [27], notification, notifyMOICreation | notifyMOICreation |
| 3GPP TS 28.532 [27], notification, notifyMOIDeletion | notifyMOIDeletion |
| 3GPP TS 28.532 [27], notification, notifyMOIAttributeValueChanges | notifyMOIAttributeValueChanges |
| 3GPP TS 28.532 [27], notification, notifyMOIChanges | notifyMOIChanges |
| 3GPP TS 28.111 [X], notification, notifyNewAlarm | notifyNewAlarm |
| 3GPP TS 28.111 [X], notification, notifyClearedAlarm | notifyClearedAlarm |
| 3GPP TS 28.111 [X], notification, notifyChangedAlarm | notifyChangedAlarm |
| 3GPP TS 28.111 [X], notification, notifyChangedAlarmGeneral | notifyChangedAlarmGeneral |
| 3GPP TS 28.111 [X], notification, notifyCorrelatedNotificationChanged | notifyCorrelatedNotificationChanged |
| 3GPP TS 28.111 [X], notification, notifyAckStateChanged | notifyAckStateChanged |
| 3GPP TS 28.111 [X], notification, notifyComments | notifyComments |
| 3GPP TS 28.111 [X], notification, notifyPotentialFaultyAlarmlist | notifyPotentialFaultyAlarmList |
| 3GPP TS 28.111 [X], notification, notifyAlarmlistRebuilt | notifyAlarmListRebuilt |
| 3GPP TS 28.111 [X], notification, notifyFileReady | notifyFileReady |
| 3GPP TS 28.532 [27], notification, notifyFilePreparationError | notifyFilePreparationError |
| 3GPP TS 28.620 [9], IOC, Domain_ | Domain_ |
| 3GPP TS 28.620 [9], IOC, ManagedElement_ | ManagedElement_ |
| 3GPP TS 28.620 [9], IOC, Function_ | Function_ |
| 3GPP TS 28.620 [9], IOC, ManagementSystem_ | ManagementSystem_ |
| 3GPP TS 28.620 [9], IOC, TopologicalLink_ | TopologicalLink_ |
| 3GPP TS 28.620 [9], IOC, Top_ | Top_ |
| 3GPP TS 28.541 [48], dataType, S-NSSAI | S-NSSAI |

## 4.2	Class diagrams

### 4.2.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for this IRP. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

The following figure shows the containment/naming hierarchy and the associations of the classes defined in the present document. See Annex A of a class diagram that combines this figure with Figure 1 of [2], the class diagram of UIM.

NOTE 1:	ManagedElement may be contained either

-	in a SubNetwork (since SubNetwork inherits from Domain_ and ManagedElement inherits from ManagedElement_ and Domain_ name-contained ManagedElement_ as observed in the figure of Annex A) or

-	in a MeContext instance as observed by the above figure or in the figure of Annex A.

This either-or relation cannot be shown by using an {xor} constraint in the above figure.

ManagedElement may also have no parent instance at all.

NOTE 2:	Void

NOTE 3:	If the configuration contains several instances of SubNetwork, exactly one SubNetwork instance shall directly or indirectly contain all the other SubNetwork instances.

NOTE 4:	The SubNetwork instance not contained in any other instance of SubNetwork is referred to as "the root SubNetwork instance".

NOTE 5:	ManagementNode shall be contained in the root SubNetwork instance.

NOTE 6:	If contained in a SubNetwork instance, MnsAgent shall be contained in the root SubNetwork instance.

NOTE 7:	Void

NOTE 8:	Void

# Figure 4.2.1-1: NRM fragment

Each Managed Object is identified with a Distinguished Name (DN) according to 3GPP TS 32.300 [13] that expresses its containment hierarchy. As an example, the DN of a ManagedElement instance could have a format like:

SubNetwork=Sweden,MeContext=MEC-Gbg-1,ManagedElement=RNC-Gbg-1.

NOTE 8:	Void

NOTE 9:	Void

Figure 4.2.1-2: Vendor specific data container NRM fragment

Figure 4.2.1-3: PM control NRM fragment

Figure 4.2.1-4: Threshold monitoring control NRM fragment

Figure 4.2.1-5: Notification subscription and heartbeat notification control NRM fragment

Figure 4.2.1-6: Void

Figure 4.2.1-7: Trace control NRM fragment

Figure 4.2.1-8: MnS Registry NRM fragment

Figure 4.2.1-9: File retrieval NRM fragment

Figure 4.2.1-10: File download NRM fragment

Figure 4.2.1-11: Management data collection NRM fragment

Figure 4.2.1-12: QoE Measurement Collection NRM fragment

Figure 4.2.1-A: SupportedNotifications NRM fragment

### 4.2.2	Inheritance

# This clause depicts the inheritance relationships.

# Figure 4.2.2-1: NRM fragment

# Figure 4.2.2-2: PM control NRM fragment

# Figure 4.2.2-3: Threshold monitoring control NRM fragment

# Figure 4.2.2-4: Notification subscription and heartbeat notification control NRM fragment

Figure 4.2.2-5: Void

Figure 4.2.2-6: Trace control NRM fragment

Figure 4.2.2-7: MnS Registry NRM fragment

Figure 4.2.2-8: File retrieval NRM fragment

Figure 4.2.2-9: File download NRM fragment

Figure 4.2.2-10: Management data collection NRM fragment

Figure 4.2.2-11: QoE Measurement Collection NRM fragment

Figure 4.2.2-12: SupportedNotifications NRM fragment

Next change

### 4.3.26	Void

### Editors note: When this draftCR is transformed to a real CR, all text in this clause shall be removed.

### 4.3.27	Void

Editors note: When this draftCR is transformed to a real CR, all text in this clause shall be removed.

Next change

### 4.4.1	Attribute properties

The following table defines the properties of attributes specified in the present document.

Editors note: When this draftCR is transformed to a real CR, all the full table shall be included, not just the rows to be removed..

End of  changes