3GPP TSG-SA5 Meeting #151 	S5-237133

Xiamen, China, 9 - 13 October 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | 0061 | rev | 1 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR TS 28.313 correction performance measurements for CHO and DAPS MRO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-09-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Change the the chapter numbers in the performance measurement which are not aligned with those in TS 28.552. The performance measurement of “number of failed conditional handover executions” has been already removed from version 17.6.0 in TS 28.552, so remove the performance measurement “number of failed conditional handover executions” to align with the TS 28.552. Some table numbers still contain x. Change them to correct numbers. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the chapter numbers of the performance measurements. Remove the performance measurement “number of failed conditional handover executions”. Change the “x” in the table number to correct number. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It would be unclear for the consumer to monitoring the performance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.5.3.1, 7.1.6.3.1, 7.1.7.3.1, 7.2.2.2.1, 7.2.3.1, 7.2.3.3.1, 7.2.4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

7.1.5.3	MnS Component Type C definition

7.1.5.3.1	Performance measurements

Performance measurements related LBO are captured in Table 7.1.5.3.1-1:

Table 7.1.5.3.1-1. D-LBO related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| DL Total PRB Usage | This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the downlink (see clause 5.1.1.2.1 in TS 28.552 [5]). |  |
| UL Total PRB Usage | This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the uplink (see clause 5.1.1.2.2 in TS 28.552 [5]). |  |
| Distribution of DL Total PRB Usage | This distribution measurement is to monitor when a cell may experience overload situation in the downlink (see clause 5.1.1.2.3 in TS 28.552 [5]). |  |
| Distribution of UL Total PRB Usage | This distribution measurement is to monitor when a cell may experience overload situation in the uplink (see clause 5.1.1.2.4 in TS 28.552 [5]). |  |
| DL PRB used for data traffic | This measurement provides the number of physical resource blocks (PRBs) in average used in downlink for data traffic (see clause 5.1.1.2.5 in TS 28.552 [5]). |  |
| UL PRB used for data traffic | This measurement provides the number of physical resource blocks (PRBs) in average used in uplink for data traffic (see clause 5.1.1.2.7 in TS 28.552 [5]). |  |
| Mean number of RRC Connections | This measurement provides the mean number of users in RRC connected mode during the granularity period (see clause 5.1.1.4.1 in TS 28.552 [5]). |  |
| Max number of RRC Connections | This measurement provides the maximum number of users in RRC connected mode during the granularity period (see clause 5.1.1.4.2 in TS 28.552 [5]). |  |
| Mean number of stored inactive RRC Connections | This measurement provides the mean number of users in RRC inactive mode during each granularity period (see clause 5.1.1.4.3 in TS 28.552 [5]). |  |
| Max number of stored inactive RRC Connections | This measurement provides the maximum number of users in RRC inactive mode during each granularity period (see clause 5.1.1.4.4 in TS 28.552 [5]). |  |

Start of Next change

7.1.6.3	MnS Component Type C definition

7.1.6.3.1	Performance measurements

Performance measurements related to MRO for CHO are captured in Table 7.1.6.3.1.-1:

Table 7.1.6.3.1-1. MRO for CHO related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| Number of requested conditional handover preparations | Counts the number of successful and unsuccessful inter-gNB conditional handover preparations sent (see TS 28.552 clause 5.1.1.6.6.1) |  |
| Number of successful conditional handover preparations | Counts the number of unsuccessful inter-gNB conditional handover preparations sent (see TS 28.552 clause 5.1.1.6.6.2) |  |
| Number of failed conditional handover preparations | Counts the number of unsuccessful inter-gNB conditional handover preparations sent (see TS 28.552 clause 5.1.1.6.6.3) |  |
| Number of requested conditional handover resource allocations | Counts the number of successful and unsuccessful inter-gNB conditional handover preparations (see TS 28.552 clause 5.1.1.6.6.4) |  |
| Number of successful conditional handover resource allocations | Counts the number of successful inter-gNB conditional handover preparations (see TS 28.552 clause 5.1.1.6.6.5) |  |
| Number of failed conditional handover resource allocations | Counts the number of unsuccessful inter-gNB conditional handover preparations (see TS 28.552 clause 5.1.1.6.6.6) |  |
| Number of configured conditional handover candidates | Counts the number of outgoing inter-gNB conditional handover candidates requested (see TS 28.552 clause 5.1.1.6.6.7) |  |
| Number of UEs configured with conditional handover. | Counts the number of UEs that has been configured with inter-gNB conditional handover (see TS 28.552 clause 5.1.1.6.6.8) |  |
| Number of successful conditional handover executions | Counts the number of successful inter-gNB conditional handover executions received (see TS 28.552 clause 5.1.1.6.6.9) |  |
| Mean Time of requested conditional handover executions | Counts the mean time of inter-gNB conditional handover executions (see TS 28.552 clause 5.1.1.6.6.11) |  |
| Max Time of requested conditional handover executions | Counts the max time of inter-gNB conditional handover executions (see TS 28.552 clause 5.1.1.6.6.12) |  |
| Number of configured conditional handover candidates | Counts the number of outgoing intra-gNB conditional handover candidates requested (see TS 28.552 clause 5.1.1.6.7.1 |  |
| Number of UEs configured with conditional handover | Countes the the number of UEs that has been configured with conditional handover (see TS 28.552 clause 5.1.1.6.7.2) |  |
| Number of successful conditional handover executions | Counts the number of successful intra-gNB conditional handover executions received (see TS 28.552 clause 5.1.1.6.7.3) |  |
| Number of requested conditional handover preparations | Counts the number of outgoing intra-gNB conditional handover preparations requested, for a split gNB deployment (see TS 28.552 clause 5.1.3.7.1.3) |  |
| Number of successful conditional handover preparations | Countes the number of successful intra-gNB conditional handover preparations, for a split gNB deployment (see TS 28.552 clause 5.1.3.7.1.4) |  |

Start of Next change

7.1.7.3	MnS Component Type C definition

7.1.7.3.1	Performance measurements

Performance measurements related to MRO for DAPS handover are captured in Table 7.1.7.3.1.-1:

Table 7.1.7.3.1-1. MRO for DAPS handover related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| Number of requested DAPS handover preparations | Counts the number of successful and unsuccessful inter-gNB DAPS handover preparations sent (see TS 28.552 clause 5.1.1.6.8.1) |  |
| Number of successful DAPS handover preparations | Counts the number of unsuccessful inter-gNB DAPS handover preparations sent (see TS 28.552 clause 5.1.1.6.8.2) |  |
| Number of failed DAPS handover preparations | Counts the number of unsuccessful inter-gNB DAPS handover preparations sent (see TS 28.552 clause 5.1.1.6.8.3) |  |
| Number of requested DAPS handover resource allocations | Counts the number of successful and unsuccessful inter-gNB DAPS handover preparations (see TS 28.552 clause 5.1.1.6.8.4) |  |
| Number of successful DAPS handover resource allocations | Counts the number of successful inter-gNB DAPS handover preparations (see TS 28.552 clause 5.1.1.6.8.5) |  |
| Number of failed DAPS handover resource allocations | Counts the number of unsuccessful inter-gNB DAPS handover preparations (see TS 28.552 clause 5.1.1.6.8.6) |  |
| Number of requested DAPS handover executions | Counts the number of outgoing inter-gNB DAPS handover candidates requested (see TS 28.552 clause 5.1.1.6.8.7) |  |
| Number of successful DAPS handover executions | Counts the number of successful inter-gNB DAPS handover executions received (see TS 28.552 clause 5.1.1.6.8.8) |  |
| Number of failed DAPS handover executions | Counts the the number of failed inter-gNB DAPS handover executions received (see TS 28.552 clause 5.1.1.6.8.9) |  |
| Number of DAPS handover requested | Counts the number of outgoing intra-gNB DAPS handovers requested (see TS 28.552 clause 5.1.1.6.9.1) |  |
| Number of successful DAPS handovers | Counts the number of successful intra-gNB DAPS handovers (see TS 28.552 clause 5.1.1.6.9.2) |  |
| Number of requested DAPS handover preparations | Counts the number of outgoing intra-gNB DAPS handover preparations requested, for a split gNB deployment (see TS 28.552 clause 5.1.3.7.1.5) |  |
| Number of successful DAPS handover preparations | Countes the number of successful intra-gNB DAPS handover preparations, for a split gNB deployment (see TS 28.552 clause 5.1.3.7.1.6) |  |

Start of Next change

7.2.2.2	MnS Component Type B definition

7.2.2.2.1	Parameters to be updated

Table 7.2.2.2.1-1 defines the RRM related parameters to be updated by C-SON function.

Table 7.2.2.2.1-1: RRM related parameters

| Control parameters | Definition | Legal Values |
| --- | --- | --- |
| Maximum RRM policy ratio | This parameter specifies the maximum percentage of RRM resources used by the network slice instance(s). See attribute rRMPolicyMaxRatio in TS 28.541 [13]. | 0..100 |
| Minimum RRM policy ratio | This parameter specifies the minimum percentage of RRM resources used by the network slice instance(s). See attribute rRMPolicyMinRatio in TS 28.541 [13]. | 0..100 |
| Dedicated RRM policy ratio | This parameter specifies the maximum percentage of RRM resources that are dedicated to the network slice instance(s). See attribute rRMPolicyDedicatedRatio in TS 28.541 [13]. | 0..100 |

Start of Next change

7.2.3	Centralized Capacity and Coverage Optimization

7.2.3.1	MnS component type A

Table 7.2.3.1-1: CCO type A

| MnS Component Type A | Note |
| --- | --- |
| Operations and notifications defined in clause 5 of TS 28.532 [3]: - createMOI operation - getMOIAttributes operation - modifyMOIAttributes operation - deleteMOI operation - notifyMOIAttributeValueChanges operation - notifyMOICreation - notifyMOIDeletion - notifyMOIChanges | It is supported by Provisioning MnS for NF, as defined in TS 28.531 [11]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in TS 28.550 [12]. |

Start of Next change

7.2.3.3	MnS Component Type C definition

7.2.3.3.1	Performance measurements

CCO related performance measurements are captured in Table 7.2.3.3.1-1:

Table 7.2.3.3.1-1. CCO related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| Distribution of SS-RSRP per SSB | Provides the distribution of SS_RSRP per SSB (see clause 5.1.1.22.1 in TS 28.552 [5]). |  |
| Distribution of SS-RSRQ | Provides the distribution of SS_RSRQ (see clause 5.1.1.31.1 in TS 28.552 [5]). |  |
| Distribution of the number of active UE per SSB | Provides the distribution of the number of active UE per SSB. |  |
| Number of requested handover executions | Provides the number of requested handover executions (see clause 5.1.1.6.1.7 in TS 28.552 [5]). |  |
| Number of failed handover executions | Provides the number of failed handover executions (see clause 5.1.1.6.1.9 in TS 28.552 [5]). |  |
| Distribution of DL Total PRB Usage | Provides the distribution of samples with total usage (in percentage) of PRBs on the downlink in different ranges (see clause 5.1.1.2.3 in TS 28.552 [5]). |  |
| Distribution of UL Total PRB Usage | Provides the distribution of samples with total usage (in percentage) of PRBs on the uplink in different ranges (see clause 5.1.1.2.4 in TS 28.552 [5]). |  |
| DL PRB used for data traffic | Provides the number of PRBs in average used in downlink for data traffic (see clause 5.1.1.2.5 in TS 28.552 [5]). |  |
| DL total available PRB | provides the total number of PRBs in average available downlink (see clause 5.1.1.2.6 in TS 28.552 [5]). |  |
| UL PRB used for data traffic | Provides the number of PRBs in average used in uplink for data traffic (see clause 5.1.1.2.7 in TS 28.552 [5]). |  |
| UL total available PRB | Provides the total number of PRBs in average available uplink (see clause 5.1.1.2.8 in TS 28.552 [5]). |  |
| Average DL UE throughput in gNB | Provides the average UE throughput in downlink (see clause 5.1.1.3.1 in TS 28.552 [5]). |  |
| Distribution of DL UE throughput in gNB | Provides the distribution of the UE throughput in downlink (see clause 5.1.1.3.1 in TS 28.552 [5]). |  |
| Average UL UE throughput in gNB | Provides the average UE throughput in uplink (see clause 5.1.1.3.3 in TS 28.552 [5]). |  |
| Distribution of UL UE throughput in gNB | Provides the distribution of the UE throughput in uplink (see clause 5.1.1.3.4 in TS 28.552 [5]). |  |
| Mean number of RRC Connections | Provides the mean number of users in RRC connected mode (see clause 5.1.1.4.1 in TS 28.552 [5]). |  |
| Max number of RRC Connections | Provides the maximum number of users in RRC connected mode (see clause 5.1.1.4.2 in TS 28.552 [5]). |  |
| Number of PDU Sessions requested to setup | Provides the number of PDU Sessions by the gNB (see clause 5.1.1.5.1 in TS 28.552 [5]). |  |
| Number of PDU Sessions successfully setup | Provides the number of PDU Sessions successfully setup by the gNB from AMF (see clause 5.1.1.5.2 in TS 28.552 [5]). |  |
| Number of PDU Sessions failed to setup | Provides the number of PDU Sessions failed to setup by the gNB (see clause 5.1.1.5.3 in TS 28.552 [5]). |  |

Start of Next change

7.2.4.3	MnS Component Type C definition

7.2.4.3.1	Performance measurements

Table 7.2.4.3.1-1. lists the performance measurements that are used to monitor the load of NR cells (see clause 15.5.1.2 in TS 38.300 [7]).

Table 7.2.4.3.1-1. C-LBO load performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| DL Total PRB Usage | This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the downlink (see clause 5.1.1.2.1 in TS 28.552 [5]). |  |
| UL Total PRB Usage | This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the uplink (see clause 5.1.1.2.2 in TS 28.552 [5]). |  |
| Distribution of DL Total PRB Usage | This distribution measurement is to monitor when a cell may experience overload situation in the downlink (see clause 5.1.1.2.3 in TS 28.552 [5]). |  |
| Distribution of UL Total PRB Usage | This distribution measurement is to monitor when a cell may experience overload situation in the uplink (see clause 5.1.1.2.4 in TS 28.552 [5]). |  |
| DL PRB used for data traffic | This measurement provides the number of physical resource blocks (PRBs) in average used in downlink for data traffic (see clause 5.1.1.2.5 in TS 28.552 [5]). |  |
| UL PRB used for data traffic | This measurement provides the number of physical resource blocks (PRBs) in average used in uplink for data traffic (see clause 5.1.1.2.7 in TS 28.552 [5]). |  |
| Mean number of RRC Connections | This measurement provides the mean number of users in RRC connected mode during the granularity period (see clause 5.1.1.4.1 in TS 28.552 [5]). |  |
| Max number of RRC Connections | This measurement provides the maximum number of users in RRC connected mode during the granularity period (see clause 5.1.1.4.2 in TS 28.552 [5]). |  |
| Mean number of stored inactive RRC Connections | This measurement provides the mean number of users in RRC inactive mode during each granularity period (see clause 5.1.1.4.3 in TS 28.552 [5]). |  |
| Max number of stored inactive RRC Connections | This measurement provides the maximum number of users in RRC inactive mode during each granularity period (see clause 5.1.1.4.4 in TS 28.552 [5]). |  |

Table 7.2.4.3.1-2 lists the performance measurements used to monitor the LBO performance:

Table 7.2.4.3.1-2. C-LBO related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| Attempted RRC connection establishments | Includes the number of RRC connection establishment attempts (see clause 5.1.1.15.1 in TS 28.552 [5]). |  |
| Successful RRC connection establishments | Includes the number of successful RRC establishments (see clause 5.1.1.15.2 in TS 28.552 [5]). |  |
| Number of RRC connection re-establishment attempts | Includes the number of RRC connection re-establishment attempts (see clauses 5.1.1.17.1 in TS 28.552 [5]). |  |
| Successful RRC connection re-establishment | Includes the number of successful RRC connection re-establishment (see clauses 5.1.1.17.2 and 5.1.1.17.3 in TS 28.552 [5]). |  |
| Number of RRC connection resuming attempts | Includes Number of RRC connection resuming attempts (see clause 5.1.1.18.1 in TS 28.552 [5]). |  |
| Successful RRC connection resuming | Includes the number of successful RRC connection resuming (see clause 5.1.1.18.2 in TS 28.552 [5]). |  |

Start of End change