3GPP TSG-SA5 Meeting #151 	S5-237183

Xiaman, China, 09 - 13 October 2023

# Source:	Samsung

# Title:	Rel-18 CR 28.538 Federation Procedure.docx

# Document for:	Approval

Agenda Item:	6.5.8.3

# 1	Decision/action requested

For approval.

# 2	References

[1]	S5-235989 Rel-18 draftCR 28.538

# 3	Rationale

This contribution provides federation procedure for 28.538 which is very much needed to support GSMA requirements.

# 4	Detailed proposal

The following changes are proposed to [1].

First Modified Section

## 7.x	Federation management

### 7.x.1	Description

The clause contains procedures associated with federation management.

### 7.x.2	Edge Federation Establishment

Figure 7.x.2.1-1 depicts a procedure that describes how a federation relationship is established between LO and PO.

Figure 7.x.2.1-1: Federation Establishment

- In order to establish the edge federation with the PO (Producer), LO (Consumer) send createMOI request to instantiate EdgeFederation IOC.

Editor’s Note: Whether consumer may send a ModifyMOI operation is FFS.

- Producer send the response

- Producer instantiate the OperatorEdgeFederation IOC providing details on the offered EDN.

- Producer send notification for creation of the OperatorEdgeFederation IOC using notifyMOICreation as defined in 3GPP TS 28.532.

- Consumer reads the OperatorEdgeFederation MOI using GetMOIAttributes operation to know the offered EDN and other related information.

- Producer send the response

- Based on the offered EDN list consumer decides on to which EDN it want to accept.

- Consumer updates the value of acceptedEDN attribute, indicating the accepted EDN, using  ModifyMOIAttributes operation.

- Producer send the response. At this point the federation establishment is completed.

- Based on the accepted EDN information, producer instantiate OperatorEdgeDataNetwork IOC to represent the accepted EDN which is shared with the LO.

- The LO, behaving as Provisioning MnS Producer, instantiate the FederationIOC, OperatorFederation IOC and OperatorEdgeDataNetwork IOC.

End Of Modified Section